CAPSTONE PROJECT

Test 6:

OFDM MODULATION

# Orthogonal Frequency Division Multiplexing

Group 1:

# 58620_Harsh Prasad

# 58618_Vinayak Majhi

# 58633_Diptendra Maity

# 58625_Keshav Janghala

# 58621_Diyanisha Dey

# Table of Contents

# 1. Introduction to OFDM*

# -1.1 What is OFDM (Orthogonal Frequency Division Multiplexing)?

# - 1.2 Historical Evolution and Applications

# - 1.3 Why OFDM? Problem Statement

# - 1.3.1 Challenge 1: Multipath Fading

# - 1.3.2 Challenge 2: Spectral Efficiency

# - 1.3.3 Challenge 3: Equalization Complexity

# - 1.4 OFDM vs Traditional FDM

# - 1.5 Real-World OFDM Applications

# 2. OFDM Fundamentals & Mathematical Foundation*

# - 2.1 Orthogonality Principle

# - 2.1.1 Mathematical Definition of Orthogonality

# - 2.1.2 Subcarrier Frequency Spacing

# - 2.1.3 3GPP 5G NR Numerology

# - 2.2 Subcarrier Spacing and Null Points

# - 2.2.1 Sinc Function Characteristics

# - 2.2.2 Spectral Overlap Without Interference

# - 2.3 IFFT/FFT Implementation

# - 2.3.1 Discrete Fourier Transform (DFT) Foundation

# - 2.3.2 FFT Algorithm Efficiency

# - 2.4 Time-Frequency Duality

# - 2.5 OFDM Symbol Structure

# - 2.6 Mathematical Derivation with Examples

# 3. OFDM System Architecture*

# - 3.1 OFDM Transmitter Block Diagram

# - 3.1.1 Complete Transmitter Architecture

# - 3.1.2 Transmitter Block Diagram

# - 3.2 OFDM Receiver Block Diagram

# - 3.3 Serial-to-Parallel Conversion

# - 3.4 Modulation Mapping (BPSK, QPSK, 16-QAM, 64-QAM)

# - 3.4.1 BPSK Modulation

# - 3.4.2 QPSK Modulation

# - 3.4.3 16-QAM Modulation

# - 3.4.4 64-QAM Modulation

# - 3.5 IFFT/FFT Processing

# - 3.6 Parallel-to-Serial Conversion

# - 3.7 Cyclic Prefix Insertion and Removal

# 4. Cyclic Prefix (CP) - Deep Dive*

# - 4.1 What is Cyclic Prefix?

# - 4.2 Why CP is Essential

# - 4.2.1 Problem 1: Inter-Symbol Interference (ISI)

# - 4.2.2 Problem 2: Inter-Carrier Interference (ICI)

# - 4.3 CP Length Calculation

# - 4.3.1 Step-by-Step Calculation Example (LTE 20 MHz)

# - 4.4 ISI (Inter-Symbol Interference) Mitigation

# - 4.5 ICI (Inter-Carrier Interference) Prevention

# - 4.6 CP Overhead Trade-offs

# - 4.7 Real-World CP Examples

# - 4.7.1 LTE Cyclic Prefix Configuration

# - 4.7.2 5G NR Cyclic Prefix Configuration

# - 4.7.3 Wi-Fi (IEEE 802.11) Guard Interval

# 5. Channel Effects and OFDM

# - 5.1 Multipath Fading Channels

# - 5.1.1 Channel Impulse Response (CIR)

# - 5.1.2 Power Delay Profile (PDP) and Delay Spread

# - 5.2 Frequency Selective Fading

# - 5.2.1 Flat vs. Frequency Selective Fading

# - 5.2.2 Coherence Bandwidth and Subcarrier Design

# - 5.3 Doppler Shift Effects

# - 5.3.1 Doppler Frequency and Coherence Time

# - 5.3.2 Doppler Analysis Across Standards

# - 5.4 Channel Equalization in OFDM

# - 5.5 One-Tap Frequency Domain Equalization

# - 5.6 Channel Estimation Techniques

# 6. OFDM Parameters and Design

# - 6.1 Subcarrier Count Selection

# - 6.1.1 Factors Affecting Subcarrier Count

# - 6.1.2 Mathematical Relationships

# - 6.2 Symbol Duration

# - 6.2.1 Useful Symbol Duration (Tu)

# - 6.2.2 Total Symbol Duration (Ts)

# - 6.3 Subcarrier Spacing

# - 6.4 FFT Size Selection

# - 6.5 Bandwidth and Data Rate Calculation

# - 6.6 Spectral Efficiency

# - 6.7 PAPR (Peak-to-Average Power Ratio)

# 7. OFDM in Standards

# - 7.1 OFDM in Wi-Fi (IEEE 802.11a/g/n/ac/ax/be)

# - 7.1.1 Evolution of OFDM in Wi-Fi

# - 7.1.2 IEEE 802.11ax (Wi-Fi 6) - OFDMA Implementation

# - 7.2 OFDM in 4G LTE (3GPP Release 8-15)

# - 7.2.1 LTE OFDM Parameters (3GPP TS 36.211)

# - 7.2.2 Cyclic Prefix Configuration

# - 7.3 OFDM in 5G NR (3GPP Release 15+)

# - 7.3.1 Scalable OFDM Numerology

# - 7.4 OFDM in Digital Broadcasting (DVB-T/DVB-T2)

# - 7.5 OFDM in DSL (Asymmetric Digital Subscriber Line)

# 8. Advanced OFDM Concepts

# - 8.1 OFDMA (Orthogonal Frequency Division Multiple Access)

# - 8.1.1 Resource Block Structure

# - 8.1.2 OFDMA vs OFDM: Key Differences

# - 8.1.3 Scheduling and Multi-User Diversity

# - 8.2 SC-FDMA (Single Carrier FDMA) for Uplink

# - 8.2.1 DFT-Spread OFDM Architecture

# - 8.2.2 PAPR Reduction Mechanism

# - 8.2.3 SC-FDMA vs OFDMA Comparison

# - 8.3 MIMO-OFDM (Multiple Input Multiple Output)

# - 8.3.1 MIMO Channel Model in OFDM

# - 8.3.2 MIMO Modes in 5G NR

# - 8.3.3 Per-Subcarrier MIMO Equalization

# - 8.4 Beamforming with OFDM

# - 8.4.1 Analog, Digital, and Hybrid Beamforming

# - 8.4.2 Phased Array Steering and Per-Subcarrier Optimization

# - 8.5 Pilot Subcarriers and Channel Estimation

# - 8.5.1 3GPP Reference Signal Types

# - 8.5.2 Zadoff-Chu Sequences for DMRS

# - 8.6 PAPR (Peak-to-Average Power Ratio) Reduction Techniques

# 9. OFDM Advantages and Disadvantages

# - 9.1 Advantages of OFDM

# - 9.1.1 Superior Multipath Resistance

# - 9.1.2 High Spectral Efficiency

# - 9.1.3 Simplified Equalization

# - 9.1.4 Flexible Resource Allocation

# - 9.1.5 MIMO Compatibility

# - 9.1.6 Efficient DSP Implementation

# - 9.2 Disadvantages and Challenges

# - 9.3 Comparison with Other Modulation Schemes

# 10. C++ Implementation

# - 10.1 Complete OFDM Transmitter Code

# - 10.1.1 NumerologyManager Class

# - 10.1.2 ModulationEngine Class

# - 10.1.3 ResourceGridManager Class

# - 10.1.4 FFTProcessor Class

# - 10.2 Complete OFDM Receiver Code

# - 10.3 Channel Modeling

# - 10.4 BER Performance Analysis

# 11. Simulation Results & Analysis

# - 11.1 Time Domain Waveforms

# - 11.2 Frequency Domain Spectrum

# - 11.3 BER vs SNR Curves

# - 11.4 Effect of Cyclic Prefix Length

# - 11.5 Effect of FFT Size

# - 11.6 Constellation Diagrams at Different SNR

# - 11.7 Resource Grid Structure

# 12. Key Concepts & References

# - 12.1 Important Formulas Summary

# - 12.1.1 Basic OFDM Parameters

# - 12.1.2 Orthogonality Condition

# - 12.1.3 IFFT/FFT Implementation

# - 12.1.4 Channel Model and Equalization

# - 12.2 3GPP Technical Specifications

# - 12.2.1 3GPP TS 38.211 - 5G NR Physical Channels

# - 12.2.2 3GPP TS 36.211 - LTE Physical Channels

# - 12.3 IEEE 802.11 Standards (Wi-Fi)

# - 12.4 Further Reading

# 13. Compilation & Execution Guide

# - 13.1 MATLAB Setup and Execution

# - 13.1.1 System Requirements

# - 13.1.2 Installation Steps

# - 13.1.3 Project File Structure

# - 13.1.4 Running the Simulation

# - 13.1.5 Expected Output

# - 13.2 Python Setup and Execution

# - 13.2.1 System Requirements

# - 13.2.2 Installation Steps

# - 13.3 C++ Compilation and Execution

# - 13.4 Troubleshooting

# 1. Introduction to OFDM

Orthogonal Frequency Division Multiplexing (OFDM) is a revolutionary digital multi-carrier modulation technique that has become the cornerstone of modern wireless communication systems. OFDM addresses fundamental challenges in high-speed data transmission over frequency-selective fading channels by dividing a wideband channel into multiple narrowband orthogonal subcarriers.

This technique is extensively deployed in contemporary communication standards including 4G LTE (3GPP Release 8 onwards), 5G New Radio (3GPP Release 15/16), Wi-Fi (IEEE 802.11a/g/n/ac/ax), Digital Video Broadcasting (DVB-T/T2), and WiMAX. The widespread adoption of OFDM stems from its inherent robustness against multipath fading, high spectral efficiency, and compatibility with Multiple Input Multiple Output (MIMO) antenna systems.

## 1.1 What is OFDM (Orthogonal Frequency Division Multiplexing)?

OFDM is a multi-carrier modulation scheme where data is transmitted simultaneously over multiple closely-spaced orthogonal subcarriers. Unlike traditional Frequency Division Multiplexing (FDM), which requires guard bands between carriers to prevent interference, OFDM allows subcarriers to overlap in the frequency domain while maintaining orthogonality.

### Key Characteristics:

- Multiple Subcarriers: The available bandwidth is divided into N narrowband subcarriers, each carrying a portion of the data at a much lower symbol rate.

- Orthogonality: Subcarriers are mathematically orthogonal, meaning they do not interfere with each other despite spectral overlap. This is achieved by carefully spacing subcarriers at integer multiples of 1/T, where T is the symbol duration.

- Spectral Efficiency: OFDM eliminates the need for guard bands between subcarriers, achieving significantly higher spectral efficiency (typically 90-95%) compared to traditional FDM (60-70%).

- Cyclic Prefix (CP): A copy of the end portion of each OFDM symbol is prepended to the beginning, creating a guard interval that eliminates Inter-Symbol Interference (ISI) and preserves orthogonality in multipath channels.

- IFFT/FFT Processing: OFDM utilizes Inverse Fast Fourier Transform (IFFT) at the transmitter and Fast Fourier Transform (FFT) at the receiver for efficient modulation and demodulation.

### Real-World Analogy:

Imagine a highway (available bandwidth) with multiple lanes (subcarriers). In traditional FDM, each lane has wide barriers (guard bands) separating them to prevent cars from crossing over. This wastes a lot of road space. OFDM is like removing those barriers and painting precise lane markings (orthogonality) - cars can drive very close to each other without colliding, utilizing almost every inch of the highway. The cyclic prefix is like a safety buffer zone at traffic merges that prevents accidents (interference) even when there are delays or detours (multipath propagation).

## 1.2 Historical Evolution and Applications

The journey of OFDM from theoretical concept to ubiquitous deployment spans over five decades:

- 1971: Weinstein and Ebert filed the original OFDM patent, introducing the concept of using discrete Fourier transform for multi-carrier modulation.

- 1985: Cyclic prefix was introduced by Peled and Ruiz to combat ISI in multipath channels.

- 1995: DVB-T (Digital Video Broadcasting - Terrestrial) became the first major commercial deployment of OFDM for digital television.

- 1999: IEEE 802.11a standardized OFDM for Wi-Fi, supporting data rates up to 54 Mbps at 5 GHz.

- 2009: 3GPP Release 8 introduced LTE with OFDMA (Orthogonal Frequency Division Multiple Access) for the downlink, revolutionizing mobile broadband.

- 2019: 3GPP Release 15 finalized 5G NR with CP-OFDM (Cyclic Prefix OFDM) supporting flexible numerology (multiple subcarrier spacings: 15, 30, 60, 120, 240 kHz).

Figure 1.1: OFDM Technology Evolution Timeline

## 1.3 Why OFDM? Problem Statement

Traditional single-carrier modulation schemes face significant challenges in modern wireless environments:

### Challenge 1: Multipath Fading

In wireless communication, transmitted signals reflect off buildings, terrain, and obstacles, creating multiple signal paths with different delays. This phenomenon, known as multipath propagation, causes:

- Inter-Symbol Interference (ISI): Delayed copies of transmitted symbols overlap with subsequent symbols, causing severe distortion.

- Frequency-Selective Fading: Different frequency components of the signal experience varying attenuation, creating deep nulls in the frequency domain.

OFDM Solution: By dividing the wideband channel into narrow subcarriers, each subcarrier experiences flat fading (constant attenuation across its bandwidth). The cyclic prefix absorbs multipath delays, eliminating ISI between symbols.

Figure 1.2: Single Carrier vs OFDM in Multipath Channel

### Challenge 2: Spectral Efficiency

Traditional FDM requires substantial guard bands (typically 10-20% of carrier bandwidth) between adjacent carriers to prevent inter-carrier interference (ICI). This represents significant spectrum wastage.

OFDM Solution: Orthogonal subcarriers overlap in frequency without interfering, eliminating guard bands and achieving near-optimal spectral efficiency. In 5G NR, OFDM achieves >90% spectral efficiency.

## 1.4 OFDM vs Traditional FDM

Understanding the fundamental difference between FDM and OFDM is crucial:

Figure 1.3: Traditional FDM vs OFDM Spectrum Comparison

### Detailed Comparison:

| Parameter | Traditional FDM | OFDM |
| --- | --- | --- |
| Guard Bands | Required (10-20% overhead) | Not required |
| Spectral Efficiency | 60-70% | 90-95% |
| Multipath Robustness | Poor - requires complex equalization | Excellent - simple one-tap equalization |
| Implementation | Analog filters (complex) | Digital FFT/IFFT (efficient) |
| Frequency Selectivity | Entire signal affected | Each subcarrier independent |

## 1.5 Real-World OFDM Applications

### 1.5.1 4G LTE (3GPP TS 36.211)

Long-Term Evolution (LTE), standardized in 3GPP Release 8 (2008), uses OFDMA for the downlink:

- Subcarrier Spacing: Fixed at 15 kHz (maintaining consistency across all channel bandwidths)

- FFT Size: Scalable from 128 (1.4 MHz) to 2048 (20 MHz)

- Cyclic Prefix: Normal CP = 4.7 µs (7 symbols/slot), Extended CP = 16.7 µs (6 symbols/slot)

- Resource Block: 12 subcarriers × 7 symbols = 84 resource elements (180 kHz × 0.5 ms)

- Peak Data Rate: Up to 300 Mbps (2×2 MIMO) or 1 Gbps (4×4 MIMO) with 64-QAM

### 1.5.2 5G NR (3GPP TS 38.211)

5G New Radio, finalized in 3GPP Release 15 (2018), introduces flexible numerology with CP-OFDM:

- Subcarrier Spacing (μ): 15 kHz (μ=0), 30 kHz (μ=1), 60 kHz (μ=2), 120 kHz (μ=3), 240 kHz (μ=4)

- Scalable Numerology: Symbol duration = 1/(14 × Δf), where Δf = 15 × 2^μ kHz

- FFT Size: Up to 4096 for 100 MHz (FR1) and 400 MHz (FR2)

- Frequency Ranges: FR1 (450 MHz - 6 GHz), FR2 (24.25 GHz - 52.6 GHz, mmWave)

- Peak Data Rate: 20 Gbps downlink, 10 Gbps uplink (theoretical with 8×8 MIMO, 256-QAM)

Figure 1.4: LTE vs 5G NR OFDM Parameters (3GPP Comparison)

### 1.5.3 Wi-Fi (IEEE 802.11 Standards)

OFDM has been integral to Wi-Fi evolution since 802.11a:

- 802.11a/g (1999/2003): 64-point FFT, 52 subcarriers (48 data + 4 pilots), 312.5 kHz spacing, 54 Mbps max

- 802.11n (2009): MIMO-OFDM with up to 4 spatial streams, 40 MHz channels, 600 Mbps max

- 802.11ac (2013): 160 MHz channels, 8 spatial streams, 256-QAM, 6.93 Gbps max

- 802.11ax Wi-Fi 6 (2019): OFDMA, 1024-QAM, 9.6 Gbps theoretical, improved efficiency in dense environments

### 1.5.4 Digital Broadcasting (DVB-T/T2)

OFDM revolutionized digital television broadcasting:

- DVB-T (1997): 2K/8K FFT modes, QPSK/16-QAM/64-QAM, supports mobility (Doppler shift tolerance)

- DVB-T2 (2009): 32K FFT, 256-QAM, LDPC coding, 50% capacity improvement over DVB-T

## 1.6 The Orthogonality Principle - Core Foundation

Orthogonality is the mathematical property that enables OFDM subcarriers to overlap without interference. Two signals s₁(t) and s₂(t) are orthogonal over an interval T if their inner product equals zero:

∫₀ᵀ s₁(t) × s₂(t) dt = 0

For OFDM, subcarrier frequencies are chosen as:

fₖ = f₀ + k × Δf,    where Δf = 1/T

Here, T is the OFDM symbol duration, and k is an integer (0, 1, 2, ..., N-1). This spacing ensures that at the center frequency of each subcarrier, all other subcarriers have zero amplitude (null points), preventing inter-carrier interference.

Figure 1.5: Mathematical Proof of Orthogonality Between Subcarriers

### Mathematical Example:

Consider two subcarriers with frequencies f₁ = 5 Hz and f₂ = 7 Hz over a symbol period T = 1 second. Despite being only 2 Hz apart, they are perfectly orthogonal:

s₁(t) = sin(2π × 5t)    [completes exactly 5 cycles in T]

s₂(t) = sin(2π × 7t)    [completes exactly 7 cycles in T]

∫₀¹ sin(2π × 5t) × sin(2π × 7t) dt = 0

This orthogonality holds for any pair of subcarriers as long as their frequency spacing is an integer multiple of 1/T. In practical OFDM systems:

- LTE: Δf = 15 kHz, T = 1/15000 = 66.67 µs

- 5G NR (μ=1): Δf = 30 kHz, T = 1/30000 = 33.33 µs

- Wi-Fi 802.11a: Δf = 312.5 kHz, T = 1/312500 = 3.2 µs

## 1.7 Summary and Key Takeaways

This introduction has established the foundational understanding of OFDM:

- OFDM divides wideband channels into multiple orthogonal narrowband subcarriers

- Orthogonality eliminates guard bands, achieving 90-95% spectral efficiency

- Cyclic prefix provides immunity to multipath fading and ISI

- IFFT/FFT implementation enables efficient digital modulation/demodulation

- Deployed in 4G LTE (3GPP TS 36.211), 5G NR (3GPP TS 38.211), Wi-Fi, and DVB-T

- 5G NR introduces flexible numerology (multiple subcarrier spacings) for diverse use cases

The subsequent sections will delve into the mathematical foundations, system architecture, cyclic prefix mechanics, channel effects, and practical C++ implementation of OFDM systems, all grounded in 3GPP standards and real-world deployments.

## 1.8 References

- 3GPP TS 38.211: NR; Physical channels and modulation (Release 16)

- 3GPP TS 36.211: Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation

- IEEE 802.11-2020: IEEE Standard for Information Technology—Telecommunications and Information Exchange between Systems—Local and Metropolitan Area Networks—Specific Requirements—Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

- Weinstein, S. B., & Ebert, P. M. (1971): Data Transmission by Frequency-Division Multiplexing Using the Discrete Fourier Transform. IEEE Transactions on Communication Technology

- Peled, A., & Ruiz, A. (1980): Frequency Domain Data Transmission using Reduced Computational Complexity Algorithms. ICASSP

# 2.1 Orthogonality Principle

The concept of orthogonality is the cornerstone of OFDM technology. Two signals are orthogonal if their inner product (correlation) is zero over a specific time interval. This mathematical property enables OFDM subcarriers to overlap in frequency without interfering with each other.

### Mathematical Definition of Orthogonality

Two complex exponential signals φₘ(t) and φₙ(t) are orthogonal over interval [0, T] if:

∫₀ᵀ φₘ(t) · φₙ*(t) dt = 0,    for m ≠ n

Where:

φₘ(t) = exp(j2πfₘt) = cos(2πfₘt) + j·sin(2πfₘt) - Complex exponential for subcarrier m

φₙ*(t) - Complex conjugate of subcarrier n

T - OFDM symbol duration (excluding cyclic prefix)

fₘ, fₙ - Frequencies of subcarriers m and n

### Subcarrier Frequency Spacing

For orthogonality, subcarriers must be spaced by integer multiples of 1/T:

fₙ = f₀ + n·Δf,    where Δf = 1/T

This critical relationship ensures that within one OFDM symbol period T, each subcarrier completes an integer number of cycles, making them orthogonal to all other subcarriers.

### 3GPP 5G NR Numerology (3GPP TS 38.211)

5G NR introduces flexible numerology with different subcarrier spacings:

| μ | Δf (kHz) | T (μs) | Use Case | Doppler Tolerance |
| --- | --- | --- | --- | --- |
| 0 | 15 | 66.67 | Sub-6 GHz | ±7.5% |
| 1 | 30 | 33.33 | FR1/FR2 | ±15% |
| 2 | 60 | 16.67 | mmWave | ±30% |
| 3 | 120 | 8.33 | mmWave | ±60% |
| 4 | 240 | 4.17 | mmWave | ±120% |

Table 2.1: 5G NR Numerology Parameters (3GPP TS 38.211 Section 4.2)

The relationship Δf = 2^μ × 15 kHz provides scalability for different deployment scenarios, from low-mobility sub-6 GHz to high-speed mmWave applications.

# 2.2 Subcarrier Spacing and Null Points

The frequency response of each OFDM subcarrier follows a sinc function pattern (sin(x)/x), which is the Fourier transform of a rectangular time-domain pulse. The key insight is that subcarriers are positioned at the nulls of adjacent subcarriers, allowing them to overlap in frequency without causing interference.

### Sinc Function Characteristics

The frequency response of subcarrier n centered at frequency fₙ is:

Hₙ(f) = T · sinc[πT(f - fₙ)] = T · sin[πT(f - fₙ)] / [πT(f - fₙ)]

Key properties of the sinc function:

Main lobe width: 2/T Hz (measured between first nulls)

First null occurs at f = fₙ ± 1/T = fₙ ± Δf

Side lobes decay at -13 dB per decade

Peak value at center frequency: Hₙ(fₙ) = T

### Orthogonality Through Null Positioning

When subcarriers are spaced by Δf = 1/T, each subcarrier's peak aligns with the nulls of all other subcarriers:

Hₙ(fₘ) = 0  for all m ≠ n (when fₘ - fₙ = k/T, k ∈ ℤ)

This null-positioning ensures zero inter-carrier interference (ICI) at sampling instants, even though subcarrier spectra overlap significantly in the frequency domain.

### Spectral Efficiency Advantage

Traditional FDM requires guard bands between channels to prevent interference. OFDM's orthogonality eliminates this need:

| Parameter | Traditional FDM | OFDM |
| --- | --- | --- |
| Guard Band | Required (10-20%) | Not required |
| Spectral Efficiency | ~80-85% | ~95-98% |
| Filter Complexity | High (N filters) | Low (FFT/IFFT) |

Table 2.2: Spectral Efficiency Comparison

### 3GPP LTE Example (TS 36.211)

In LTE with 15 kHz subcarrier spacing:

Symbol duration T = 1/15 kHz = 66.67 μs

First null of each subcarrier at ±15 kHz from center

Adjacent subcarriers perfectly aligned at null points

20 MHz bandwidth supports 1200 subcarriers (100 resource blocks × 12 subcarriers)

# 2.3 IFFT/FFT Implementation

The Fast Fourier Transform (FFT) and its inverse (IFFT) are at the heart of practical OFDM implementations. These algorithms efficiently compute the Discrete Fourier Transform (DFT), enabling real-time OFDM modulation and demodulation with minimal computational complexity.

### DFT to IFFT/FFT Relationship

The Inverse Discrete Fourier Transform (IDFT) generates the time-domain OFDM signal:

x[n] = (1/N) Σₖ₌₀^(N-1) Xₖ · e^(j2πkn/N),   n = 0, 1, ..., N-1

Where:

N = FFT size (number of subcarriers)

Xₖ = Complex symbol on subcarrier k (from QAM/PSK modulation)

x[n] = Time-domain sample n

e^(j2πkn/N) = cos(2πkn/N) + j·sin(2πkn/N) - Complex exponential basis

At the receiver, the DFT recovers frequency-domain symbols:

Xₖ = Σₙ₌₀^(N-1) x[n] · e^(-j2πkn/N),   k = 0, 1, ..., N-1

### Computational Complexity

The FFT algorithm (Cooley-Tukey) reduces complexity dramatically:

| Method | Complexity | Operations (N=1024) |
| --- | --- | --- |
| Direct DFT | O(N²) | ~1,048,576 |
| FFT (Cooley-Tukey) | O(N log₂N) | ~10,240 |
| Speedup Factor | - | ~102× |

Table 2.3: DFT vs FFT Computational Complexity

### 3GPP Implementation Details

3GPP standards specify FFT sizes based on system bandwidth:

| Bandwidth | FFT Size (LTE) | Active Subcarriers | Resource Blocks |
| --- | --- | --- | --- |
| 1.4 MHz | 128 | 72 | 6 |
| 5 MHz | 512 | 300 | 25 |
| 10 MHz | 1024 | 600 | 50 |
| 20 MHz | 2048 | 1200 | 100 |

Table 2.4: LTE FFT Configurations (3GPP TS 36.211, Δf = 15 kHz)

Note that not all FFT bins are used - guard bands occupy unused subcarriers at band edges to prevent out-of-band emissions and satisfy regulatory requirements.

# 2.4 Time-Frequency Duality

OFDM exemplifies the fundamental duality between time and frequency domains. The Fourier transform establishes an inverse relationship: longer time-domain signals correspond to narrower frequency-domain representations, and vice versa.

### Fundamental Duality Principle

For an OFDM system with symbol duration T and subcarrier spacing Δf:

Δf · T = 1

This relationship shows that:

Longer OFDM symbols → Smaller subcarrier spacing → Better frequency resolution

Shorter OFDM symbols → Larger subcarrier spacing → Better Doppler tolerance

Δf and T are inversely proportional - cannot optimize both simultaneously

### Trade-offs in OFDM Design

The time-frequency duality creates several design trade-offs:

| Parameter | Large Δf (Small T) | Small Δf (Large T) |
| --- | --- | --- |
| Doppler Tolerance | ✓ Better (high mobility) | ✗ Worse (static/low mobility) |
| Delay Spread Tolerance | ✗ Worse (needs longer CP) | ✓ Better (shorter CP fraction) |
| Overhead | ✗ Higher (CP is larger % of symbol) | ✓ Lower (CP is smaller % of symbol) |
| Latency | ✓ Lower (shorter symbol) | ✗ Higher (longer symbol) |
| Use Case | High-speed mobility, mmWave, URLLC | Static/low mobility, eMBB |

Table 2.5: Impact of Subcarrier Spacing on System Performance

### 5G NR Flexible Numerology (3GPP TS 38.211)

5G NR exploits time-frequency duality through scalable numerology, allowing different subcarrier spacings for different scenarios:

Sub-6 GHz eMBB (μ=0, 15 kHz): Long symbols for spectral efficiency

mmWave (μ=2,3,4, 60-240 kHz): Short symbols for Doppler tolerance and low latency

URLLC (μ=1,2, 30-60 kHz): Balanced trade-off for reliability and latency

Mixed numerology: Multiple μ values can coexist in time/frequency

Example calculation for μ=2 (60 kHz):

T = 1/Δf = 1/(60 kHz) = 16.67 μs

Symbol period (with CP) = 17.84 μs (4× faster than LTE)

# 2.5 OFDM Symbol Structure

An OFDM symbol consists of the useful symbol period plus a cyclic prefix (CP). Understanding this structure is critical for system design and performance analysis.

### Complete OFDM Symbol Structure

Total symbol duration:

Tsymbol = TCP + T = TCP + 1/Δf

Where:

TCP = Cyclic prefix duration (typically 4.7-16.7 μs in LTE)

T = Useful symbol duration = 1/Δf (66.67 μs for 15 kHz spacing)

Tsymbol = Complete OFDM symbol including overhead

### Cyclic Prefix Overhead

The cyclic prefix introduces overhead that reduces spectral efficiency:

Overhead = TCP / Tsymbol = TCP / (TCP + T)

| CP Type | TCP (μs) | T (μs) | Tsymbol (μs) | Overhead |
| --- | --- | --- | --- | --- |
| Normal CP (1st symbol) | 5.21 | 66.67 | 71.88 | 7.25% |
| Normal CP (other symbols) | 4.69 | 66.67 | 71.35 | 6.57% |
| Extended CP | 16.67 | 66.67 | 83.33 | 20.0% |

Table 2.6: LTE Cyclic Prefix Overhead (Δf = 15 kHz, 3GPP TS 36.211)

### Slot and Frame Structure

OFDM symbols are organized into larger time structures according to 3GPP specifications:

LTE Slot: 7 symbols (normal CP) or 6 symbols (extended CP) = 0.5 ms

LTE Subframe: 2 slots = 14 symbols (normal CP) = 1 ms

LTE Frame: 10 subframes = 140 symbols = 10 ms

5G NR Slot: 14 symbols (normal CP) with flexible duration based on μ

# 2.6 Mathematical Derivation with Examples

This section provides detailed mathematical derivations with concrete numerical examples to solidify understanding of OFDM principles.

### Example 1: Orthogonality Proof

Given: Two subcarriers at frequencies f₁ = 1000 Hz and f₂ = 1015 Hz with T = 66.67 μs (Δf = 15 kHz).

Prove they are orthogonal:

Step 1: Define the signals

φ₁(t) = e^(j2π·1000·t)

φ₂(t) = e^(j2π·1015·t)

Step 2: Compute inner product over [0, T]

∫₀ᵀ φ₁(t)·φ₂*(t) dt = ∫₀ᵀ e^(j2π·1000·t)·e^(-j2π·1015·t) dt

= ∫₀ᵀ e^(j2π·(1000-1015)·t) dt = ∫₀ᵀ e^(-j2π·15·t) dt

Step 3: Evaluate integral

= [e^(-j2π·15·t) / (-j2π·15)]₀ᵀ

= [e^(-j2π·15·66.67×10⁻⁶) - 1] / (-j2π·15)

= [e^(-j2π·1.000) - 1] / (-j2π·15) = [1 - 1] / (-j2π·15) = 0 ✓

Result: The signals are orthogonal because the integral equals zero. The frequency difference (15 Hz) equals 1/T, ensuring that subcarrier 2 completes exactly one more cycle than subcarrier 1 over the symbol period.

### Example 2: IFFT Generation

Given: N = 4 subcarriers with QAM symbols X = [1+j, -1+j, -1-j, 1-j].

Calculate: Time-domain samples using IFFT.

Step 1: Apply IDFT formula

x[n] = (1/4) Σₖ₌₀³ Xₖ·e^(j2πkn/4),   n = 0,1,2,3

Step 2: Calculate x[0]

x[0] = (1/4)[(1+j)·1 + (-1+j)·1 + (-1-j)·1 + (1-j)·1]

= (1/4)[0 + 0j] = 0

Step 3: Calculate x[1]

x[1] = (1/4)[(1+j)·1 + (-1+j)·j + (-1-j)·(-1) + (1-j)·(-j)]

= (1/4)[(1+j) + (-j-1) + (1+j) + (j+1)] = (1/4)[2+2j] = 0.5+0.5j

Continuing similarly for x[2] and x[3] yields the complete time-domain OFDM signal. These samples would then be converted to analog waveform by a DAC.

### Example 3: 5G NR Bandwidth Calculation

Given: 100 MHz 5G NR carrier with μ=1 (30 kHz subcarrier spacing).

Calculate: Number of resource blocks and data subcarriers.

Step 1: Determine FFT size

According to 3GPP TS 38.104, for 100 MHz with Δf = 30 kHz:

FFT size = 4096 subcarriers

Step 2: Calculate occupied bandwidth

Total bandwidth = 4096 × 30 kHz = 122.88 MHz

Step 3: Determine active subcarriers

With guard bands (per 3GPP TS 38.104 Table 5.3.2-1):

Active subcarriers = 3276 (273 RBs × 12 subcarriers/RB)

Occupied bandwidth = 3276 × 30 kHz = 98.28 MHz

Step 4: Calculate spectral efficiency

Efficiency = 3276/4096 = 80% (subcarrier utilization)

# 2.7 Summary and Key Takeaways

This section covered the mathematical foundations and fundamental principles of OFDM. Understanding these concepts is essential for implementing and optimizing OFDM systems.

### Core Mathematical Principles

Orthogonality Condition: ∫₀ᵀ φₘ(t)·φₙ*(t) dt = 0 for m ≠ n enables subcarrier overlap without interference

Subcarrier Spacing: Δf = 1/T is the fundamental relationship ensuring orthogonality

Sinc Function Nulls: Each subcarrier's peak aligns with nulls of all other subcarriers

FFT Efficiency: O(N log₂N) complexity vs O(N²) for direct DFT enables real-time processing

Time-Frequency Duality: Δf · T = 1 creates fundamental design trade-offs

### 3GPP Implementation Insights

Flexible Numerology: 5G NR supports μ = 0,1,2,3,4 with Δf = 2^μ × 15 kHz

Cyclic Prefix Overhead: Ranges from 6.57% (LTE normal) to 20% (LTE extended)

Resource Blocks: Fixed 12 subcarriers per RB across all numerologies

Guard Bands: Necessary for regulatory compliance, typically 10-20% of bandwidth

### Design Trade-offs

System designers must balance multiple conflicting requirements:

Spectral Efficiency vs Robustness: Smaller CP increases efficiency but reduces delay spread tolerance

Doppler Tolerance vs Overhead: Larger Δf handles mobility better but requires longer CP relative to symbol duration

Latency vs Reliability: Shorter symbols reduce latency but may sacrifice robustness

FFT Size vs Complexity: Larger FFT provides finer granularity but increases processing requirements

### Practical Implementation Considerations

FFT Size Selection: Must be power of 2 for efficient Cooley-Tukey algorithm

Subcarrier Mapping: Not all FFT bins used - DC subcarrier often nulled, edge subcarriers reserved for guard bands

Windowing: May be applied to reduce spectral leakage in non-ideal channels

Synchronization: Critical for maintaining orthogonality - timing and frequency offsets destroy orthogonality

# 2.8 References

### 3GPP Technical Specifications

3GPP TS 38.211 V18.1.0 (2024-06) - "NR; Physical channels and modulation"

3GPP TS 38.214 V18.1.0 (2024-06) - "NR; Physical layer procedures for data"

3GPP TS 38.104 V18.1.0 (2024-03) - "NR; Base Station (BS) radio transmission and reception"

3GPP TS 36.211 V17.4.0 (2023-12) - "E-UTRA; Physical channels and modulation"

3GPP TS 36.213 V17.4.0 (2023-12) - "E-UTRA; Physical layer procedures"

### IEEE Standards

IEEE 802.11-2020 - "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

IEEE 802.16-2017 - "Air Interface for Broadband Wireless Access Systems"

### Textbooks and Research Papers

Van Nee, R. and Prasad, R. (2000), "OFDM for Wireless Multimedia Communications", Artech House

Dahlman, E., Parkvall, S., and Sköld, J. (2021), "5G NR: The Next Generation Wireless Access Technology", 2nd Edition, Academic Press

Cooley, J.W. and Tukey, J.W. (1965), "An Algorithm for the Machine Calculation of Complex Fourier Series", Mathematics of Computation, Vol. 19, pp. 297-301

Weinstein, S.B. and Ebert, P.M. (1971), "Data Transmission by Frequency-Division Multiplexing Using the Discrete Fourier Transform", IEEE Trans. Communications, Vol. 19, pp. 628-634

### Online Resources

3GPP Portal: https://www.3gpp.org/specifications-technologies

IEEE Xplore Digital Library: https://ieeexplore.ieee.org

Sharetechnote (5G NR): https://www.sharetechnote.com

# 3. OFDM SYSTEM ARCHITECTURE

OFDM (Orthogonal Frequency Division Multiplexing) is a sophisticated multi-carrier modulation technique that divides a high-speed data stream into multiple parallel lower-speed streams, each transmitted over orthogonal subcarriers. The OFDM system architecture consists of carefully designed transmitter and receiver components that work together to achieve robust, high-speed wireless communication.

This section provides a comprehensive exploration of the OFDM system architecture, including detailed block diagrams, signal processing stages, and the mathematical foundations underlying each component.

## 3.1 OFDM Transmitter Block Diagram

The OFDM transmitter transforms the incoming binary data stream into a multi-carrier modulated signal suitable for transmission over a wireless channel.

### 3.1.1 Complete Transmitter Architecture

The OFDM transmitter consists of the following key blocks:

- Binary Data Source: Generates or receives the input bit stream

- Serial-to-Parallel Converter: Converts serial bit stream into N parallel streams

- Modulation Mapper: Maps groups of bits to complex symbols

- IFFT: Converts frequency-domain symbols to time-domain samples

- Parallel-to-Serial Converter: Converts parallel IFFT outputs to serial stream

- Cyclic Prefix Insertion: Adds guard interval

- DAC: Converts digital to analog signal

- RF Up-conversion: Shifts to carrier frequency

### 3.1.2 Transmitter Block Diagram

Transmitter Block Diagram:

┌─────────┐   ┌─────────┐   ┌─────────┐

│ Binary  │──>│ Serial  │──>│Modulation│

│  Data   │   │   to    │   │ Mapper   │

│ Source  │   │Parallel │   │         │

└─────────┘   └─────────┘   └─────────┘

│

▼

┌─────────┐   ┌─────────┐   ┌─────────┐

│   RF    │<──│   DAC   │<──│  IFFT   │

│Upconvert│   │         │   │ N-Point │

└─────────┘   └─────────┘   └─────────┘

│                           ▲

▼         ┌─────────┐      │

┌────────┐    │  Cyclic │      │

│Antenna │    │ Prefix  │<─────┘

│   TX   │    │   Add   │

└────────┘    └─────────┘

Signal Flow Through OFDM Transmitter

| Stage | Input | Output | Dimensions |
| --- | --- | --- | --- |
| Binary Data Source | Information bits | Serial bit stream | Rb bits/second |
| Serial-to-Parallel | Serial bits | N parallel streams | Each: Rb/N bits/sec |
| Modulation Mapper | Bit groups | Complex symbols | N symbols X[k] |
| IFFT | Freq domain X[k] | Time domain x[n] | N samples |
| Parallel-to-Serial | N parallel samples | Serial stream | N samples |
| CP Insertion | N samples | N+Ncp samples | (N+Ncp) samples |
| DAC | Digital samples | Analog signal | Continuous waveform |
| RF Upconversion | Baseband | RF signal | Carrier freq fc |

## 3.2 OFDM Receiver Block Diagram

The OFDM receiver performs inverse operations to recover the original data from the received signal. It must compensate for channel distortions, synchronize with the transmitter, and extract transmitted information.

### 3.2.1 Complete Receiver Architecture

The OFDM receiver consists of:

- RF Down-conversion: Converts RF signal to baseband

- ADC: Samples analog signal

- Synchronization: Detects symbol timing and corrects frequency offset

- CP Removal: Removes guard interval

- Serial-to-Parallel: Converts to N parallel streams

- FFT: Converts time to frequency domain

- Channel Equalization: Compensates for channel effects

- Demodulation: Maps symbols back to bits

- Parallel-to-Serial: Combines to serial bit stream

### 3.2.2 Receiver Block Diagram

┌────────┐    ┌─────────┐   ┌─────────┐

│Antenna │───>│   RF    │──>│   ADC   │

│   RX   │    │Downconv │   │         │

└────────┘    └─────────┘   └─────────┘

│

▼

┌─────────┐   ┌─────────┐   ┌─────────┐

│ Timing  │<──│  Cyclic │<──│  Sync   │

│  Freq   │   │ Prefix  │   │  Block  │

│ Offset  │   │ Removal │   │         │

└─────────┘   └─────────┘   └─────────┘

│

▼

┌─────────┐   ┌─────────┐   ┌─────────┐

│ Serial  │──>│   FFT   │──>│ Channel │

│   to    │   │ N-Point │   │  Equal  │

│Parallel │   │         │   │   ize   │

└─────────┘   └─────────┘   └─────────┘

│

▼

┌─────────┐   ┌─────────┐   ┌─────────┐

│ Binary  │<──│Parallel │<──│Demodul- │

│  Data   │   │   to    │   │ ation   │

│  Sink   │   │ Serial  │   │         │

└─────────┘   └─────────┘   └─────────┘

## 3.3 Serial-to-Parallel Conversion

Serial-to-Parallel (S/P) conversion transforms a high-speed serial data stream into multiple lower-speed parallel streams, distributing data across subcarriers.

Key Objectives:

- Rate Reduction: Reduces symbol rate by factor of N

- ISI Mitigation: Longer symbol duration increases robustness

- Parallel Processing: Enables simultaneous processing

S/P Conversion Impact on System Parameters

| Parameter | Before S/P | After S/P |
| --- | --- | --- |
| Bit Rate | 100 Mbps | 1.5625 Mbps per stream (N=64) |
| Symbol Rate | 50 Msymbols/sec | 781.25 ksymbols/sec |
| Symbol Duration | 20 ns | 1.28 μs |
| Number of Streams | 1 (serial) | 64 (parallel) |

## 3.4 Modulation Mapping (BPSK, QPSK, 16-QAM, 64-QAM)

Modulation mapping converts groups of bits into complex constellation symbols. The choice determines spectral efficiency and robustness.

### 3.4.1 BPSK (Binary Phase Shift Keying)

- 1 bit per symbol

- Two constellation points: ±1

- Most robust modulation

- Lowest spectral efficiency

### 3.4.2 QPSK (Quadrature Phase Shift Keying)

- 2 bits per symbol

- Four constellation points

- Double spectral efficiency of BPSK

- Widely used in LTE, Wi-Fi

### 3.4.3 16-QAM

- 4 bits per symbol

- 16 constellation points (4×4 grid)

- 4× spectral efficiency of BPSK

- Requires ~4 dB more SNR than QPSK

### 3.4.4 64-QAM

- 6 bits per symbol

- 64 constellation points (8×8 grid)

- 6× spectral efficiency of BPSK

- Requires ~7-8 dB more SNR than QPSK

Comparison of Modulation Schemes

| Modulation | Bits/Symbol | Points | SNR Required | Efficiency | Use Case |
| --- | --- | --- | --- | --- | --- |
| BPSK | 1 | 2 | 0 dB | Low | Deep space, noisy |
| QPSK | 2 | 4 | 0 dB | Medium | LTE, Wi-Fi |
| 16-QAM | 4 | 16 | +4 dB | High | Wi-Fi 5/6 |
| 64-QAM | 6 | 64 | +7-8 dB | Very High | Wi-Fi 6/7, 5G |
| 256-QAM | 8 | 256 | +11-12 dB | Ultra High | Wi-Fi 7 |

## 3.5 IFFT/FFT Processing

The IFFT and FFT are the heart of OFDM, enabling efficient frequency-to-time and time-to-frequency conversions.

### 3.5.1 Mathematical Foundation

Discrete Fourier Transform (DFT):

X[k] = Σ(n=0 to N-1) x[n] × e^(-j2πkn/N)

Inverse DFT (IDFT):

x[n] = (1/N) × Σ(k=0 to N-1) X[k] × e^(j2πkn/N)

### 3.5.2 Computational Complexity

DFT vs FFT Complexity

| Method | Multiplications | Additions | Complexity |
| --- | --- | --- | --- |
| Direct DFT | N² | N² - N | O(N²) |
| Radix-2 FFT | N/2 × log₂(N) | N × log₂(N) | O(N log N) |
| N=1024 DFT | 1,048,576 | 1,047,552 | Direct |
| N=1024 FFT | 5,120 | 10,240 | 204× faster |

FFT Sizes in Wireless Standards

| Standard | FFT Size | Used Subcarriers | Application |
| --- | --- | --- | --- |
| Wi-Fi 802.11a/g | 64 | 52 | 20 MHz channel |
| Wi-Fi 802.11ac | 256 | 234 | 80 MHz channel |
| LTE (20 MHz) | 2048 | 1200 | Standard LTE |
| 5G NR | 4096 | Variable | Flexible numerology |
| DVB-T | 2048/8192 | Variable | Digital TV |

## 3.6 Parallel-to-Serial Conversion

Parallel-to-Serial (P/S) conversion combines multiple parallel streams back into a single serial stream.

Role in Transmitter:

- Combines N parallel IFFT outputs

- Prepares for CP insertion

- Maintains temporal ordering

Role in Receiver:

- Combines demodulated symbols

- Reconstructs original sequence

- Delivers to higher layers

## 3.7 Cyclic Prefix Insertion and Removal

The Cyclic Prefix (CP) is critical for OFDM, enabling robust performance in multipath channels by preventing ISI and maintaining orthogonality.

### 3.7.1 What is Cyclic Prefix?

The CP copies the last Ncp samples of an OFDM symbol and prepends them to the beginning.

Original Symbol:

x = [x[0], x[1], ..., x[N-2], x[N-1]]

With CP:

x_cp = [x[N-Ncp], ..., x[N-1], x[0], ..., x[N-1]]

### 3.7.2 Why CP is Essential

1. Eliminates ISI:

CP absorbs multipath overlap if Tcp ≥ τmax (channel delay spread)

2. Maintains Orthogonality:

Converts linear convolution to circular, preserving subcarrier orthogonality

3. Simplifies Equalization:

Enables simple one-tap frequency-domain equalization

Cyclic Prefix in Wireless Standards

| Standard | FFT Size | CP Length | CP Ratio | Overhead |
| --- | --- | --- | --- | --- |
| Wi-Fi 802.11a | 64 | 16 | 1/4 | 20% |
| LTE Normal | 2048 | 144 | 1/14.2 | 7% |
| LTE Extended | 2048 | 512 | 1/4 | 20% |
| 5G NR (15 kHz) | 4096 | 288 | 1/14.2 | 7% |
| DVB-T | 2048 | 256 | 1/8 | 11.1% |

### 3.7.3 Trade-offs

Longer CP:

- ✓ Better ISI protection

- ✓ More sync tolerance

- ✗ Reduced efficiency

- ✗ Lower data rate

## Summary

This document has provided a comprehensive exploration of OFDM system architecture. Key takeaways:

- OFDM Transmitter: S/P, modulation, IFFT, P/S, CP insertion

- OFDM Receiver: Inverse operations plus sync and equalization

- S/P Conversion: Reduces symbol rate by factor of N

- Modulation: BPSK, QPSK, 16-QAM, 64-QAM trade efficiency vs robustness

- IFFT/FFT: Efficient O(N log N) frequency-time conversion

- Cyclic Prefix: Critical for ISI elimination and orthogonality

Understanding these components is fundamental for implementing OFDM in Wi-Fi, LTE, and 5G systems.

# 4. Cyclic Prefix (CP) - Deep Dive

The Cyclic Prefix (CP) is arguably the most ingenious innovation in OFDM that enables its practical deployment in real-world wireless channels. While orthogonality provides the theoretical foundation for OFDM's spectral efficiency, it is the cyclic prefix that transforms OFDM from a fragile mathematical concept into a robust communication scheme capable of operating in harsh multipath environments. This section provides an exhaustive analysis of CP mechanics, design trade-offs, and implementation in modern standards.

## 4.1 What is Cyclic Prefix?

Definition: The Cyclic Prefix is a guard interval created by copying the last portion of an OFDM symbol and prepending it to the beginning of the same symbol before transmission. Mathematically, if the OFDM symbol has N samples indexed from 0 to N-1, the CP consists of the last LCP samples (from index N-LCP to N-1) duplicated and placed at the start.

Figure 4.1: Cyclic Prefix Creation Process

Key Characteristics:

Repetition Structure: The CP is not random padding but an exact copy of the tail end of the symbol, creating a periodic extension.

Overhead Cost: The CP increases symbol duration from T to (T + T_CP), reducing effective data rate by a factor of T/(T + T_CP).

Discarded at Receiver: After serving its purpose of absorbing channel effects, the CP is removed before FFT processing, so it carries no data.

Length Requirement: T_CP ≥ τ_max, where τ_max is the maximum multipath delay spread of the channel.

## 4.2 Why CP is Essential

In wireless communication, transmitted signals propagate through multiple paths (reflections, diffractions, scattering) before reaching the receiver. These multipath components arrive at different times, creating delayed replicas of the transmitted symbol. Without a CP, these delayed replicas would cause two catastrophic problems:

### Problem 1: Inter-Symbol Interference (ISI)

When a multipath component from OFDM symbol k-1 arrives during the reception window of symbol k, it contaminates symbol k with residual energy from the previous symbol. This overlap destroys the integrity of symbol k.

Figure 4.2: ISI Without Cyclic Prefix

Solution with CP: By inserting a guard interval (CP) between symbols that is longer than the maximum delay spread, any multipath components from the previous symbol will arrive within the CP duration and can be discarded. The useful symbol portion remains uncontaminated.

Figure 4.3: ISI Eliminated With Cyclic Prefix

### Problem 2: Inter-Carrier Interference (ICI)

OFDM subcarriers are spaced exactly 1/T apart to maintain orthogonality. Orthogonality relies on integrating each subcarrier over exactly one full period T. Multipath delays cause the received signal integration window to be misaligned, breaking orthogonality and causing subcarriers to interfere with each other.

Mathematical Insight: For two subcarriers with frequencies fk and fm to be orthogonal, the following must hold:

∫₀ᵀ exp(j2πfkt) × exp(-j2πfmt) dt = { T  if k=m; 0  if k≠m }

If the integration window is shifted or truncated due to multipath, this equality breaks down, causing ICI. The cyclic nature of the CP ensures that even with a delayed signal, the integration is performed over a complete period of all subcarriers, preserving orthogonality.

Figure 4.4: Orthogonality Preservation Mechanism

## 4.3 CP Length Calculation

The CP length must be carefully chosen based on channel characteristics and system requirements. The fundamental requirement is:

TCP ≥ τmax + τsync

Where:

τ_max = Maximum multipath delay spread (channel-dependent)

τ_sync = Timing synchronization tolerance (typically small compared to τ_max)

### Step-by-Step Calculation Example (LTE 20 MHz)

Step 1: Determine Channel Delay Spread

For urban environments, typical RMS delay spread is 1-3 μs. The maximum delay spread τ_max is approximately 3-5 times the RMS delay. For design purposes, assume τ_max = 4.7 μs (LTE normal CP target).

Step 2: Calculate Symbol Duration

LTE 20 MHz uses FFT size N = 2048 with subcarrier spacing Δf = 15 kHz.

T = 1/Δf = 1/15000 = 66.67 μs

Step 3: Determine CP Sample Count

The sampling rate is f_s = 30.72 MHz (for 20 MHz bandwidth). Number of CP samples:

L_CP = ⌈T_CP × f_s⌉ = ⌈4.7 × 10⁻⁶ × 30.72 × 10⁶⌉ = ⌈144.38⌉ = 144 samples

Step 4: Calculate Overhead

CP overhead is the fraction of symbol duration consumed by the guard interval:

Overhead = L_CP / (N + L_CP) = 144 / (2048 + 144) ≈ 6.57%

## 4.4 ISI (Inter-Symbol Interference) Mitigation

Inter-Symbol Interference occurs when the tail of one OFDM symbol overlaps with the beginning of the next symbol due to channel delay spread. The CP acts as a sacrificial buffer zone that absorbs these delayed components.

### Mathematical Proof of ISI Elimination

Consider the transmitted signal s(t) and the multipath channel h(τ) with maximum delay τ_max. The received signal r(t) is the convolution:

r(t) = ∫₀ᵗᵐᵃˣ h(τ) s(t-τ) dτ

Without CP: If we sample r(t) starting at t=0, components from the previous symbol (negative time) leak into the current symbol window [0, T], causing ISI.

With CP: The receiver delays sampling by T_CP and extracts the interval [T_CP, T_CP + T]. Since T_CP ≥ τ_max, all delayed components from the previous symbol have fully arrived by t = T_CP, and discarding the CP eliminates them entirely.

Figure 4.5: OFDM Transmission Flow with CP Integration

## 4.5 ICI (Inter-Carrier Interference) Prevention

The cyclic structure of the CP transforms the linear convolution of the channel into a circular convolution, which is diagonalized by the FFT. This property is critical for maintaining subcarrier orthogonality.

### Circular Convolution Property

In the frequency domain, the received signal on subcarrier k can be expressed as:

Y[k] = H[k] × X[k] + W[k]

Where:

Y[k] = received symbol on subcarrier k

H[k] = frequency response of the channel at subcarrier k (single complex value)

X[k] = transmitted symbol on subcarrier k

W[k] = additive white Gaussian noise

This simple one-tap equalization Y[k]/H[k] = X[k] + W[k]/H[k] is only possible because the CP ensures that the channel acts independently on each subcarrier. Without CP, H[k] would be a full matrix coupling all subcarriers, requiring complex matrix inversion for equalization.

## 4.6 CP Overhead Trade-offs

Selecting the optimal CP length involves balancing robustness against spectral efficiency. A longer CP provides better protection against multipath but reduces the data rate, while a shorter CP maximizes throughput but increases vulnerability to ISI/ICI.

Figure 4.6: CP Length Optimization Curve

### Quantitative Analysis

Spectral Efficiency Loss:

η_CP = T / (T + T_CP) = N / (N + L_CP)

Example Comparison:

| Scenario | CP Length | Overhead | Use Case |
| --- | --- | --- | --- |
| Short CP (Indoor) | 0.8 μs | 3-5% | Wi-Fi indoor |
| Normal CP (Urban) | 4.7 μs | 6.7% | LTE/5G standard |
| Extended CP (Rural) | 16.7 μs | 20% | High-speed rail |

## 4.7 Real-World CP Examples (LTE, 5G NR, Wi-Fi)

Different wireless standards have adopted CP strategies optimized for their specific use cases, channel environments, and latency requirements. This section examines how leading standards implement cyclic prefix.

### 4.7.1 LTE (3GPP TS 36.211)

LTE defines two CP configurations:

Normal CP (Standard Urban/Suburban):

FFT Size: N = 2048 (20 MHz), 1024 (10 MHz), 512 (5 MHz), etc.

Subcarrier Spacing: Δf = 15 kHz

Symbol Duration: T = 66.67 μs

CP Length: First symbol L_CP = 160 samples (5.2 μs), remaining 6 symbols L_CP = 144 samples (4.7 μs)

Slot Structure: 7 OFDM symbols per 0.5 ms slot

Overhead: ≈ 6.7% (average across 7 symbols)

Extended CP (Rural/High-Speed Scenarios):

CP Length: Uniform L_CP = 512 samples (16.67 μs)

Slot Structure: 6 OFDM symbols per 0.5 ms slot

Overhead: ≈ 20% (sacrifices efficiency for extreme delay spreads)

Use Case: High-speed trains (Doppler resilience), mountainous terrain

### 4.7.2 5G NR (3GPP TS 38.211)

5G NR introduces flexible numerology, allowing different subcarrier spacings (Δf = 15, 30, 60, 120, 240 kHz) to optimize for diverse use cases. CP length scales inversely with subcarrier spacing:

Numerology μ=0 (Δf = 15 kHz): Identical to LTE, backward compatibility

Numerology μ=1 (Δf = 30 kHz): T = 33.33 μs, T_CP = 2.34 μs (most common for sub-6 GHz)

Numerology μ=2 (Δf = 60 kHz): T = 16.67 μs, T_CP = 1.17 μs (mmWave, shorter delays)

Numerology μ=3 (Δf = 120 kHz): T = 8.33 μs, T_CP = 0.59 μs (ultra-low latency applications)

Key Insight: Higher subcarrier spacing reduces symbol duration and thus CP overhead in absolute time, but maintains similar relative overhead (≈7%). This enables 5G to support latency-critical applications (URLLC) while preserving ISI immunity.

### 4.7.3 Wi-Fi (IEEE 802.11)

802.11a/g/n/ac (Legacy Wi-Fi):

FFT Size: N = 64

Subcarrier Spacing: Δf = 312.5 kHz (wide spacing for indoor channels)

Symbol Duration: T = 3.2 μs

CP Length: T_CP = 0.8 μs (16 samples, suitable for indoor delay spreads < 500 ns)

Overhead: 20% (higher than cellular, but indoor channels are benign)

802.11ax (Wi-Fi 6):

Introduces longer OFDM symbols (13.6 μs vs 3.2 μs) with 4× longer CP (3.2 μs)

Benefits: Improved outdoor performance, better Doppler tolerance, reduced overhead to ≈19%

Trade-off: 4× latency increase (acceptable for typical Wi-Fi use cases)

## 4.8 C++ Implementation

Below is a production-grade C++ implementation of cyclic prefix addition and removal, following best practices for numerical stability and computational efficiency.

// C++ Implementation: Cyclic Prefix Addition

void addCyclicPrefix(

const vector<complex<double>>& ofdm_symbol,

int cp_length

) {

int N = ofdm_symbol.size();

vector<complex<double>> output(N + cp_length);

// Copy last cp_length samples to the beginning

for (int i = 0; i < cp_length; ++i) {

output[i] = ofdm_symbol[N - cp_length + i];

}

// Copy the entire OFDM symbol after CP

for (int i = 0; i < N; ++i) {

output[cp_length + i] = ofdm_symbol[i];

}

return output;

}

// Complexity: O(N + L_CP) time, O(N + L_CP) space

// Typical values: N=2048 (LTE 20MHz), L_CP=144 (normal CP)

## 4.9 Summary and Key Takeaways

This section has provided a comprehensive analysis of the cyclic prefix:

The CP is a guard interval created by copying the last L_CP samples to the symbol beginning

Primary functions: Eliminate ISI, preserve orthogonality, enable simple one-tap equalization

Design requirement: T_CP ≥ τ_max (maximum channel delay spread)

Trade-off: Longer CP improves robustness but reduces spectral efficiency

LTE: Normal CP ≈ 6.7% overhead, Extended CP ≈ 20% overhead

5G NR: Flexible numerology enables adaptive CP for diverse scenarios

Wi-Fi: Larger overhead (≈20%) acceptable for short-range indoor communication

The next section will explore channel effects, equalization techniques, and practical MIMO integration with OFDM systems.

# 5. Channel Effects and OFDM

In real-world wireless communication systems, transmitted signals do not travel in a straight line through a perfect medium. Instead, they interact with the physical environment—reflecting off buildings, diffracting around obstacles, and scattering from irregular surfaces. This results in multiple versions of the signal arriving at the receiver at different times, with different phases and amplitudes. This phenomenon, known as multipath propagation, creates one of the most challenging problems in wireless communications: frequency-selective fading.

OFDM was specifically designed to overcome these channel impairments. By converting a wideband channel into many narrowband sub-channels (subcarriers), each experiencing approximately flat fading, OFDM sidesteps the complexity of wideband equalization. When combined with the Cyclic Prefix (Section 4), OFDM enables simple, elegant one-tap equalization in the frequency domain—a key reason for its dominance in modern wireless standards including 4G LTE, 5G NR, and Wi-Fi.

This section provides a rigorous treatment of channel models, fading phenomena, Doppler effects, channel estimation techniques, and equalization algorithms, grounded in 3GPP standardization and real deployment considerations.

## 5.1 Multipath Fading Channels

Multipath fading arises when the transmitted signal reaches the receiver via multiple propagation paths. Each path has a distinct delay τᵢ, amplitude αᵢ, and phase φᵢ. The superposition of these components at the receiver creates a composite signal that fluctuates in amplitude and phase as a function of frequency and time.

### 5.1.1 Channel Impulse Response (CIR)

The time-varying channel impulse response is the fundamental mathematical description of a multipath channel. It characterizes how the channel transforms any input signal.

h(τ, t) = Σᵢ αᵢ(t) · e^{-jφᵢ(t)} · δ(τ - τᵢ(t)) Where: αᵢ = amplitude of the i-th path φᵢ = phase of the i-th path τᵢ = delay of the i-th path L = total number of multipath components

Figure 5.1: Multipath Propagation Environment TRANSMITTER (BS) RECEIVER (UE) ┌───┐ ┌───┐ │ Tx│══════════ Direct Path ═══════════│ Rx│ └───┘ (τ₀ = 0) └───┘ ║ / ║ ║ ╔══════════════════════════════╗ ║ ║══►║ Building Reflection (τ₁) ║═══►║ ║ ╚══════════════════════════════╝ ║ ║ ║ ╠═══════ Ground Reflection (τ₂) ════════╣ ║ ║ ╚══════ Diffraction around corner ══════╝ (τ₃ > τ₂ > τ₁ > τ₀) Received Signal: r(t) = Σᵢ αᵢ · s(t - τᵢ) [sum of delayed copies] Power Delay Profile (PDP): shows αᵢ² vs τᵢ RMS Delay Spread: στ = sqrt(Σᵢ αᵢ² (τᵢ - τ̄)² / Σᵢ αᵢ²)

Figure 5.1: Multipath propagation creates delayed replicas of the transmitted signal

### 5.1.2 Power Delay Profile (PDP) and Delay Spread

The Power Delay Profile (PDP) characterizes the power distribution of multipath components as a function of delay. It is a key statistical descriptor of the channel.

| Channel Parameter | Indoor (Office) | Urban (Macro) | Rural (Highway) |
| --- | --- | --- | --- |
| RMS Delay Spread (στ) | 10–50 ns | 100–1000 ns | 0.1–2 μs |
| Max Delay (τ_max) | < 100 ns | < 5 μs | < 20 μs |
| Coherence BW (Bc) | 10–50 MHz | 100 kHz–2 MHz | < 100 kHz |
| Doppler Spread (fd) | < 5 Hz | 10–200 Hz | 200–2000 Hz |
| 3GPP Channel Model | InH-Office (TR 38.901) | UMa / UMi (TR 38.901) | RMa (TR 38.901) |

Table 5.1: Channel delay spread and Doppler characteristics for different environments (3GPP TR 38.901)

📋 3GPP / Standards Reference 3GPP TR 38.901 (Section 7.5): Defines standardized channel models for 5G NR frequency ranges 0.5–100 GHz. Channel scenarios: InH (Indoor Hotspot), UMa (Urban Macro), UMi (Urban Micro), RMa (Rural Macro), InF (Indoor Factory). Each scenario specifies: delay spread distributions, angular spreads, K-factor (LOS/NLOS), cross-correlation parameters. 3GPP TS 38.211 Section 7.4: OFDM numerology selection is guided by delay spread requirements.

## 5.2 Frequency Selective Fading

Frequency selective fading is the defining channel impairment that makes wideband equalization difficult—and that OFDM elegantly solves. It occurs when the signal bandwidth exceeds the coherence bandwidth of the channel.

### 5.2.1 Flat vs. Frequency Selective Fading

Figure 5.2: Flat Fading vs. Frequency-Selective Fading FLAT FADING (BW << Bc): FREQUENCY SELECTIVE FADING (BW >> Bc): ───────────────────── ────────────────────────────────────── ¦H(f)¦ ¦H(f)¦ │ ┌──────────────────┐ │ ╲__ ╱╲ ╱──╲ ╱╲ ╱ │ │ ~constant │ │ ╲ ╱ ╲ ╱ ╲╱ ╲╱ │ │ │ │ ╲/ varies dramatically └──┼───────────────────┼──► f └────────────────────────────────► f │←── Signal BW ───→│ │←─────────── Signal BW ─────────→│ BW << Bc │←Bc→│ BW >> Bc Channel response: Channel response: Y[k] = H · X[k] Y[k] = H[k] · X[k] (different H per subcarrier!) (single H for all frequencies) H[k] = Σᵢ αᵢ e^{-j2πkΔf·τᵢ}

Figure 5.2: Frequency-selective fading distorts each frequency bin differently, requiring per-subcarrier equalization

### 5.2.2 Coherence Bandwidth and Subcarrier Design

The coherence bandwidth (Bc) determines the maximum subcarrier spacing for which each OFDM subcarrier experiences approximately flat fading. This is a fundamental design constraint in OFDM system design.

Coherence Bandwidth: Bc ≈ 1 / (5 · στ) [90% correlation definition] or Bc ≈ 1 / (2π · στ) [50% correlation definition (commonly used)] Design Criterion: Δf (subcarrier spacing) << Bc → flat fading per subcarrier 5G NR Example (Urban, στ ≈ 100 ns): Bc ≈ 1 / (5 × 100×10⁻⁹) = 2 MHz Δf = 30 kHz << 2 MHz ✓ (each subcarrier experiences flat fading) Wi-Fi 6 Example (Indoor, στ ≈ 30 ns): Bc ≈ 1 / (5 × 30×10⁻⁹) = 6.67 MHz Δf = 78.125 kHz << 6.67 MHz ✓

| Standard / Numerology | Subcarrier Spacing Δf | Fading Type per SC | Target Environment | 3GPP Reference |
| --- | --- | --- | --- | --- |
| LTE (μ = 0) | 15 kHz | Flat (≈ 2 MHz < Bc) | Urban / Suburban | TS 36.211 §5.6 |
| 5G NR (μ = 1) | 30 kHz | Flat (sub-6 GHz) | Urban / Dense | TS 38.211 §4.4.3 |
| 5G NR mmWave (μ = 3) | 120 kHz | Flat (mmWave, short delay) | Indoor / mmWave | TS 38.211 §4.4.3 |
| Wi-Fi 6 (802.11ax) | 78.125 kHz | Flat (indoor) | Indoor / Outdoor | IEEE 802.11ax §27 |

Table 5.2: OFDM subcarrier spacing selection ensures flat fading per subcarrier in the target environment

## 5.3 Doppler Shift Effects

Doppler shift occurs when the transmitter or receiver is in motion relative to the channel scatterers. The resulting frequency shift destroys the orthogonality between OFDM subcarriers if not properly accounted for, leading to Inter-Carrier Interference (ICI)—a phenomenon distinct from the ISI addressed by the cyclic prefix.

### 5.3.1 Doppler Frequency and Coherence Time

Doppler Shift: fD = (v / λ) · cos(θ) = (v · fc) / c · cos(θ) Maximum Doppler: fD_max = v · fc / c Where: v = mobile velocity (m/s), fc = carrier frequency (Hz) c = speed of light = 3×10⁸ m/s, θ = angle of arrival Coherence Time: Tc ≈ 0.423 / fD_max [Clarke's model, 50% correlation] Condition for OFDM: Ts (OFDM symbol duration) << Tc → quasi-static channel

Figure 5.3: Doppler Effect and ICI in OFDM Without Doppler: With Doppler (ICI generation): ───────────────── ──────────────────────────── ¦H(f)¦ per subcarrier: Doppler shifts energy across subcarriers: f0 f1 f2 f3 f4 f5 f0 f1 f2 f3 f4 f5 │ │ │ │ │ │ │ │ │ │ │ │ ╔╗ ╔╗ ╔╗ ╔╗ ╔╗ ╔╗ ╔╗╔╗ ╔╗╔╗ ╔╗╔╗ ╔╗╔╗ ╔╗╔╗ ╔╗ ║║ ║║ ║║ ║║ ║║ ║║ ╚╝║║ ║║╚╝ ╚╝║║ ║║╚╝ ╚╝║║ ║║ no overlap = orthogonal overlapping = ICI contamination ICI power ∝ (fD / Δf)² → higher speed or lower Δf worsens ICI 5G NR High Speed Train (350 km/h at 3.5 GHz): fD_max = (350/3.6) × 3.5×10⁹ / (3×10⁸) = 1138 Hz Δf = 30 kHz >> fD → ICI level ≈ -35 dB below signal ✓ acceptable

Figure 5.3: Doppler shift causes ICI by spreading spectral energy across adjacent subcarriers

### 5.3.2 Doppler Analysis Across Standards

| Scenario | Speed (km/h) | fc (GHz) | fD_max (Hz) | Tc (ms) | 5G NR Recommendation |
| --- | --- | --- | --- | --- | --- |
| Pedestrian | 3 | 3.5 | 9.7 Hz | 43.6 ms | μ=0 (15 kHz) |
| Urban Vehicle | 60 | 3.5 | 194 Hz | 2.18 ms | μ=1 (30 kHz) |
| High Speed Train | 350 | 3.5 | 1138 Hz | 0.37 ms | μ=2/3 (60-120 kHz) |
| mmWave Vehicular | 120 | 28 | 3111 Hz | 0.14 ms | μ=3/4 (120-240 kHz) |

Table 5.3: Doppler characteristics and 5G NR numerology selection (3GPP TS 38.211 Table 4.2-1)

📋 3GPP / Standards Reference 3GPP TS 38.211 Table 4.2-1: Defines subcarrier spacings μ=0,1,2,3,4 (15/30/60/120/240 kHz). Higher numerology (larger Δf) is mandated for higher Doppler scenarios to maintain ICI < threshold. 3GPP TR 38.901 Table 7.3-1: V2X channel (ITS-5.9 GHz) specifies fD_max up to 1190 Hz. TS 38.211 §7.3.1: PDCCH/PDSCH use cyclic prefix length that scales inversely with subcarrier spacing.

## 5.4 Channel Equalization in OFDM

Equalization is the process of reversing the distortion introduced by the channel. In traditional single-carrier systems, this requires a complex time-domain equalizer (e.g., a DFE or MLSE) that may span many symbol periods. OFDM's key advantage is that with the cyclic prefix in place, the channel simplifies to a single complex multiplication per subcarrier in the frequency domain.

### 5.4.1 Channel Model in Frequency Domain

After removing the cyclic prefix and applying the FFT, the OFDM received signal in the frequency domain is described by a beautifully simple model:

Y[k] = H[k] · X[k] + W[k] Where: k = subcarrier index (0, 1, ..., N-1) Y[k] = received frequency-domain symbol on subcarrier k H[k] = channel frequency response (CFR) at subcarrier k H[k] = Σᵢ hᵢ · e^{-j2πk·Δf·τᵢ} (DFT of CIR) X[k] = transmitted QAM/PSK symbol on subcarrier k W[k] = AWGN noise term This holds IF and ONLY IF: CP length ≥ max multipath delay (L_CP ≥ L_ch - 1)

Figure 5.4: OFDM System Model with Channel TRANSMITTER CHANNEL RECEIVER ─────────── ─────── ──────── X[k] ──► IFFT ──► + CP ──► h(τ,t) ──► - CP ──► FFT ──► Y[k] QAM time multipath remove freq. symbols domain channel CP domain X[0] s[n] r[n] Y[0] X[1] ◄──IFFT── ... ─channel─ ─FFT──► Y[1] ... ... ... ... X[N-1] Y[N-1] Frequency domain equation: Y[k] = H[k] · X[k] + W[k] Equalized output: X̂[k] = Y[k] / H[k] (if H[k] ≠ 0)

Figure 5.4: OFDM channel model showing how multipath becomes a per-subcarrier scalar multiplication

## 5.5 One-Tap Frequency Domain Equalization

The fundamental elegance of OFDM equalization lies in its simplicity: since each subcarrier experiences independent flat fading (characterized by a single complex number H[k]), the equalizer needs only one complex multiplication per subcarrier to undo the channel's effect. This is the 'one-tap equalizer'.

### 5.5.1 Zero Forcing (ZF) Equalizer

The Zero Forcing equalizer divides the received signal by the channel estimate to completely invert the channel. It is the simplest frequency-domain equalizer.

ZF Equalizer: X̂_ZF[k] = Y[k] / Ĥ[k] = X[k] + W[k] / H[k] Advantage: Perfect channel inversion (zero ISI/ICI when H[k] is exact) Disadvantage: Noise amplification when ¦H[k]¦ is small (deep fades) SNR_out[k] = SNR_in · ¦H[k]¦² → poor in nulls

### 5.5.2 Minimum Mean Square Error (MMSE) Equalizer

The MMSE equalizer balances channel inversion against noise enhancement, providing better performance than ZF in low SNR conditions. It is the preferred equalizer in practical systems.

MMSE Equalizer: X̂_MMSE[k] = Ĥ*[k] / (¦Ĥ[k]¦² + 1/SNR) · Y[k] Where: Ĥ*[k] = complex conjugate of channel estimate SNR = signal-to-noise ratio (per subcarrier, if known) When SNR >> 1: MMSE → ZF (channel inversion dominates) When SNR << 1: MMSE → MRC (noise suppression dominates) MMSE minimizes E[¦X̂[k] - X[k]¦²] (mean square error)

Figure 5.5: ZF vs MMSE Equalization Performance BER performance vs SNR: Equalizer comparison: ───────────────────── ───────────────────── BER ¦ 10⁻¹ ZF: simple, but noise amplified ¦ ╲ MMSE: better BER, minimal complexity ¦ 10⁻² ╲ ZF increase vs ZF ¦ ╲ ╲ MMSE ¦ 10⁻³ ╲ ╲ ¦ ╲ ╲ Ideal AWGN ¦ 10⁻⁴ ╲ ╲ ¦ ╲ ╲ └──────────────────────────► SNR (dB) 5 10 15 20 25 Complexity (both equalizers): O(N) multiplications ← huge advantage vs. O(N²) time-domain

Figure 5.5: MMSE equalizer provides 1–3 dB SNR gain over ZF, especially at low SNR and in deep fades

| Equalizer Type | Complexity | Noise Behavior | Deep Fade Perf. | Practical Use |
| --- | --- | --- | --- | --- |
| ZF (Zero Forcing) | O(N) — 1 div/SC | Amplified at nulls | Poor (noise boost) | Simple, high SNR links |
| MMSE | O(N) — 1 div+add/SC | Balanced (regularized) | Good (SNR-aware) | LTE/5G receivers |
| MRC (Matched Filter) | O(N) — 1 mult/SC | Maximized SNR | Best SNR, biased | MIMO combining |
| TD DFE (reference) | O(N·L) — high | ISI cancellation | Complex | Single-carrier legacy |

Table 5.4: OFDM frequency-domain equalizers — complexity comparison with single-carrier alternatives

## 5.6 Channel Estimation Techniques

Frequency-domain equalization requires knowledge of H[k] — the channel frequency response. In practice, H[k] is unknown and must be estimated from the received signal. Channel estimation is one of the most critical operations in any OFDM receiver and directly impacts system performance.

### 5.6.1 Pilot-Based Channel Estimation

The dominant approach in all modern standards is pilot-based estimation: known symbols (pilots) are inserted at specific time-frequency positions. The receiver uses these known values to estimate H[k] at pilot positions, then interpolates across all subcarriers.

Figure 5.6: Pilot Pattern in OFDM Time-Frequency Grid (3GPP Reference Signals) Frequency (subcarrier index) ↑ 12 │ D D D D D D D D D D D D │ 11 │ D D D D D D D D D D D D │ 10 │ D D D D D D D D D D D D │ 9 │ D D D D D D D D D D D D │ 8 │ D D D D D D D D D D D D │ 7 │ D D D D D D D D D D D D │ 6 │ D D D D D D D D D D D D │ 5 │ D D D D D D D D D D D D │ 4 │ D D D D D D D D D D D D │ 3 │[P] D D [P] D D [P] D D [P] D D │ ← DMRS row 2 │ D D D D D D D D D D D D │ 1 │ D D D D D D D D D D D D │ 0 │[P] D D [P] D D [P] D D [P] D D │ ← DMRS row └──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──→ OFDM symbol (time) 0 1 2 3 4 5 6 7 8 9 10 11 12 [P] = Pilot (DMRS/CRS) D = Data subcarrier Pilot spacing: Δf_p = 3 subcarriers, Δt_p = every N_s symbols

Figure 5.6: Pilot/DMRS pattern in OFDM resource grid — pilots enable channel interpolation across time and frequency

### 5.6.2 Least Squares (LS) Channel Estimation

At pilot positions k_p: Ĥ_LS[k_p] = Y[k_p] / X_pilot[k_p] Advantage: Simple, no noise statistics required MSE of LS: E[¦Ĥ_LS - H¦²] = σ²_noise / ¦X_pilot¦² (noise amplification) Typical pilot power boost: Pilot power ≥ Data power (3GPP: +6 dB for DMRS)

### 5.6.3 MMSE Channel Estimation

Ĥ_MMSE = R_HH · (R_HH + σ²_n/σ²_x · I)⁻¹ · Ĥ_LS Where: R_HH = channel autocorrelation matrix (requires statistical knowledge) σ²_n = noise power, σ²_x = pilot symbol power MSE reduction: MMSE achieves 10–15 dB lower MSE than LS in typical channels Trade-off: Requires channel statistics (R_HH) → estimated offline or adaptively

### 5.6.4 Interpolation Techniques

After estimating H[k] at pilot positions, interpolation recovers the full channel frequency response at all data subcarrier positions.

| Interpolation Method | Complexity | Accuracy | Use Case / Standard |
| --- | --- | --- | --- |
| Linear Interpolation | Very low O(N) | Good (slowly varying H) | LTE CRS-based (TS 36.211) |
| Cubic Spline | Low O(N·log N) | Better (fast fading) | 5G NR PDSCH (low mobility) |
| DFT-based (OFDM-IDFT) | Medium O(N·log N) | Excellent (exploit sparsity) | 5G NR high-perf. UE receivers |
| 2D Wiener Filter | High O(N_t·N_f·L) | Optimal (MMSE) | Research / advanced URLLC |

Table 5.5: Channel interpolation methods with complexity and accuracy trade-offs

📋 3GPP / Standards Reference 3GPP TS 38.211 §7.4.1: Defines PDSCH DMRS (Demodulation Reference Signals) patterns — Type 1 (single-port) and Type 2 (double-port CDM). TS 38.211 §7.4.1.1: DMRS are used for channel estimation; front-loaded DMRS placed at 2nd symbol for low-latency decoding. TS 38.214 §5.1.6.2: PDSCH DMRS configuration determines pilot density as a function of UE velocity category. TS 38.214 Table 5.1.6.2-1: For high-speed UE (>120 km/h), additional DMRS positions are configured to track faster channel variations. TS 38.211 §6.4.1: PUSCH DMRS patterns for uplink channel estimation (hopping and non-hopping configurations).

## 5.7 C++ Implementation: Channel Simulation and Equalization

The following production-grade C++ implementation demonstrates the complete chain: multipath channel simulation, pilot-based channel estimation, and one-tap frequency-domain equalization. The code uses the Eigen library for efficient complex matrix operations—standard practice in DSP implementations.

// OFDM_Channel.hpp — Multipath Channel Model #pragma once #include <complex> #include <vector> #include <cmath> #include <random> #include <Eigen/Dense> // sudo apt-get install libeigen3-dev #include <fftw3.h> // sudo apt-get install libfftw3-dev using Complex = std::complex<double>; using CplxVec = std::vector<Complex>; using RealVec = std::vector<double>; using CplxMatrix = Eigen::MatrixXcd; // ─── Multipath Channel Parameters ───────────────────────────────────── struct MultipathChannel { RealVec delays; // tap delays in samples RealVec amplitudes; // tap amplitudes (linear scale) RealVec phases; // tap phases in radians double doppler_hz; // maximum Doppler frequency in Hz double sampling_rate; // system sampling rate in Hz }; // ─── Predefined 3GPP Channel Models ─────────────────────────────────── MultipathChannel createEVA_Channel(double fs = 30.72e6) { // Extended Vehicular A (EVA) model — 3GPP TS 36.104 Table B.2-2 // Max delay spread: 2510 ns ¦ RMS delay spread: ~357 ns MultipathChannel ch; ch.sampling_rate = fs; // Path delays in ns: 0, 30, 150, 310, 370, 710, 1090, 1730, 2510 std::vector<double> delays_ns = {0, 30, 150, 310, 370, 710, 1090, 1730, 2510}; // Relative power in dB: 0, -1.5, -1.4, -3.6, -0.6, -9.1, -7.0, -12.0, -16.9 std::vector<double> power_dB = {0, -1.5, -1.4, -3.6, -0.6, -9.1, -7.0, -12.0, -16.9}; for (size_t i = 0; i < delays_ns.size(); ++i) { ch.delays.push_back(delays_ns[i] * 1e-9 * fs); // convert ns→samples ch.amplitudes.push_back(std::pow(10.0, power_dB[i] / 20.0)); ch.phases.push_back(0.0); // random per realization in practice } ch.doppler_hz = 70.0; // 70 Hz ≈ 72 km/h at 1 GHz return ch; } MultipathChannel createCDL_C_Channel(double fs = 30.72e6) { // CDL-C model — 3GPP TR 38.901 Table 7.7.2-3 // Clustered Delay Line C: RMS delay spread ~1014 ns (NLOS urban) MultipathChannel ch; ch.sampling_rate = fs; // Simplified 5-tap representation of CDL-C ch.delays = {0, 65*1e-9*fs, 200*1e-9*fs, 650*1e-9*fs, 2100*1e-9*fs}; ch.amplitudes = {0.7071, 0.5012, 0.3981, 0.1778, 0.1000}; ch.phases = {0.0, 0.3, 1.2, 2.1, 0.9}; ch.doppler_hz = 100.0; // ~108 km/h at 1 GHz return ch; }

// OFDM_Channel.cpp — Apply Channel Convolution + AWGN // Apply the multipath channel to an OFDM time-domain signal // Input: signal = OFDM samples (after IFFT + CP insertion) // Output: received = signal passed through multipath + noise CplxVec applyChannel(const CplxVec& signal, const MultipathChannel& ch, double snr_dB) { const size_t N = signal.size(); const size_t L = ch.delays.size(); CplxVec received(N, {0.0, 0.0}); // ── Step 1: Multipath convolution ───────────────────────────────── for (size_t tap = 0; tap < L; ++tap) { int delay_samples = static_cast<int>(std::round(ch.delays[tap])); double amp = ch.amplitudes[tap]; double phase = ch.phases[tap]; Complex h_tap = amp * Complex(std::cos(phase), std::sin(phase)); for (size_t n = delay_samples; n < N; ++n) { received[n] += h_tap * signal[n - delay_samples]; } } // ── Step 2: Add AWGN noise ───────────────────────────────────────── // Compute signal power double sig_power = 0.0; for (const auto& s : received) sig_power += std::norm(s); sig_power /= N; // Noise power from target SNR double snr_linear = std::pow(10.0, snr_dB / 10.0); double noise_sigma = std::sqrt(sig_power / (2.0 * snr_linear)); // Generate complex AWGN std::mt19937_64 rng(std::random_device{}()); std::normal_distribution<double> gauss(0.0, noise_sigma); for (size_t n = 0; n < N; ++n) { received[n] += Complex(gauss(rng), gauss(rng)); } return received; }

// OFDM_Equalizer.cpp — Pilot Estimation + MMSE Equalization // Extract channel estimate at pilot positions using LS // pilots_rx : received freq-domain samples at pilot indices // pilots_tx : known transmitted pilot symbols CplxVec lsChannelEstimate(const CplxVec& pilots_rx, const CplxVec& pilots_tx) { CplxVec H_pilots(pilots_tx.size()); for (size_t i = 0; i < pilots_tx.size(); ++i) { H_pilots[i] = pilots_rx[i] / pilots_tx[i]; // LS: Y_p / X_p } return H_pilots; } // Interpolate H from pilot positions to all subcarriers (linear) CplxVec interpolateChannel(const CplxVec& H_pilots, const std::vector<int>& pilot_indices, int N_subcarriers) { CplxVec H_full(N_subcarriers, {1.0, 0.0}); // default: identity int num_pilots = static_cast<int>(pilot_indices.size()); for (int i = 0; i < num_pilots - 1; ++i) { int k0 = pilot_indices[i], k1 = pilot_indices[i + 1]; Complex H0 = H_pilots[i], H1 = H_pilots[i + 1]; // Linear interpolation between consecutive pilots for (int k = k0; k <= k1; ++k) { double t = static_cast<double>(k - k0) / (k1 - k0); H_full[k] = H0 * (1.0 - t) + H1 * t; } } return H_full; } // MMSE one-tap frequency-domain equalizer // Ĥ_MMSE[k] = H*[k] / (¦H[k]¦² + 1/SNR) CplxVec mmseEqualizer(const CplxVec& Y, // received freq-domain const CplxVec& H_est, // channel estimate double snr_linear) { const size_t N = Y.size(); CplxVec X_hat(N); double inv_snr = 1.0 / snr_linear; for (size_t k = 0; k < N; ++k) { double H_mag2 = std::norm(H_est[k]); // ¦H[k]¦² Complex W_mmse = std::conj(H_est[k]) / (H_mag2 + inv_snr); X_hat[k] = W_mmse * Y[k]; } return X_hat; } // ── Complete OFDM Receiver (channel estimation + equalization) ───────── CplxVec ofdmReceive(const CplxVec& r_time, // time-domain received const std::vector<int>& pilot_idx, const CplxVec& pilot_symbols, int N, int L_cp, double snr_dB) { // 1. Remove cyclic prefix CplxVec r_ofdm(r_time.begin() + L_cp, r_time.begin() + L_cp + N); // 2. FFT: time-domain → frequency-domain CplxVec Y(N); fftw_complex* in = reinterpret_cast<fftw_complex*>(r_ofdm.data()); fftw_complex* out = reinterpret_cast<fftw_complex*>(Y.data()); fftw_plan p = fftw_plan_dft_1d(N, in, out, FFTW_FORWARD, FFTW_ESTIMATE); fftw_execute(p); fftw_destroy_plan(p); for (auto& y : Y) y /= static_cast<double>(N); // normalize FFT // 3. Extract pilots from received signal CplxVec Y_pilots(pilot_idx.size()); for (size_t i = 0; i < pilot_idx.size(); ++i) Y_pilots[i] = Y[pilot_idx[i]]; // 4. LS channel estimation at pilots CplxVec H_pilots = lsChannelEstimate(Y_pilots, pilot_symbols); // 5. Interpolate to all subcarriers CplxVec H_full = interpolateChannel(H_pilots, pilot_idx, N); // 6. MMSE equalization double snr_lin = std::pow(10.0, snr_dB / 10.0); return mmseEqualizer(Y, H_full, snr_lin); } // Complexity: O(N·log N) for FFT + O(N) for equalization = O(N·log N) total // Typical: N=2048 (LTE 20MHz) → ~22k operations per OFDM symbol

⚡ Implementation Notes — 3GPP Compliance • EVA channel model corresponds to 3GPP TS 36.104 Table B.2-2 (LTE) and TS 38.104 (5G NR) • CDL-C model is defined in 3GPP TR 38.901 Table 7.7.2-3 for NR system-level simulations • DMRS (pilot) density follows TS 38.211: 1/6 subcarriers in Type 1 DMRS configuration • SNR per subcarrier should be estimated from DMRS for adaptive MMSE coefficient update • Real receivers (e.g., in 5G NR UE) use iterative channel estimation + turbo equalization combining channel decoder feedback to refine H[k] across HARQ retransmissions

## 5.8 Summary and Key Takeaways

Section 5 has provided a comprehensive treatment of how wireless channel effects are handled in OFDM systems:

- Multipath Channel (§5.1): Real channels consist of multiple reflected paths with different delays and amplitudes. The Channel Impulse Response (CIR) captures this fully. RMS delay spread (στ) is the key channel parameter driving OFDM design.

- Frequency-Selective Fading (§5.2): When signal bandwidth exceeds the coherence bandwidth (Bc ≈ 1/5στ), different frequencies are attenuated differently. OFDM converts this into per-subcarrier flat fading by making Δf << Bc.

- Doppler Effects (§5.3): Mobile motion causes frequency shifts (Doppler) that create ICI if the OFDM symbol duration is too long. 5G NR addresses this with scalable numerology — higher subcarrier spacing at higher speeds.

- Frequency-Domain Equalization (§5.4–5.5): With the CP in place, the channel is a single scalar H[k] per subcarrier. ZF and MMSE equalization require only O(N) operations, vs O(N²) for time-domain equalizers.

- Channel Estimation (§5.6): Pilot/DMRS-based LS or MMSE estimation recovers H[k] at pilot positions; interpolation fills the remaining subcarriers. 3GPP defines standardized DMRS patterns for all NR channels.

- C++ Implementation (§5.7): Production-grade code for 3GPP EVA/CDL channel simulation, LS estimation, linear interpolation, and MMSE one-tap equalization is provided.

➡️ Up Next: Section 6 — OFDM Parameters and Design Section 6 builds on the channel understanding developed here to explain: • How to select FFT size, subcarrier count, and symbol duration for a given deployment • Spectral efficiency calculations under real channel constraints • PAPR (Peak-to-Average Power Ratio) — the main OFDM disadvantage and mitigation techniques • Complete parameter design examples for LTE, 5G NR, and Wi-Fi 6

## 5.9 Key Formulas Reference

| Formula | Expression | Reference |
| --- | --- | --- |
| Channel Impulse Response | h(τ,t) = Σᵢ αᵢ(t)·e^{-jφᵢ(t)}·δ(τ - τᵢ(t)) | Proakis, Digital Comms §14.1 |
| RMS Delay Spread | στ = √(Σ αᵢ²(τᵢ - τ̄)² / Σ αᵢ²) | 3GPP TR 38.901 §7.5 |
| Coherence Bandwidth | Bc ≈ 1 / (5·στ) [90% corr.] | Rappaport, Wireless Comms §5.4 |
| Max Doppler Shift | fD = v·fc / c · cos(θ) | ITU-R P.1238-9 |
| Coherence Time | Tc ≈ 0.423 / fD_max | Clarke, Bell Sys. Tech. J. 1968 |
| Freq-Domain OFDM Model | Y[k] = H[k]·X[k] + W[k] | 3GPP TS 38.211 §4.3 |
| ZF Equalizer | X̂[k] = Y[k] / Ĥ[k] | Standard DSP reference |
| MMSE Equalizer | X̂[k] = Ĥ*[k] / (¦Ĥ[k]¦² + 1/SNR) · Y[k] | Proakis §10.2.3 |
| LS Channel Estimate | Ĥ_LS[k_p] = Y[k_p] / X_p[k_p] | 3GPP TS 38.211 §7.4.1 |

Table 5.6: Section 5 formula summary with references

## 5.10 3GPP Standards Summary for Section 5

| Standard | Section / Clause | Relevance to Section 5 |
| --- | --- | --- |
| 3GPP TR 38.901 (Rel-15+) | §7.5 — Fast Fading Model Parameters | Defines delay spread, Doppler, CDL/TDL models for 5G NR simulation |
| 3GPP TS 38.211 (Rel-15+) | §4.3 — OFDM baseband signal; §7.4.1 — DMRS | Defines Y[k] = H[k]X[k] model; DMRS pilot patterns for channel estimation |
| 3GPP TS 36.104 (LTE) | Annex B — Channel models (EPA, EVA, ETU) | LTE channel models for equalization testing; EVA most widely used |
| 3GPP TR 25.943 | §5 — Deployment scenarios and channel conditions | Urban/rural classification used in multipath model selection |
| 3GPP TS 38.214 (Rel-15+) | §5.1.6 — PDSCH DMRS configuration | Defines DMRS density vs. UE velocity for 5G NR channel estimation |

Table 5.7: Key 3GPP standards and sections relevant to OFDM channel effects and equalization

# 6. OFDM PARAMETERS AND DESIGN

The design of an OFDM system involves careful selection and optimization of multiple interdependent parameters. Each parameter affects system performance in terms of data rate, spectral efficiency, robustness to channel impairments, implementation complexity, and power consumption. This section provides a comprehensive guide to OFDM parameter selection and system design.

Understanding the trade-offs between different parameters is crucial for designing OFDM systems that meet specific application requirements while operating within practical constraints.

## 6.1 Subcarrier Count Selection

The number of subcarriers (N) is one of the most fundamental parameters in OFDM design. It determines the FFT size and significantly impacts system performance, complexity, and flexibility.

### 6.1.1 Factors Affecting Subcarrier Count

Channel Characteristics:

Delay Spread: Higher delay spread requires longer symbols → more subcarriers

Doppler Spread: Higher Doppler requires shorter symbols → fewer subcarriers

Coherence Bandwidth: Subcarrier spacing should be < coherence bandwidth for flat fading

System Requirements:

Data Rate: Higher data rates may require more subcarriers

Latency: Low latency applications prefer fewer subcarriers (shorter symbols)

Spectral Efficiency: More subcarriers reduce guard band overhead

Implementation Constraints:

Computational Complexity: FFT complexity is O(N log N)

Memory Requirements: Larger N requires more memory

Synchronization: More subcarriers → more sensitive to frequency offset

### 6.1.2 Mathematical Relationships

Given bandwidth B and N subcarriers:

Subcarrier Spacing:

Δf = B / N

Useful Symbol Duration:

Tu = 1 / Δf = N / B

Maximum Data Rate (without coding):

Rmax = N × log₂(M) / Ts

where M = constellation size, Ts = total symbol duration

Common FFT Sizes and Their Characteristics (Assuming 20 MHz Bandwidth)

| FFT Size (N) | Subcarrier Spacing (kHz) | Symbol Duration (μs) | Complexity (Mults) | Use Case |
| --- | --- | --- | --- | --- |
| 64 | 312.5 | 3.2 | 192 | Wi-Fi 802.11a/g |
| 128 | 156.25 | 6.4 | 448 | Narrow bandwidth |
| 256 | 78.125 | 12.8 | 1,024 | Wi-Fi 802.11ac 20MHz |
| 512 | 39.06 | 25.6 | 2,304 | Medium bandwidth |
| 1024 | 19.53 | 51.2 | 5,120 | LTE partial |
| 2048 | 9.77 | 102.4 | 11,264 | LTE 20 MHz |
| 4096 | 4.88 | 204.8 | 24,576 | 5G NR |

## 6.2 Symbol Duration

Consists of Tu (useful) and Tcp (cyclic prefix).

ISI Immunity: Longer Tu robust to delay spread.
Doppler: Longer Tu sensitive to Doppler.
CP Overhead: Longer Tu reduces relative overhead.

Ts = Tu + Tcp
Rs = 1 / Ts = B / (N + Ncp)

### 6.2.1 Useful Symbol Duration (Tu)

Definition:

Tu = 1 / Δf = N / B

Design Considerations:

- ISI Immunity: Longer Tu makes system more robust to delay spread

- Doppler Sensitivity: Longer Tu increases sensitivity to Doppler/frequency offsets

- Latency: Longer Tu increases system latency

- CP Overhead: Longer Tu reduces relative CP overhead

### 6.2.2 Total Symbol Duration (Ts)

Calculation:

Ts = Tu + Tcp

Ts = (N + Ncp) / B

Symbol Rate:

Rs = 1 / Ts = B / (N + Ncp)

## 6.3 Subcarrier Spacing

Δf = B / N. Must be < Coherence BW for flat fading. fd,max < 0.1 * Δf for ICI.

Subcarrier spacing (Δf) determines the frequency separation between adjacent subcarriers and is inversely related to symbol duration. It's a key parameter in managing the trade-off between spectral efficiency and robustness.

### 6.3.1 Design Formula

Δf = B / N = 1 / Tu

### 6.3.2 Relationship to Channel Parameters

Coherence Bandwidth (Bc):

Bc ≈ 1 / (5 × τrms)

where τrms = RMS delay spread

Design Rule:

Δf << Bc    (for flat fading per subcarrier)

### 6.3.3 Doppler Considerations

Maximum Doppler Shift:

fd,max = v × fc / c

where v = velocity, fc = carrier frequency, c = speed of light

ICI Constraint:

fd,max < 0.1 × Δf    (for acceptable ICI)

Subcarrier Spacing and Mobility Support

| Standard | Subcarrier Spacing | Max Doppler (0.1×Δf) | Max Speed @ 2 GHz | Application |
| --- | --- | --- | --- | --- |
| LTE | 15 kHz | 1.5 kHz | 225 km/h | Cellular mobility |
| 5G NR μ=0 | 15 kHz | 1.5 kHz | 225 km/h | Wide area |
| 5G NR μ=1 | 30 kHz | 3 kHz | 450 km/h | High speed trains |
| 5G NR μ=2 | 60 kHz | 6 kHz | 900 km/h | V2X, aviation |
| 5G NR μ=3 | 120 kHz | 12 kHz | 1800 km/h | mmWave stationary |
| Wi-Fi 802.11a | 312.5 kHz | 31.25 kHz | 4687 km/h | Indoor stationary |

## 6.4 FFT Size Selection

Complexity = (N/2)log2(N). More subcarriers = finer resolution but higher complexity/power.

FFT size selection involves balancing computational complexity, frequency granularity, and system flexibility. Modern OFDM systems typically use power-of-2 FFT sizes to leverage efficient radix-2 FFT algorithms.

### 6.4.1 Selection Criteria

Primary Factors:

- Bandwidth Requirements: Wider bandwidth → larger FFT for adequate subcarrier count

- Frequency Selectivity: More subcarriers provide finer frequency resolution

- Computational Resources: Hardware capabilities limit maximum FFT size

- Power Consumption: Larger FFTs consume more power

- Latency Requirements: Smaller FFTs reduce processing latency

### 6.4.2 Computational Complexity

Radix-2 FFT Algorithm:

Complex Multiplications = (N/2) × log₂(N)

Complex Additions = N × log₂(N)

Memory Requirements:

RAM = 2 × N × (bits per sample)

(factor of 2 for complex numbers: I and Q)

FFT Computational Requirements

| FFT Size | Multiplications | Additions | RAM (16-bit) | Processing Time @ 1 GFLOP |
| --- | --- | --- | --- | --- |
| 64 | 192 | 384 | 256 bytes | 0.58 μs |
| 128 | 448 | 896 | 512 bytes | 1.34 μs |
| 256 | 1,024 | 2,048 | 1 KB | 3.07 μs |
| 512 | 2,304 | 4,608 | 2 KB | 6.91 μs |
| 1024 | 5,120 | 10,240 | 4 KB | 15.36 μs |
| 2048 | 11,264 | 22,528 | 8 KB | 33.79 μs |
| 4096 | 24,576 | 49,152 | 16 KB | 73.73 μs |

## 6.5 Bandwidth and Data Rate

R = N_used * log2(M) * Rc / Ts
Spectral Efficiency η = R / B

Understanding the relationship between bandwidth, modulation, coding, and achievable data rate is essential for OFDM system design.

### 6.5.1 Theoretical Data Rate

Without Channel Coding:

R = Nused × log₂(M) / Ts

where:

• Nused = number of data subcarriers

• M = modulation constellation size

• Ts = total symbol duration

With Channel Coding (Rate Rc):

R = Nused × log₂(M) × Rc / Ts

### 6.5.2 Spectral Efficiency

Definition:

η = R / B    (bits/s/Hz)

Expanded Form:

η = (Nused / N) × log₂(M) × Rc × (Tu / Ts)

= (Nused / N) × log₂(M) × Rc × (N / (N + Ncp))

### 6.5.3 Practical Example: LTE 20 MHz

System Parameters:

• Bandwidth: B = 20 MHz

• FFT Size: N = 2048

• Used Subcarriers: Nused = 1200

• CP Length: Ncp = 144 (normal CP)

• Modulation: 64-QAM (M = 64, 6 bits/symbol)

• Coding Rate: Rc = 5/6

Calculations:

Step 1: Subcarrier Spacing

Δf = 20 MHz / 2048 = 9.77 kHz

Step 2: Symbol Durations

Tu = 1 / 9.77 kHz = 102.4 μs

Tcp = 144 / 2048 × 102.4 μs = 7.2 μs

Ts = 102.4 + 7.2 = 109.6 μs

Step 3: Data Rate

R = 1200 × 6 × (5/6) / 109.6 μs

= 54.74 Mbps (per antenna)

Step 4: Spectral Efficiency

η = 54.74 Mbps / 20 MHz = 2.74 bits/s/Hz

LTE 20 MHz Calculation Summary

| Parameter | Formula | Value |
| --- | --- | --- |
| Subcarrier Spacing | Δf = B / N | 9.77 kHz |
| Useful Symbol Duration | Tu = 1 / Δf | 102.4 μs |
| CP Duration | Tcp = (Ncp/N) × Tu | 7.2 μs |
| Total Symbol Duration | Ts = Tu + Tcp | 109.6 μs |
| Symbol Rate | Rs = 1 / Ts | 9.124 ksymbols/s |
| Raw Data Rate | R = Nused × log₂(M) / Ts | 65.69 Mbps |
| Coded Data Rate | R = Nused × log₂(M) × Rc / Ts | 54.74 Mbps |
| Spectral Efficiency | η = R / B | 2.74 bits/s/Hz |

## 6.6 Spectral Efficiency

Spectral efficiency measures how effectively an OFDM system utilizes the available bandwidth. Maximizing spectral efficiency is crucial for spectrum-constrained wireless systems.

### 6.6.1 Factors Affecting Spectral Efficiency

1. Modulation Order:

η ∝ log₂(M)

Higher modulation increases bits per symbol but requires higher SNR

2. Cyclic Prefix Overhead:

Efficiency Loss = Ncp / (N + Ncp)

Typical overhead: 7-25% depending on channel conditions

3. Guard Bands:

Utilization = Nused / N

Typical utilization: 85-95% (more subcarriers = better utilization)

4. Channel Coding:

Coded Efficiency = Uncoded × Rc

Coding reduces data rate but improves reliability (typical Rc = 1/2 to 5/6)

### 6.6.2 Comparison Across Standards

Spectral Efficiency Comparison Across Standards

| Standard | Modulation | Coding Rate | CP Overhead | Guard Band | Peak η (bits/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| Wi-Fi 802.11a | 64-QAM | 3/4 | 20% | 18.75% | 3.6 |
| Wi-Fi 802.11n (20MHz) | 64-QAM | 5/6 | 12.5% | 12.5% | 4.3 |
| Wi-Fi 802.11ac (80MHz) | 256-QAM | 5/6 | 12.5% | 8.6% | 6.0 |
| Wi-Fi 802.11ax (HE160) | 1024-QAM | 5/6 | 12.5% | 4.3% | 7.2 |
| LTE (20 MHz) | 256-QAM | 0.93 | 7% | 41.4% | 5.0 |
| 5G NR (100 MHz) | 256-QAM | 0.95 | 7% | 10% | 6.5 |

### 6.6.3 Optimization Strategies

- Adaptive Modulation and Coding (AMC): Adjust M and Rc based on channel quality

- Minimize CP Overhead: Use smallest CP that satisfies delay spread requirement

- Maximize Subcarrier Utilization: Reduce guard bands through better filtering

- MIMO Techniques: Spatial multiplexing increases spectral efficiency

- Carrier Aggregation: Combine multiple bands to increase throughput

## 6.7 Peak-to-Average Power Ratio (PAPR)

Issue: High peaks reduce PA efficiency (typically drops to 20-30%). Increases complexity.
Reduction: Clipping, SLM, PTS, Tone Reservation, ACE.

| Technique | Reduction | Complexity | Loss | Impact |
| --- | --- | --- | --- | --- |
| Clipping | 3-4dB | Low | None | BER/Distortion |
| SLM | 3-5dB | High | Side Info | None |
| PTS | 4-6dB | Medium | Side Info | None |
| ACE | 2-3dB | Low | None | None |

PAPR is a critical issue in OFDM systems that affects power amplifier efficiency, battery life, and overall system cost. High PAPR occurs when multiple subcarriers align in phase, creating large instantaneous peaks.

### 6.7.1 PAPR Definition

Mathematical Definition:

PAPR = max(|x(t)|²) / E[|x(t)|²]

where:

• max(|x(t)|²) = peak instantaneous power

• E[|x(t)|²] = average power

In dB:

PAPR(dB) = 10 × log₁₀(PAPR)

### 6.7.2 PAPR Characteristics

Statistical Behavior:

- PAPR increases with number of subcarriers (N)

- Theoretical maximum: PAPR = N (occurs when all subcarriers align)

- Typical PAPR for OFDM: 10-13 dB

- Single carrier: PAPR ≈ 0-3 dB

### 6.7.3 Impact of High PAPR

1. Power Amplifier Efficiency:

- PA must operate with large back-off to avoid distortion

- Efficiency typically drops from 70% to 20-30%

- Higher power consumption and heat dissipation

2. Component Requirements:

- ADC/DAC need higher dynamic range

- Increased bit resolution required (12-14 bits vs 8-10 bits)

- Higher cost and complexity

3. Battery Life:

- Critical for mobile devices

- High PAPR reduces battery efficiency by 40-50%

### 6.7.4 PAPR Reduction Techniques

A. Clipping and Filtering:

- Simple: Limit peak amplitude to threshold

- Causes out-of-band radiation and BER degradation

- Typical PAPR reduction: 3-4 dB

B. Selective Mapping (SLM):

- Generate U different representations of same data

- Select representation with lowest PAPR

- PAPR reduction: 3-5 dB, complexity: U × FFT operations

C. Partial Transmit Sequence (PTS):

- Partition subcarriers into V sub-blocks

- Optimize phase factors for each sub-block

- PAPR reduction: 4-6 dB

D. Tone Reservation:

- Reserve specific subcarriers for PAPR reduction

- Generate cancellation signal on reserved tones

- No side information needed at receiver

E. Active Constellation Extension (ACE):

- Extend outer constellation points dynamically

- Maintains minimum distance between points

- No BER degradation

PAPR Reduction Techniques Comparison

| Technique | PAPR Reduction | Complexity | Data Rate Loss | BER Impact |
| --- | --- | --- | --- | --- |
| Clipping | 3-4 dB | Low | None | Moderate degradation |
| SLM | 3-5 dB | High (U×FFT) | Side info | None |
| PTS | 4-6 dB | Medium | Side info | None |
| Tone Reservation | 4-5 dB | Medium | ~5% | None |
| ACE | 2-3 dB | Low | None | None |
| Companding | 3-4 dB | Low | None | Slight degradation |

## 6.8 OFDM Parameter Design Methodology

1. Define Requirements
2. Determine BW
3. Select FFT Size
4. Calculate Δf/Tu
5. Design CP
6. Choose Modulation/Coding
7. Verify Performance

Designing an OFDM system requires a systematic approach that balances conflicting requirements and constraints. The following flowchart illustrates the design process:

### 6.8.1 Design Steps

Step 1: Define Requirements

- Target data rate

- Bit error rate (BER) / Packet error rate (PER)

- Maximum latency

- Mobility requirements (maximum speed)

- Coverage area and environment

Step 2: Determine Bandwidth

- Available spectrum allocation

- Regulatory constraints

- Shannon capacity: C = B × log₂(1 + SNR)

Step 3: Select FFT Size

- Balance between granularity and complexity

- Typical choices: 64, 128, 256, 512, 1024, 2048, 4096

- Larger N → better frequency selectivity, higher complexity

Step 4: Calculate Subcarrier Spacing and Symbol Duration

Δf = B / N

Tu = 1 / Δf

Step 5: Design Cyclic Prefix

- Measure or estimate channel delay spread (τmax)

- Select Tcp ≥ τmax

- Minimize overhead while maintaining ISI immunity

Step 6: Choose Modulation and Coding

- Based on required SNR and channel quality

- Consider adaptive modulation and coding (AMC)

- Link budget analysis

Step 7: Verify Data Rate and Performance

- Calculate achievable data rate

- Simulate BER performance

- Iterate if requirements not met

### 6.8.2 Design Trade-offs

- Symbol Duration vs Doppler: Longer Tu robust to ISI but sensitive to Doppler.
- CP vs Efficiency: Long CP protection vs low rate.
- FFT Size vs Complexity: granularity vs cost.

Key Trade-offs:

1. Symbol Duration vs Doppler Sensitivity

- Long symbols → robust to ISI but sensitive to Doppler

- Short symbols → robust to Doppler but vulnerable to ISI

2. CP Length vs Spectral Efficiency

- Long CP → better ISI protection, lower efficiency

- Short CP → higher efficiency, less ISI protection

3. FFT Size vs Complexity

- Large FFT → fine frequency resolution, high complexity

- Small FFT → low complexity, coarse frequency resolution

4. Modulation Order vs Robustness

- High-order → high data rate, requires high SNR

- Low-order → robust to noise, lower data rate

## Summary

This comprehensive guide has covered all critical aspects of OFDM parameter design and selection. Key takeaways include:

- Subcarrier Count: Balance between frequency granularity and computational complexity

- Symbol Duration: Trade-off between ISI immunity and Doppler sensitivity

- Subcarrier Spacing: Must be smaller than coherence bandwidth for flat fading

- FFT Size: Power-of-2 sizes leverage efficient FFT algorithms

- Data Rate: Function of bandwidth, modulation, coding, and overhead

- Spectral Efficiency: Maximized through high modulation, low overhead, and good subcarrier utilization

- PAPR: Critical challenge requiring careful mitigation for efficient PA operation

Successful OFDM system design requires understanding these interdependencies and making informed trade-offs based on application requirements, channel characteristics, and implementation constraints.

# 7.1 OFDM in Wi-Fi (IEEE 802.11a/g/n/ac/ax/be)

OFDM has been the cornerstone of Wi-Fi technology since IEEE 802.11a standardized in 1999. Each successive generation enhanced OFDM implementation to achieve higher data rates, better spectral efficiency, and improved performance in challenging RF environments. The evolution from 64-subcarrier systems to modern 4096-subcarrier implementations in Wi-Fi 7 demonstrates OFDM's remarkable scalability and adaptability.

Figure 7.1: OFDM Transmitter/Receiver Block Diagram (3GPP/IEEE 802.11)

## 7.1.1 Evolution of OFDM in Wi-Fi

| Standard | Year | FFT Size | Subcarrier Spacing | Channel BW | Max Data Rate |
| --- | --- | --- | --- | --- | --- |
| 802.11a/g | 1999/2003 | 64 | 312.5 kHz | 20 MHz | 54 Mbps |
| 802.11n | 2009 | 64/128 | 312.5 kHz | 20/40 MHz | 600 Mbps |
| 802.11ac | 2013 | 256/512 | 312.5 kHz | 20–160 MHz | 6.93 Gbps |
| 802.11ax (Wi-Fi 6) | 2021 | 1024/2048 | 78.125 kHz | 20–160 MHz | 9.6 Gbps |
| 802.11be (Wi-Fi 7) | 2024 | 4096 | 78.125 kHz | 20–320 MHz | 46 Gbps |

| KEY | The 4× reduction in subcarrier spacing from 312.5 kHz to 78.125 kHz in Wi-Fi 6 enabled longer symbol durations (12.8 μs vs 3.2 μs), dramatically improving robustness to delay spread while allowing smaller Resource Unit allocations for OFDMA multi-user scheduling. |
| --- | --- |

Figure 7.2: IEEE 802.11 Wi-Fi Standard Evolution — FFT Size and Data Rate Growth

## 7.1.2 IEEE 802.11ax (Wi-Fi 6) — OFDMA Implementation

Wi-Fi 6 revolutionized wireless LANs through OFDMA (Orthogonal Frequency Division Multiple Access), allowing multiple users to share the same channel simultaneously via Resource Units (RUs). This represents a fundamental shift from contention-based CSMA/CA to scheduled multi-user transmission, dramatically improving efficiency in dense deployment scenarios such as stadiums, airports, and enterprise environments.

### Key Technical Parameters

| Subcarrier Spacing | 78.125 kHz (4× narrower than 802.11ac) |
| --- | --- |
| FFT Size | 1024 for 80 MHz; 2048 for 160 MHz |
| Symbol Duration | 12.8 μs (4× longer than 802.11ac) |
| Guard Interval | 0.8 μs, 1.6 μs, 3.2 μs (flexible) |
| Maximum MCS | MCS 11 with 1024-QAM modulation |

### Resource Unit (RU) Allocation

| RU Size (Tones) | Bandwidth | Max Users (20 MHz) | Use Case |
| --- | --- | --- | --- |
| 26 | ~2 MHz | 9 | IoT, sensors, low-rate devices |
| 52 | ~4 MHz | 4 | Voice calls, video conferencing |
| 106 | ~8 MHz | 2 | Standard video streaming |
| 242 | ~20 MHz | 1 | High-throughput single user |
| 484/996 | 40/80 MHz | 1 | 4K/8K video, file transfers |

| EXAMPLE | In an 80 MHz channel, an Access Point can simultaneously serve 9 users by allocating one 26-tone RU to each, dramatically reducing latency and improving efficiency compared to traditional CSMA/CA where users contend for the entire channel. |
| --- | --- |

# 7.2 OFDM in 4G LTE (3GPP Release 8–15)

LTE adopted OFDMA for downlink transmission, marking a significant departure from CDMA-based 3G systems. The LTE OFDM implementation centers on a fixed 15 kHz subcarrier spacing, providing robust balance between spectral efficiency and mobility support across sub-6 GHz frequency bands. This design choice, validated by extensive field trials, enables operation from stationary users to high-speed trains (up to 350 km/h).

Figure 7.3: LTE Resource Grid Structure (3GPP TS 36.211) — 20 MHz Bandwidth

## 7.2.1 LTE OFDM Parameters (3GPP TS 36.211)

| Bandwidth | FFT Size | Resource Blocks | Subcarriers | Sample Rate | CP Duration |
| --- | --- | --- | --- | --- | --- |
| 1.4 MHz | 128 | 6 | 72 | 1.92 Msps | 5.21 / 4.69 μs |
| 3 MHz | 256 | 15 | 180 | 3.84 Msps | 5.21 / 4.69 μs |
| 5 MHz | 512 | 25 | 300 | 7.68 Msps | 5.21 / 4.69 μs |
| 10 MHz | 1024 | 50 | 600 | 15.36 Msps | 5.21 / 4.69 μs |
| 15 MHz | 1536 | 75 | 900 | 23.04 Msps | 5.21 / 4.69 μs |
| 20 MHz | 2048 | 100 | 1200 | 30.72 Msps | 5.21 / 4.69 μs |

### Resource Block Structure

Frequency Domain: 1 RB = 12 consecutive subcarriers = 180 kHz bandwidth

Time Domain: 1 RB = 1 slot = 7 OFDM symbols (normal CP) or 6 symbols (extended CP)

Total Resource Elements: 1 RB = 12 subcarriers × 7 symbols = 84 resource elements per slot

| CALC | For 20 MHz LTE bandwidth with 100 RBs, the eNodeB scheduler can allocate resources with 1 RB granularity (180 kHz) to different users every 1 ms subframe, enabling flexible multi-user scheduling with minimal resource fragmentation. |
| --- | --- |

## 7.2.2 Cyclic Prefix Configuration

| CP Type | Duration (μs) | Overhead per Symbol |
| --- | --- | --- |
| Normal CP (1st symbol) | 5.21 | 7.25% |
| Normal CP (other 6 symbols) | 4.69 | 6.57% |
| Extended CP (all 6 symbols) | 16.67 | 20.0% |
| Normal CP Average | 4.76 | 6.67% |

| DESIGN | The first symbol in each slot uses a longer CP (5.21 μs) to accommodate timing advance uncertainties when multiple UEs at different distances synchronize uplink transmissions. This asymmetric design enables efficient multi-user scheduling without inter-symbol interference while minimizing overall overhead. |
| --- | --- |

# 7.3 OFDM in 5G NR (3GPP Release 15+)

5G New Radio revolutionizes OFDM through flexible numerology, enabling a single air interface to serve diverse use cases from enhanced mobile broadband (eMBB) to ultra-reliable low-latency communications (URLLC) and massive machine-type communications (mMTC). The scalable numerology, where subcarrier spacing doubles with each numerology index (Δf = 2^μ × 15 kHz), is 5G NR's defining innovation, allowing seamless operation from sub-6 GHz to mmWave frequencies.

Figure 7.4: 5G NR Scalable Numerology — Δf = 2^μ × 15 kHz (3GPP TS 38.211 Section 4)

## 7.3.1 Scalable OFDM Numerology (3GPP TS 38.211 Section 4)

| μ | Δf (kHz) | Symbol (μs) | Slot (ms) | Frequency Range | Primary Use Case |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 | 66.67 | 1.0 | FR1 (sub-6 GHz) | Coverage, eMBB |
| 1 | 30 | 33.33 | 0.5 | FR1/FR2 | Balanced performance |
| 2 | 60 | 16.67 | 0.25 | FR2 (mmWave) | Low latency |
| 3 | 120 | 8.33 | 0.125 | FR2 (mmWave) | URLLC, <1 ms |
| 4 | 240 | 4.17 | 0.0625 | FR2 (mmWave) | Extreme mobility |

| KEY | Δf = 2^μ × 15 kHz. As μ increases, symbol duration decreases (T = 1/Δf), trading spectral efficiency for latency and Doppler tolerance. This exponential scaling enables seamless operation from static IoT devices (μ=0) to 500 km/h high-speed rail (μ=3,4), with each doubling of subcarrier spacing halving the symbol duration. |
| --- | --- |

## 7.3.2 Bandwidth Parts (BWP) and Mixed Numerology

5G NR introduces Bandwidth Parts, allowing a UE to operate on a subset of the carrier bandwidth with specific numerology. A single 100 MHz carrier can simultaneously support multiple BWPs with different μ values, enabling dynamic adaptation to traffic requirements, power constraints, and service types. This flexibility is unprecedented in cellular standards.

| BWP Type | Numerology (μ) | Bandwidth | Purpose |
| --- | --- | --- | --- |
| Initial BWP | 0 (15 kHz) | 20 MHz | Initial access, paging |
| Active BWP #1 | 1 (30 kHz) | 100 MHz | High-throughput data (eMBB) |
| Active BWP #2 | 2 (60 kHz) | 40 MHz | Low-latency data (URLLC) |
| Inactive BWP | 0 (15 kHz) | 5 MHz | Power saving, monitoring |

| UE ADAPT. | A UE can switch BWPs dynamically: from a wide 100 MHz BWP (μ=1) for high-throughput video streaming to a narrow 20 MHz BWP (μ=0) for power-efficient control signaling, reducing power consumption by up to 50% during idle periods while maintaining connectivity. |
| --- | --- |

# 7.4 OFDM in Digital Broadcasting (DVB-T/T2, ATSC 3.0)

OFDM revolutionized digital television broadcasting by enabling Single Frequency Networks (SFNs) where multiple transmitters broadcast identical content on the same frequency without mutual interference. This capability, impossible with traditional single-carrier systems, dramatically improves spectrum efficiency and coverage uniformity. OFDM's long symbol duration and configurable guard interval transform what would be destructive multipath into constructive combining.

Figure 7.5: DVB-T2 Single Frequency Network (SFN) — Guard Interval Enables 67 km Transmitter Spacing

## 7.4.1 DVB-T2 (Second Generation Terrestrial)

| Parameter | Options | Typical Value |
| --- | --- | --- |
| FFT Size | 1K, 2K, 4K, 8K, 16K, 32K | 32K (32768 subcarriers) |
| Guard Interval | 1/128, 1/32, 1/16, 1/8, 19/128, 1/4 | 1/4 (224 μs for 8 MHz) |
| Modulation | QPSK, 16-QAM, 64-QAM, 256-QAM | 256-QAM for high SNR |
| FEC Coding | LDPC + BCH | Code rate 2/3 to 5/6 |
| Bandwidth | 1.7, 5, 6, 7, 8, 10 MHz | 8 MHz (Europe) |
| Typical Throughput | Varies by configuration | ~45 Mbps (8 MHz ch.) |

| SFN DESIGN | With a 224 μs guard interval (1/4 mode, 32K FFT), transmitters can be up to 67 km apart (224 μs × 3×10⁸ m/s ≈ 67 km) before causing ISI. National broadcasters deploy hundreds of synchronized transmitters creating seamless coverage with a single frequency allocation. |
| --- | --- |

# 7.5 OFDM in DSL Technologies (VDSL2)

Digital Subscriber Line technologies adopted OFDM (termed Discrete Multi-Tone — DMT in DSL context) to achieve high-speed data transmission over existing copper telephone lines. OFDM's per-subcarrier adaptation is critical in the challenging DSL channel environment where attenuation varies dramatically across the 0–30 MHz spectrum, with high frequencies experiencing 60+ dB more loss than low frequencies over typical loop lengths.

Figure 7.6: VDSL2 Adaptive Bit Loading (ITU-T G.993.2) — 500 m Loop Example

## 7.5.1 VDSL2 (ITU-T G.993.2)

| Parameter | Specification | Notes |
| --- | --- | --- |
| FFT Size | 4096 subcarriers | Flexible tone allocation |
| Subcarrier Spacing | 4.3125 kHz | 17.664 MHz max spectrum |
| Frequency Range | 0–30 MHz (profile 30a) | Higher freq. attenuated |
| Modulation per Tone | 1–15 bits/symbol | Adaptive based on SNR |
| Symbol Rate | 4 kHz | 250 μs symbol period |
| Max Throughput | Up to 300 Mbps | Very short loops (<300 m) |

### Adaptive Bit Loading

Subcarriers in 0–5 MHz experience high SNR (>45 dB) and use 15-bit/symbol loading (equivalent to 32768-QAM). Meanwhile, subcarriers above 20 MHz, attenuated by cable loss and FEXT interference, may only support 2–4 bits/symbol (QPSK/16-QAM) or be disabled entirely. This fine-grained adaptation, updated every few hundred milliseconds based on channel measurements, maximizes throughput across the frequency-selective DSL channel spanning six orders of magnitude in attenuation.

# 7.6 Comparative Analysis Across Standards

While all standards employ OFDM as the fundamental waveform, their implementations vary significantly based on application requirements, channel characteristics, and design trade-offs. This comparison reveals universal principles and standard-specific optimizations.

Figure 7.7: Cross-Standard OFDM Capability Comparison (Normalized Metrics)

| Standard | Δf | FFT Size | Guard Interval | Key Feature |
| --- | --- | --- | --- | --- |
| Wi-Fi 6 (802.11ax) | 78.125 kHz | 256–2048 | 0.8–3.2 μs | OFDMA multi-user |
| Wi-Fi 7 (802.11be) | 78.125 kHz | 4096 | 0.8–3.2 μs | 320 MHz channels, MLO |
| LTE | 15 kHz | 128–2048 | 4.7–16.7 μs | Fixed numerology |
| 5G NR (μ=0) | 15 kHz | 256–4096 | 4.7 μs | Coverage layer |
| 5G NR (μ=3) | 120 kHz | 256–4096 | 0.59 μs | URLLC, mmWave |
| DVB-T2 | ~4.46 kHz | 1K–32K | Up to 224 μs | SFN support |
| VDSL2 | 4.3125 kHz | 4096 | 8–32 samples | Per-tone bit loading |

## Key Insights from Cross-Standard Analysis

1. Subcarrier Spacing vs Mobility: Wide spacing (15–240 kHz) in cellular for Doppler tolerance; narrow spacing (4–78 kHz) in broadcast/DSL/Wi-Fi for stationary channels.

2. FFT Size vs Channel Delay: Large FFT (4K–32K) in broadcast for SFN; moderate FFT (256–4096) in cellular for complexity-performance balance.

3. Guard Interval Philosophy: Broadcast maximizes GI (up to 1/4 symbol) for SFN. Cellular minimizes GI (7%) for efficiency. DSL varies GI with loop length.

4. Adaptive Modulation Granularity: Per-RB in cellular (180 kHz); per-RU in Wi-Fi (2–996 tones); per-subcarrier in DSL (4.3 kHz) for maximum adaptation.

# 7.7 Summary and Key Takeaways

This section explored OFDM implementations across diverse communication standards, revealing how fundamental OFDM principles adapt to specific application requirements while maintaining core mathematical properties of orthogonality and spectral efficiency.

## Core Insights

| WI-FI EV | Wi-Fi Evolution: Progressive FFT scaling (64 → 4096), OFDMA introduction, and 4× symbol duration increase culminating in Wi-Fi 7's 320 MHz channels and 46 Gbps throughput. |
| --- | --- |

| LTE FOUN | LTE Foundation: Fixed 15 kHz numerology optimized for sub-6 GHz, 12-subcarrier Resource Block structure, sophisticated reference signal design for MIMO channel estimation. |
| --- | --- |

| 5G NR RE | 5G NR Revolution: Scalable numerology (Δf = 2^μ × 15 kHz) enabling <1 ms latency with μ=3, Bandwidth Parts for UE power savings, mini-slots for URLLC applications. |
| --- | --- |

| BROADCAS | Broadcast Optimization: 32K FFT with long guard intervals enabling Single Frequency Networks, uniform modulation across spectrum, LDPC coding with >30 dB gains. |
| --- | --- |

| DSL ADAP | DSL Adaptation: Per-subcarrier bit loading across 4096 tones, vectoring for FEXT cancellation, dynamic 0–15 bits/subcarrier range adapting to channel conditions. |
| --- | --- |

## Universal OFDM Principles

| Principle | Description |
| --- | --- |
| Orthogonality Preservation | All standards maintain Δf · T = 1, ensuring zero inter-carrier interference at sampling instants. |
| Cyclic Prefix Trade-off | Balance between ISI immunity (longer CP) and overhead (shorter CP): 6–8% in cellular, 25% in broadcast SFNs. |
| Adaptive Techniques | All modern standards exploit frequency-selective fading through adaptive modulation/coding, with granularity from per-RB (cellular) to per-subcarrier (DSL). |
| PAPR Challenge | Universal 8–12 dB PAPR addressed through SC-FDMA (LTE uplink), tone reservation (DSL), amplifier linearization (broadcast). |

# 7.9 References

## 3GPP Technical Specifications

• 3GPP TS 38.211 V18.1.0 (2024-06) — NR; Physical channels and modulation

• 3GPP TS 36.211 V17.4.0 (2023-12) — E-UTRA; Physical channels and modulation

• 3GPP TS 38.214 V18.1.0 (2024-06) — NR; Physical layer procedures for data

• 3GPP TS 36.213 V17.4.0 (2023-12) — E-UTRA; Physical layer procedures

## IEEE Standards

• IEEE 802.11-2020 — Wireless LAN Medium Access Control and Physical Layer Specifications

• IEEE 802.11ax-2021 — Enhancements for High Efficiency WLAN

• IEEE 802.11be-2024 — Extremely High Throughput WLAN

## Broadcasting and DSL Standards

• ETSI EN 302 755 V1.4.1 (2015-07) — Digital Video Broadcasting (DVB-T2)

• ATSC A/322:2024 — Physical Layer Protocol (ATSC 3.0)

• ITU-T G.993.2 (2019) — Very high speed digital subscriber line transceivers 2 (VDSL2)

• ITU-T G.993.5 (2019) — Self-FEXT cancellation (vectoring) for VDSL2

# 8. Advanced OFDM Concepts

The preceding sections established the mathematical foundations, system architecture, cyclic prefix mechanics, and channel equalization of OFDM. This section advances into the frontier concepts that transform basic OFDM into the versatile, multi-user, high-capacity framework powering modern wireless networks. These are not incremental refinements — they represent fundamental architectural extensions that multiply spectral efficiency, improve uplink power efficiency, and enable massive antenna deployments.

The six advanced topics covered here are directly standardized in 3GPP Release 15 onwards for 5G New Radio: OFDMA for multi-user downlink, SC-FDMA for power-efficient uplink, MIMO-OFDM for spatial multiplexing, frequency-domain beamforming, pilot-aided channel estimation, and PAPR reduction techniques. Mastery of these concepts is essential for any engineer working on 5G NR, Wi-Fi 6, or next-generation wireless system design.

## 8.1 OFDMA (Orthogonal Frequency Division Multiple Access)

OFDMA is the multiple-access extension of OFDM. While OFDM allocates the entire bandwidth to a single user per OFDM symbol, OFDMA partitions the time-frequency resource grid into chunks called Resource Blocks (RBs) and assigns different RBs to different users simultaneously. This transforms OFDM from a point-to-point modulation into a full multi-user access scheme, and is the fundamental downlink technology in both 4G LTE and 5G NR.

### 8.1.1 Resource Block Structure

The 3GPP resource grid is organized in terms of Resource Blocks (RBs). Each RB is the smallest allocatable unit in the frequency domain. In the time domain, allocation is in units of slots.

Resource Block (RB): 12 subcarriers × 1 slot (14 OFDM symbols) Resource Element (RE): 1 subcarrier × 1 OFDM symbol [finest granularity] System Bandwidth (5G NR, μ=1, Δf=30kHz): 100 MHz → 66 RBs → 66×12 = 792 active subcarriers OFDMA Capacity: C = Σₖ B·log₂(1 + SNRₖ) [Shannon, per-user k] Multi-user gain: C_OFDMA ≥ max_k C_OFDM(k) [multi-user diversity]

Figure 8.1: OFDMA Time-Frequency Resource Grid — each colour represents a different User Equipment (UE). DMRS rows (red) carry pilot symbols for channel estimation. Reference: 3GPP TS 38.211 §7.3.1

### 8.1.2 OFDMA vs OFDM: Key Differences

| Parameter | OFDM (Single User) | OFDMA (Multi-User) |
| --- | --- | --- |
| Users per symbol | 1 | 1 to N_RB (up to 132 in 5G NR) |
| Allocation unit | All subcarriers | Resource Block (12 SCs × 14 symbols) |
| Scheduling | Not applicable | Dynamic per-slot scheduling by gNB/eNB |
| Multi-user diversity | None | Yes — exploit independent fading per user |
| Frequency reuse | Full band, one user | Fractional — each user sees different fading |
| 3GPP standard | Not a standard itself | LTE DL (TS 36.211), 5G NR DL (TS 38.211) |
| Wi-Fi standard | 802.11a/g/n/ac | 802.11ax (Wi-Fi 6) — OFDMA DL+UL |

Table 8.1: OFDM vs OFDMA — key differences and 3GPP standardization

### 8.1.3 Scheduling and Multi-User Diversity

OFDMA enables the gNB (5G base station) to exploit multi-user diversity: since different users experience different channel conditions at different frequencies and times, the scheduler can always assign each RB to the user who has the best channel for that RB. This frequency-selective scheduling gain can provide a significant throughput increase over a round-robin assignment.

📡 OFDMA Scheduling in 5G NR (3GPP TS 38.321) • MAC scheduler at gNB runs every slot (0.5 ms at 30 kHz SCS) to allocate RBs dynamically • Downlink Control Information (DCI) on PDCCH tells each UE its allocated RBs, MCS, and MIMO layers • Channel Quality Indicator (CQI) feedback from UE (5-bit value, 1–15) drives MCS selection per RB • Proportional Fair scheduler: maximizes system throughput while ensuring fairness across users • Maximum 132 RBs in 100 MHz (μ=1, 30 kHz) → 1584 active subcarriers simultaneously

📋 3GPP / Standards Reference 3GPP TS 38.211 §7.3.1: Defines the NR physical resource and Resource Block structure (12 subcarriers). 3GPP TS 38.214 §5.1.2: MCS table for PDSCH — 64 entries mapping CQI to modulation order and code rate. 3GPP TS 38.321 §6.1.2: MAC scheduling procedure — DCI format 1_0 and 1_1 for downlink allocation. 3GPP TS 38.213 §10: DCI processing and PDCCH monitoring for OFDMA scheduling decisions. IEEE 802.11ax §26.5: OFDMA resource unit (RU) sizes — 26, 52, 106, 242, 484, 996 subcarriers.

## 8.2 SC-FDMA (Single Carrier FDMA) for Uplink

SC-FDMA, officially named DFT-Spread OFDM (DFT-S-OFDM) in 3GPP standards, is the uplink multiple-access scheme for LTE and one of the uplink waveforms in 5G NR. It was specifically chosen over OFDMA for the uplink because mobile devices (UEs) have severely constrained power amplifiers — SC-FDMA achieves a dramatically lower Peak-to-Average Power Ratio (PAPR) by pre-processing data with a DFT before IFFT, effectively creating a single-carrier waveform in the time domain.

### 8.2.1 DFT-Spread OFDM Architecture

The critical difference between OFDMA and SC-FDMA is the insertion of an M-point DFT before the subcarrier mapping and N-point IFFT. This DFT 'spreads' each symbol across all M allocated subcarriers in the frequency domain, creating a time-domain signal that resembles a single-carrier waveform.

Figure 8.2: SC-FDMA (DFT-S-OFDM) transmitter architecture. The M-point DFT pre-coding step (purple) is the only addition over standard OFDMA, yet it dramatically reduces PAPR. Reference: 3GPP TS 38.211 §5.3

### 8.2.2 PAPR Reduction Mechanism

SC-FDMA Processing Chain: d[m] → M-DFT → D̃[k] → Subcarrier Map → N-IFFT → +CP → Transmit Time-domain output s[n] ≈ Single-carrier signal (PAPR ≈ 2–4 dB) vs OFDMA s[n] = sum of M sinusoids (PAPR ≈ 8–12 dB for M >> 1) PAPR Definition: PAPR = max¦s[n]¦² / E[¦s[n]¦²] SC-FDMA advantage: PAPR reduction ≈ 2–5 dB vs OFDMA Power amplifier efficiency gain: ~15–30% improvement in UE battery life

Figure 8.3: SC-FDMA vs OFDMA time-domain power comparison. SC-FDMA achieves ~2.5 dB lower PAPR, enabling more efficient power amplifier operation in UE handsets — critical for battery life. Reference: 3GPP TS 38.211 §5.3

### 8.2.3 SC-FDMA vs OFDMA Comparison

| Parameter | OFDMA | SC-FDMA | Impact |
| --- | --- | --- | --- |
| PAPR (64-QAM) | ~10–12 dB | ~5–7 dB | UE power amp efficiency ↑ |
| PA efficiency | ~30–40% | ~45–60% | Battery life ↑ ~20% |
| Coverage | Limited by PA back-off | Extended by 3–5 dB | Cell edge performance ↑ |
| Spectral efficiency | Higher (freq. flexibility) | Slightly lower | Downlink preferred OFDMA |
| Receiver complexity | Low (one-tap EQ) | Higher (IDFT needed) | gNB can handle complexity |
| 3GPP use | Downlink (DL) in LTE, NR | Uplink (UL) in LTE, NR | Asymmetric design |

Table 8.2: SC-FDMA vs OFDMA performance comparison (3GPP TS 36.211 / TS 38.211)

📋 3GPP / Standards Reference 3GPP TS 38.211 §5.3: Defines CP-OFDM and DFT-S-OFDM (SC-FDMA) waveforms for NR uplink. 3GPP TS 38.211 §5.3.2: DFT-S-OFDM baseband signal — transform precoding with M subcarriers. 3GPP TS 36.211 §5.6: LTE uplink SC-FDMA physical channels (PUSCH, PUCCH). 3GPP TS 38.214 §6.1.3: UE power control for SC-FDMA PUSCH — target SNR tracking. 3GPP TR 38.802 §4.1: 5G NR waveform evaluation including PAPR comparison for multiple waveform candidates.

## 8.3 MIMO-OFDM (Multiple Input Multiple Output)

MIMO-OFDM is the combination of Multiple Input Multiple Output antenna technology with OFDM modulation. MIMO exploits the spatial dimension of the radio channel: by transmitting from multiple antennas and receiving on multiple antennas, MIMO can simultaneously increase spectral efficiency (spatial multiplexing), improve reliability (diversity), or steer the beam toward the receiver (beamforming). OFDM is the ideal partner for MIMO because it converts the frequency-selective MIMO channel into a set of parallel flat-fading MIMO channels — one per subcarrier — each requiring only a small matrix operation for equalization.

### 8.3.1 MIMO Channel Model in OFDM

For an Nt × Nr MIMO-OFDM system (Nt transmit antennas, Nr receive antennas), the frequency-domain model per subcarrier extends the single-antenna case to a matrix equation:

y[k] = H[k] · x[k] + w[k] Where: y[k] ∈ ℂ^Nr×1 received vector on subcarrier k x[k] ∈ ℂ^Nt×1 transmitted vector on subcarrier k H[k] ∈ ℂ^Nr×Nt MIMO channel matrix on subcarrier k w[k] ∈ ℂ^Nr×1 AWGN noise vector H[k] = Σᵢ Hᵢ · e^{-j2πk·Δf·τᵢ} (DFT of the MIMO CIR tensors) MIMO Channel Capacity: C = Σₖ log₂ det(I + (SNR/Nt) · H[k] · H[k]^H) [bits/s/Hz]

Figure 8.4: 2×2 MIMO-OFDM system architecture. Each transmit antenna uses a complete OFDM chain; the MIMO channel matrix H[k] operates independently per subcarrier. Blue = direct paths (H₁₁, H₂₂); Red dashed = cross paths (H₁₂, H₂₁). Reference: 3GPP TS 38.211 §7.3.1.2

### 8.3.2 MIMO Modes in 5G NR

| MIMO Mode | Antennas (Tx×Rx) | Spatial Layers | Spectral Eff. Gain | Primary Use |
| --- | --- | --- | --- | --- |
| SISO (reference) | 1×1 | 1 | 1× (baseline) | Legacy/IoT |
| SIMO (Rx diversity) | 1×2 | 1 (diversity) | ~3 dB SNR gain | Coverage ↑ |
| Tx Diversity (SFBC) | 2×1 | 1 (diversity) | ~3 dB reliability | Robust link |
| Spatial Multiplex (SM) | 2×2 to 8×8 | 2–8 | 2–8× throughput | 5G NR PDSCH |
| Massive MIMO | 64×4 (typical) | Up to 8 DL | 3–5× cell capacity | 5G NR gNB |
| Full-Dimension MIMO | 128–256 Tx | Up to 16 DL | 5–10× capacity | 5G NR Advanced |

Table 8.3: MIMO modes supported in 5G NR (3GPP TS 38.211 Table 7.3.1.1.2-1)

### 8.3.3 Per-Subcarrier MIMO Equalization

One of OFDM's greatest advantages in MIMO systems is that the frequency-selective MIMO channel simplifies to an independent flat-fading MIMO channel per subcarrier. This allows well-understood MIMO detection algorithms to be applied subcarrier-by-subcarrier.

MIMO Zero-Forcing (ZF) Detector per subcarrier k: x̂[k] = (H[k]^H · H[k])^{-1} · H[k]^H · y[k] [pseudo-inverse] MIMO MMSE Detector per subcarrier k: x̂[k] = (H[k]^H · H[k] + (1/SNR)·I)^{-1} · H[k]^H · y[k] Complexity: O(N·L³) where L = min(Nt,Nr) layers e.g. 4×4 MIMO, 1024 subcarriers: 1024 × 4³ = 65,536 operations/symbol Compare to single-antenna: 1024 operations/symbol

📋 3GPP / Standards Reference 3GPP TS 38.211 §7.3.1.2: PDSCH spatial multiplexing — up to 8 code words, up to 4 layers per code word. 3GPP TS 38.214 §5.1.2: CSI feedback framework — RI (Rank Indicator), PMI (Precoding Matrix Indicator), CQI. 3GPP TS 38.214 §5.2.2: PDSCH HARQ and retransmission combining for MIMO spatial streams. 3GPP TS 38.211 §7.3.1.3: Precoding matrices for Nt=1,2,4 (3GPP codebook — Type I and Type II CSI). 3GPP TR 36.814: LTE-Advanced MIMO enhancements — first standardization of 8-layer DL MIMO.

## 8.4 Beamforming with OFDM

Beamforming is a signal processing technique that uses a phased array of antennas to steer transmitted and received energy in a specific spatial direction. When combined with OFDM, beamforming gains an additional dimension: since each subcarrier carries independent data, beamforming weights can be optimized independently per subcarrier to track frequency-dependent channel variations. This is known as frequency-domain beamforming or wideband beamforming, and it is the physical layer foundation of 5G NR massive MIMO and mmWave operation.

### 8.4.1 Analog, Digital, and Hybrid Beamforming

| Architecture | Phase Shifters | Flexibility | 5G NR Application |
| --- | --- | --- | --- |
| Analog BF | RF (analog, per-ant) | 1 beam, narrow BW | Sub-6 GHz initial beam mgmt |
| Digital BF | Baseband (digital) | Per-subcarrier weights | Massive MIMO (sub-6 GHz) |
| Hybrid BF | RF + Baseband | Multiple beams/streams | mmWave 5G NR (FR2) |

Table 8.4: Beamforming architectures in 5G NR — trade-offs between hardware cost and flexibility

### 8.4.2 Phased Array Steering and Per-Subcarrier Optimization

Digital beamforming applies a complex weight w[k,m] to each combination of subcarrier k and antenna m. The phase of w[k,m] steers the beam, while the magnitude adjusts the contribution of each antenna. In massive MIMO with N_ant antennas, the precoder matrix W[k] ∈ ℂ^{N_ant × L} jointly optimizes beamforming for L spatial layers.

Beamformed transmitted signal on subcarrier k: x_bf[k] = W[k] · s[k] Where: W[k] ∈ ℂ^{N_ant × L} = precoding (beamforming) matrix s[k] ∈ ℂ^L = L spatial layer symbols x_bf[k] ∈ ℂ^{N_ant} = N_ant transmitted symbols Optimal W (Singular Value Decomposition): H[k] = U · Σ · V^H → W_opt[k] = V[:, 1:L] (first L right singular vectors) Beam steering direction θ: w_m = e^{j2πmd·sin(θ)/λ} (m = antenna index)

Figure 8.5: Left — 8-element phased array with beam steered to 30°; the array factor pattern shows the main beam and sidelobes. Right — per-subcarrier beamforming weight magnitude and phase across the OFDM bandwidth, showing frequency-selective adaptation. Reference: 3GPP TS 38.214 §5.1.3

📋 3GPP / Standards Reference 3GPP TS 38.214 §5.1.3: Downlink precoding and beamforming — Type I and Type II codebooks for sub-6 GHz. 3GPP TS 38.211 §7.3.1.3: Precoding matrices (codebook subset restriction for 1,2,4 antenna ports). 3GPP TS 38.213 §7.1: Beam management procedures — P1/P2/P3 for initial beam acquisition and refinement. 3GPP TR 38.912: mmWave beamforming and beam management study for 5G NR FR2 (24–52.6 GHz). 3GPP TS 38.104 §9.3: BS transmitter requirements for beam correspondence (FR1 and FR2).

## 8.5 Pilot Subcarriers and Channel Estimation

Pilots (also called Reference Signals in 3GPP terminology) are predetermined, known symbols inserted at specific time-frequency positions in the OFDM resource grid. The receiver compares the received pilot with the known transmitted pilot to estimate the channel frequency response at that position. Channel estimates at pilot positions are then interpolated to all data subcarriers, enabling coherent demodulation across the entire bandwidth. Pilot design is a carefully engineered aspect of every 3GPP standard, balancing estimation accuracy, overhead, and compatibility with multiple MIMO layers.

### 8.5.1 3GPP Reference Signal Types

| Reference Signal | Purpose | Standard | Overhead | Mapping |
| --- | --- | --- | --- | --- |
| DMRS (Demod. RS) | PDSCH/PUSCH channel est. | TS 38.211 §7.4.1 | 1–4 symbols/slot | Front-loaded (symbol 2) |
| PTRS (Phase Tracking) | Phase noise correction | TS 38.211 §7.4.1.2 | Sparse (1/6 SCs) | mmWave specific |
| CSI-RS | Channel state info feedback | TS 38.211 §7.4.1.5 | Configurable | Periodic/aperiodic |
| SSB (Sync Signal) | Timing, beam management | TS 38.211 §7.4.3 | 4 symbols | 20 ms periodicity |
| SRS (Sounding RS) | UL channel sounding | TS 38.211 §6.4.1.4 | 1–4 symbols/slot | UL, configurable |
| CRS (LTE legacy) | LTE DL channel estimation | TS 36.211 §6.10.1 | ~15% overhead | All RBs, every slot |

Table 8.5: 3GPP Reference Signal types for channel estimation in NR and LTE

Figure 8.6: Left — pilot (DMRS) pattern in the OFDM resource grid; red stars mark pilot positions. Centre — LS channel estimate (red) vs true channel (blue) with linear interpolation (green). Right — channel magnitude heatmap across time and frequency, showing time-varying fading. Reference: 3GPP TS 38.211 §7.4.1

### 8.5.2 Zadoff-Chu Sequences for DMRS

3GPP 5G NR DMRS uses Gold sequences for data channels and Zadoff-Chu (ZC) sequences for reference signals like PUCCH and SRS. ZC sequences have the remarkable property of a constant amplitude zero autocorrelation (CAZAC) — their autocorrelation is an impulse and their DFT is also a ZC sequence. This makes them ideal for channel estimation: a received ZC sequence divided element-wise by the transmitted ZC sequence gives the channel estimate directly.

Zadoff-Chu sequence of length N_ZC and root u: x_u[n] = e^{-jπun(n+1)/N_ZC} for n = 0, 1, ..., N_ZC-1 Key properties: ¦x_u[n]¦ = 1 (constant envelope → low PAPR for UL reference signals) Cyclic autocorrelation: R_u[τ] = N_ZC · δ[τ] (ideal — no self-interference) DFT of ZC sequence is also a ZC sequence (dual property) Channel estimate from ZC pilot: Ĥ[k] = Y[k] / X_ZC[k] (element-wise division)

📋 3GPP / Standards Reference 3GPP TS 38.211 §5.2.2: Sequence generation using Gold sequences (length-31 m-sequences) for NR DMRS. 3GPP TS 38.211 §5.2.3: Zadoff-Chu base sequences for PUSCH/PUCCH DMRS and SRS. 3GPP TS 38.211 §7.4.1.1.2: PDSCH DMRS Type 1 and Type 2 — pilot density and CDM group definition. 3GPP TS 38.211 §7.4.1.1.3: Front-loaded DMRS — 1 or 2 additional DMRS symbols for high-mobility UE. 3GPP TS 38.214 §5.1.6: Higher-layer signalling for DMRS configuration (dmrs-Type, maxLength, scramblingID).

## 8.6 PAPR (Peak-to-Average Power Ratio) Reduction Techniques

PAPR is arguably the most significant practical disadvantage of OFDM. Since an OFDM signal is the sum of many independently modulated subcarriers, their amplitudes occasionally add constructively to create peaks far above the average power level. High PAPR forces power amplifiers to operate well below their saturation point (power back-off), drastically reducing efficiency and increasing hardware cost. For base stations transmitting at kilowatts, even a 1 dB improvement in PA efficiency translates to significant energy and operational cost savings.

### 8.6.1 PAPR Analysis

OFDM signal: s(t) = (1/√N) Σ_{k=0}^{N-1} X[k] · e^{j2πkΔft} PAPR = max_{0≤t<T} ¦s(t)¦² / E[¦s(t)¦²] Theoretical maximum PAPR (N subcarriers, all in phase): PAPR_max = N (i.e., 10·log₁₀(N) dB) N=64: PAPR_max = 18 dB ¦ N=1024: PAPR_max = 30 dB Practical PAPR (99.9th percentile, QPSK): N=64: ~10 dB ¦ N=256: ~11 dB ¦ N=1024: ~12 dB CCDF: P(PAPR > γ) — used to compare reduction techniques

### 8.6.2 PAPR Reduction Methods Comparison

| Technique | PAPR Reduction | Complexity | Distortion | 3GPP / Standards |
| --- | --- | --- | --- | --- |
| Clipping & Filtering | 3–6 dB | Very Low | Yes — in-band noise | LTE eNB (common) |
| SLM (Selected Mapping) | 3–4 dB | Medium | None (distortion-free) | Research/Wi-Fi |
| Tone Reservation (TR) | 3–5 dB | Medium | None (dedicated tones) | DVB-T2, LTE eNB |
| PTS (Partial Trans. Seq.) | 4–6 dB | High | None | Research |
| Active Constellation Ext. | 3–4 dB | Low-Medium | Marginal (BER impact) | Proprietary implementations |
| DFT-S-OFDM (SC-FDMA) | 4–6 dB vs OFDM | Low (1 DFT) | None | LTE/NR UL (TS 38.211) |

Table 8.6: PAPR reduction techniques — quantitative comparison of reduction, complexity, and distortion

Figure 8.7: PAPR reduction analysis. Left/Centre — time-domain instantaneous power with and without clipping. Right — CCDF curves showing probability of PAPR exceeding threshold γ. Clipping at 0.5 reduces the 99.9% PAPR from ~11 dB to ~7 dB at the cost of in-band noise (EVM degradation)

### 8.6.3 Clipping & Filtering — Detailed Analysis

Clipping is the simplest and most widely deployed PAPR reduction technique. The transmitted signal is clipped at a predefined amplitude threshold A_clip. Post-clipping filtering (using a pulse-shaping filter) suppresses the out-of-band spectral re-growth caused by clipping, but re-introduces some in-band noise.

Clipping operation: s_clip(t) = s(t) if ¦s(t)¦ ≤ A_clip A_clip · e^{j∠s(t)} if ¦s(t)¦ > A_clip Clipping Ratio (CR): CR = A_clip / σ_s (σ_s = RMS amplitude) CR = 1.0 dB: ~6 dB PAPR reduction, ~2 dB EVM degradation CR = 1.4 dB: ~4 dB PAPR reduction, ~0.5 dB EVM degradation CR = 2.0 dB: ~2 dB PAPR reduction, negligible EVM EVM (Error Vector Magnitude): 3GPP limit for BS = 3.5% (64-QAM), 8% (16-QAM)

⚠️ EVM and 3GPP Compliance 3GPP TS 38.104 §6.5.2 defines mandatory EVM requirements for gNB (base station) transmitters: • BPSK / QPSK: EVM ≤ 17.5% (allows aggressive clipping) • 16-QAM: EVM ≤ 12.5% (moderate clipping acceptable) • 64-QAM: EVM ≤ 8% (clipping must be conservative) • 256-QAM: EVM ≤ 3.5% (clipping severely constrained) Clipping must be designed so the resulting EVM stays within these mandatory limits.

📋 3GPP / Standards Reference 3GPP TS 38.104 §6.5.2: Error Vector Magnitude (EVM) requirements for NR BS transmitters. 3GPP TR 38.803: Study on new radio access technology — PAPR evaluation for NR waveform candidates. 3GPP TS 38.101-1 §6.5.2: UE EVM requirements (more relaxed than BS — allows higher PAPR reduction). ETSI EN 302 755: DVB-T2 standard — Tone Reservation PAPR technique formally standardized. IEEE 802.11ax §27.3.11: PAPR management in Wi-Fi 6 OFDMA — uplink trigger frame design.

## 8.7 C++ Implementation: Advanced OFDM Concepts

The following production-grade C++ code implements OFDMA resource block allocation, SC-FDMA DFT spreading, and PAPR clipping reduction. These modules form the core of an advanced OFDM transmitter compliant with 3GPP numerology.

// AdvancedOFDM.hpp — OFDMA + SC-FDMA + PAPR Reduction #pragma once #include <complex> #include <vector> #include <algorithm> #include <cmath> #include <stdexcept> #include <numeric> using Complex = std::complex<double>; using CplxVec = std::vector<Complex>; using RealVec = std::vector<double>; // ─── 3GPP NR Numerology (3GPP TS 38.211 Table 4.2-1) ───────────────── struct NRNumerology { int mu; // Numerology index (0–4) double scs_khz; // Subcarrier spacing in kHz int N_rb; // Number of RBs for 100 MHz BW double symbol_dur_us; // OFDM symbol duration in µs double cp_dur_us; // Normal CP duration in µs }; const NRNumerology NR_MU1 = {1, 30.0, 66, 33.33, 2.34}; // Sub-6GHz typical const NRNumerology NR_MU3 = {3, 120.0, 66, 8.33, 0.59}; // mmWave // ─── OFDMA Resource Block Allocator ────────────────────────────────── // Maps L OFDM layers' data onto N subcarriers (OFDMA multi-user) struct OFDMAAllocation { int user_id; int rb_start; // First Resource Block index int rb_count; // Number of allocated RBs (1 RB = 12 subcarriers) int mcs_index; // Modulation and Coding Scheme (0–28) }; CplxVec buildOFDMASignal(int N_fft, const std::vector<OFDMAAllocation>& allocs, const std::vector<CplxVec>& user_symbols) { CplxVec freq_domain(N_fft, {0.0, 0.0}); // Map each user's symbols onto their allocated subcarriers for (size_t u = 0; u < allocs.size(); ++u) { const auto& a = allocs[u]; int sc_start = a.rb_start * 12; // 12 subcarriers per RB int sc_count = a.rb_count * 12; for (int k = 0; k < sc_count && k < (int)user_symbols[u].size(); ++k) { freq_domain[sc_start + k] = user_symbols[u][k]; } } return freq_domain; // Apply N_fft IFFT to get time-domain signal }

// SC-FDMA DFT-Spread OFDM Transmitter (3GPP TS 38.211 §5.3.2) // Apply M-point DFT spreading before subcarrier mapping // This converts OFDMA to SC-FDMA by spreading each symbol across all M subcarriers CplxVec dftSpread(const CplxVec& input_symbols, int M) { if ((int)input_symbols.size() != M) throw std::invalid_argument("Input size must equal M"); CplxVec output(M); // Compute M-point DFT manually (for clarity; use FFTW in production) for (int k = 0; k < M; ++k) { output[k] = {0.0, 0.0}; for (int n = 0; n < M; ++n) { double angle = -2.0 * M_PI * k * n / M; output[k] += input_symbols[n] * Complex(std::cos(angle), std::sin(angle)); } output[k] /= std::sqrt(static_cast<double>(M)); // normalize } return output; } // SC-FDMA complete uplink transmitter CplxVec scfdmaTx(const CplxVec& qam_symbols, // QAM modulated data int M, // allocated subcarriers int N, // FFT size int sc_offset, // first allocated SC int L_cp) { // CP length // Step 1: M-point DFT spreading (the key SC-FDMA step) CplxVec dft_out = dftSpread(qam_symbols, M); // Step 2: Subcarrier mapping (M active SCs out of N) CplxVec freq_domain(N, {0.0, 0.0}); for (int k = 0; k < M; ++k) freq_domain[sc_offset + k] = dft_out[k]; // Step 3: N-point IFFT (same as OFDMA from here) // [Apply IFFT — identical to OFDMA, e.g. using FFTW] CplxVec time_signal = ifft(freq_domain, N); // → low-PAPR SC-like signal // Step 4: Add cyclic prefix CplxVec output(N + L_cp); for (int i = 0; i < L_cp; ++i) output[i] = time_signal[N - L_cp + i]; for (int i = 0; i < N; ++i) output[L_cp + i] = time_signal[i]; return output; }

// PAPR Clipping Reduction (3GPP TS 38.104 §6.5.2 compliant) // Compute PAPR of a signal double computePAPR_dB(const CplxVec& signal) { double peak_power = 0.0, avg_power = 0.0; for (const auto& s : signal) { double p = std::norm(s); // ¦s¦² peak_power = std::max(peak_power, p); avg_power += p; } avg_power /= signal.size(); return 10.0 * std::log10(peak_power / avg_power); } // Apply soft clipping with threshold A_clip // CR (Clipping Ratio) = A_clip / rms_amplitude CplxVec clipSignal(CplxVec signal, double clip_ratio) { // Compute RMS amplitude double rms = 0.0; for (const auto& s : signal) rms += std::norm(s); rms = std::sqrt(rms / signal.size()); double A_clip = clip_ratio * rms; int n_clipped = 0; for (auto& s : signal) { double amp = std::abs(s); if (amp > A_clip) { s = A_clip * (s / amp); // Clip to A_clip, preserve phase ++n_clipped; } } // [In production: apply band-pass filter here to reduce OOB spectral re-growth] return signal; } // Adaptive clipping: find optimal CR satisfying EVM constraint // EVM limit from 3GPP TS 38.104 §6.5.2 per MCS CplxVec adaptiveClip(const CplxVec& signal, double max_evm_pct, double target_papr_db) { double cr = 2.0; // Start conservative while (cr > 0.8) { CplxVec clipped = clipSignal(signal, cr); double papr = computePAPR_dB(clipped); // Estimate EVM (simplified): EVM ∝ clipping noise power double evm_est = (cr < 1.2) ? (2.0 / cr) : 1.0; if (papr <= target_papr_db && evm_est <= max_evm_pct) return clipped; cr -= 0.1; } return signal; // fallback: no clipping } // Usage example: // CplxVec tx = scfdmaTx(qam_data, 48, 512, 120, 36); // printf("Before clipping: PAPR = %.1f dB\n", computePAPR_dB(tx)); // CplxVec tx_clip = adaptiveClip(tx, 8.0 /*EVM%*/, 9.0 /*target PAPR dB*/); // printf("After clipping: PAPR = %.1f dB\n", computePAPR_dB(tx_clip));

✅ Implementation Notes — 3GPP Compliance • NRNumerology struct aligns with TS 38.211 Table 4.2-1 (μ=0,1,2,3,4 defined) • OFDMA RB structure: exactly 12 subcarriers per RB per TS 38.211 §4.4.4.1 • SC-FDMA: transform precoding flag in TS 38.211 §5.3.2 activates DFT spreading • EVM thresholds in adaptiveClip() must match TS 38.104 Table 6.5.2.2-1 per MCS index • Production PAPR reduction also needs out-of-band emission check per TS 38.104 §6.6.3 • For FFTW integration: replace manual DFT loops with fftw_execute() for O(N log N) performance

## 8.8 BER Performance Analysis: Advanced OFDM Variants

Understanding BER performance across the advanced OFDM variants allows engineers to select the right scheme for a given deployment scenario. The BER curves are governed by the modulation order, channel model, and the specific advantages of each multiple-access scheme.

Figure 8.8: BER performance comparison. Left — OFDM with BPSK/QPSK/16-QAM/64-QAM showing the SNR penalty for each additional bit/symbol. Right — OFDMA vs SC-FDMA vs 2×2 MIMO-OFDM: SC-FDMA gains ~2.5 dB from lower PAPR; MIMO-OFDM gains ~5 dB from spatial diversity. Reference: 3GPP TS 38.214 — link adaptation and MCS selection

| Scheme | BER = 10⁻³ SNR | Modulation | Channel | Note |
| --- | --- | --- | --- | --- |
| BPSK (OFDM) | ~7 dB | 1 bit/symbol | AWGN | Most robust |
| QPSK (OFDM) | ~10 dB | 2 bits/symbol | AWGN | LTE PDCCH MCS |
| 16-QAM (OFDM) | ~17 dB | 4 bits/symbol | AWGN | Common LTE/NR |
| 64-QAM (OFDM) | ~24 dB | 6 bits/symbol | AWGN | High-SNR links |
| SC-FDMA QPSK | ~7.5 dB | 2 bits/symbol | Rayleigh | PAPR gain: ~2.5 dB |
| 2×2 MIMO-OFDM QPSK | ~5 dB | 2 bits/symbol | Rayleigh | Diversity gain: ~5 dB |

Table 8.7: Required SNR for BER = 10⁻³ across advanced OFDM schemes (theoretical bounds)

## 8.9 Summary and Key Takeaways

Section 8 has covered the six advanced OFDM concepts that transform the basic modulation scheme into the multi-user, multi-antenna, power-efficient framework of modern wireless standards:

- OFDMA (§8.1): Extends OFDM to multiple simultaneous users by dividing the resource grid into RBs. Frequency-selective scheduling provides multi-user diversity gain. Standardized in LTE DL (TS 36.211), 5G NR DL (TS 38.211), and Wi-Fi 6 (802.11ax).

- SC-FDMA (§8.2): DFT-spread OFDM that achieves 2–5 dB PAPR reduction by pre-spreading data with a DFT, creating a near-single-carrier time-domain signal. Used exclusively for uplink in LTE and 5G NR (TS 38.211 §5.3.2).

- MIMO-OFDM (§8.3): Combining MIMO with OFDM reduces the wideband spatial channel to per-subcarrier MIMO matrices, enabling efficient spatial multiplexing (up to 8 layers) and diversity. Foundational to 5G NR massive MIMO (up to 256 antennas).

- Beamforming (§8.4): Phased array steering and per-subcarrier digital beamforming via precoding matrices. SVD-based optimal precoder. Hybrid analog-digital architectures enable mmWave 5G NR FR2 deployment.

- Pilot/DMRS Estimation (§8.5): Reference signals (DMRS, CSI-RS, SRS) enable coherent demodulation. ZC sequences provide CAZAC property ideal for channel estimation. 3GPP defines detailed pilot patterns, densities, and generation rules.

- PAPR Reduction (§8.6): Clipping, SLM, tone reservation, and PTS address the fundamental OFDM PAPR problem. Clipping dominates in practice — EVM limits from TS 38.104 constrain how aggressively it can be applied.

➡️ Up Next: Section 9 — OFDM Advantages, Disadvantages, and Comparison Section 9 synthesizes all concepts covered in Sections 1–8 to provide: • Complete advantages summary: spectral efficiency, robustness, MIMO compatibility, equalization simplicity • Comprehensive disadvantages: PAPR, frequency offset sensitivity, CP overhead, PA linearity requirements • Quantitative comparison with CDMA, single-carrier, and FBMC modulation schemes • Deployment trade-off analysis for 5G NR, Wi-Fi 6, satellite communications, and IoT

## 8.10 Key Formulas Reference — Section 8

| Formula | Expression | Reference |
| --- | --- | --- |
| OFDMA capacity | C = Σₖ B·log₂(1+SNRₖ) | Shannon bound |
| MIMO capacity | C = Σₖ log₂ det(I + SNR/Nt·H·Hᴴ) | Foschini-Gans (1998) |
| Beam steering phase | w_m = e^{j2πmd·sin(θ)/λ} | Phased array theory |
| ZF MIMO detector | x̂ = (HᴴH)⁻¹·Hᴴ·y | Standard MIMO ref. |
| MMSE MIMO detector | x̂ = (HᴴH + I/SNR)⁻¹Hᴴy | 3GPP TS 38.211 |
| PAPR (peak power) | PAPR = max¦s¦² / E[¦s¦²] | IEEE 802.11ax |
| Zadoff-Chu sequence | x_u[n] = e^{-jπun(n+1)/N} | TS 38.211 §5.2.3 |
| DFT-S-OFDM spreading | D̃[k] = Σ d[n]·e^{-j2πkn/M} | TS 38.211 §5.3.2 |
| SVD precoder | H[k] = U·Σ·Vᴴ → W = V[:,1:L] | TS 38.214 §5.1.3 |

Table 8.8: Section 8 formula summary with standards references

## 8.11 3GPP Standards Summary for Section 8

| Standard | Section / Clause | Relevance to Section 8 |
| --- | --- | --- |
| 3GPP TS 38.211 Rel-15+ | §7.3.1 — PDSCH resource mapping | OFDMA resource grid, RB structure, pilot patterns |
| 3GPP TS 38.211 Rel-15+ | §5.3.2 — DFT-S-OFDM (SC-FDMA) | Transform precoding for uplink SC-FDMA waveform |
| 3GPP TS 38.211 Rel-15+ | §7.3.1.2–7.3.1.3 — MIMO precoding | Spatial multiplexing, precoding matrices, layers |
| 3GPP TS 38.214 Rel-15+ | §5.1.2–5.1.3 — PDSCH procedure | MCS selection, beamforming precoder, CSI feedback |
| 3GPP TS 38.211 Rel-15+ | §7.4.1.1 — DMRS for PDSCH | Pilot sequence design, Type 1/2, front-loaded DMRS |
| 3GPP TS 38.104 Rel-15+ | §6.5.2 — EVM requirements | PAPR clipping constraint — EVM limits per MCS |
| 3GPP TS 38.321 Rel-15+ | §6.1 — MAC scheduling | OFDMA dynamic scheduling, RB allocation per slot |
| IEEE 802.11ax-2021 | §26.5, §27.3 — OFDMA + PAPR | Wi-Fi 6 OFDMA resource unit (RU) and PAPR management |

# 9. OFDM Advantages and Disadvantages

OFDM has revolutionized wireless communications and become the foundation of modern standards including 4G LTE, 5G NR, Wi-Fi 6/6E/7, and digital broadcasting. However, like any technology, OFDM presents both significant advantages and notable challenges. This section provides a comprehensive technical analysis of OFDM's strengths and limitations, supported by 3GPP specifications, IEEE standards, and performance metrics from deployed systems.

Understanding these trade-offs is critical for system design decisions, parameter optimization, and selecting appropriate enhancement techniques. The discussion covers theoretical foundations, practical implementations, and real-world performance data from commercial deployments.

## 9.1 Advantages of OFDM

OFDM offers multiple compelling advantages that have driven its widespread adoption across diverse wireless communication systems. These benefits stem from both the fundamental mathematical properties of orthogonal subcarriers and efficient DSP implementation techniques.

### 9.1.1 Superior Multipath Resistance

OFDM's primary strength lies in its exceptional robustness against frequency-selective fading caused by multipath propagation. In wireless channels, transmitted signals reflect off buildings, terrain, and obstacles, creating multiple propagation paths with different delays. This phenomenon severely degrades single-carrier systems but OFDM handles it elegantly.

- By dividing the wideband channel into narrow subcarriers, each subcarrier experiences flat fading

- The cyclic prefix absorbs multipath delays up to 4.7 μs (5G NR, 30 kHz SCS)

- Enables simple one-tap frequency-domain equalization instead of complex time-domain equalizers

- 3GPP TS 38.211 specifies cyclic prefix lengths optimized for typical urban delay spreads (1-5 μs)

Spectral Efficiency: OFDM achieves 90-95% spectral efficiency compared to 60-70% for traditional FDM by eliminating guard bands through orthogonality.

### 9.1.2 High Spectral Efficiency

OFDM achieves exceptional spectral efficiency through overlapping orthogonal subcarriers. For 100 MHz 5G NR bandwidth with 30 kHz subcarrier spacing, OFDM uses 3272 subcarriers while equivalent FDM would waste ~40% more spectrum in guard bands.

### 9.1.3 Simplified Equalization

OFDM transforms complex time-domain equalization into simple frequency-domain multiplication. For N=4096 subcarriers with delay spread L=100 taps: OFDM equalization is O(N) complexity while single-carrier requires O(L²) ≈ 250× more complex.

### 9.1.4 Flexible Resource Allocation

- 3GPP Resource Block (RB): 12 subcarriers × 14 OFDM symbols

- 100 MHz bandwidth: 273 RBs = 3,276 subcarriers allocable to different users

- Dynamic scheduling every 0.5 ms enables frequency-selective multiuser diversity

- Typical throughput gain: 20-40% over round-robin scheduling

### 9.1.5 MIMO Compatibility

- Per-subcarrier MIMO processing enables up to 8 spatial layers in 5G NR

- Massive MIMO: 256 antenna ports with per-tone CSI for beamforming

- Peak throughput: 20 Gbps (FR2, 800 MHz, 8 layers, 256-QAM)

- Without OFDM, MIMO over frequency-selective channels would require prohibitive complexity

### 9.1.6 Efficient DSP Implementation

FFT Algorithm: Reduces complexity from O(N²) to O(N log N)

- DFT for N=4096: 16,777,216 complex multiplications

- FFT (Radix-2): 49,152 complex multiplications → 340× speedup

- Modern chipsets: 8192-point FFT in <10 μs enables sub-ms latency

## 9.2 Disadvantages of OFDM

Despite numerous advantages, OFDM presents significant challenges that must be addressed in practical system design.

### 9.2.1 High Peak-to-Average Power Ratio (PAPR)

PAPR is OFDM's most serious limitation:

- Single Carrier QPSK: PAPR ≈ 3-4 dB

- OFDM (64 subcarriers): PAPR ≈ 10-12 dB

- OFDM (2048 subcarriers, 64-QAM): PAPR ≈ 13-14 dB

Consequences:

- Power Amplifier must operate 8-10 dB below saturation → efficiency drops from 40% to 15-20%

- Battery life reduced by 30-50% in mobile devices

- Spectral regrowth causes out-of-band emissions violating 3GPP masks (TS 38.101)

- Coverage reduction: 20-30% smaller cell radius due to lower effective transmission power

This is why 3GPP uses SC-FDMA (DFT-Spread OFDM) for LTE/5G uplink—achieves PAPR ≈ 5-6 dB, dramatically improving mobile battery life.

### 9.2.2 Frequency Offset Sensitivity

OFDM requires extremely tight frequency synchronization. Even small offsets destroy orthogonality:

- Crystal oscillator mismatch: ±10-20 ppm → 200-400 Hz error at 2 GHz

- Doppler shift: 120 km/h at 28 GHz → ±3.1 kHz

- For Δf=15 kHz, ε=0.15Δf causes 1 dB SNR loss; ε=0.5Δf causes 5 dB loss

- 3GPP requirements: ±0.05 ppm (base stations), ±0.1 ppm (UE) → expensive TCXO needed

### 9.2.3 Timing Synchronization Requirements

- Small timing error τ << TCP: Correctable phase rotation per subcarrier

- Large timing error τ > TCP: Catastrophic ISI and ICI

- 5G NR solution: PSS/SSS for coarse/fine timing + TRS for tracking (<100 ns accuracy)

- Synchronization overhead: ~10-15% of 5G NR signal → reduced throughput

### 9.2.4 Cyclic Prefix Overhead

5G NR (30 kHz SCS):

- Normal CP: 2.34 μs (first symbol), 2.08 μs (others) → 6.25% overhead

- Extended CP: 4.17 μs → 11.1% overhead

- 100 MHz bandwidth: Normal CP wastes ~6.25 MHz → ~240 Mbps throughput loss at 64-QAM

- Trade-off: Too short causes ISI; too long wastes spectrum

### 9.2.5 Phase Noise Sensitivity

- Low-frequency phase noise: Common Phase Error (CPE) → correctable with PT-RS pilots

- High-frequency phase noise: ICI → acts like additive noise, not correctable

- mmWave impact: At 28 GHz, phase noise power is 23 dB higher than 2 GHz → 2-4 dB SNR loss

- 5G NR mitigation: PT-RS every 2-4 symbols, 120/240 kHz SCS, requires phase noise < -110 dBc/Hz at 1 MHz offset

### 9.2.6 Doppler Spread Sensitivity

High-speed train (350 km/h = 97.2 m/s):

- At 2 GHz: fD = 648 Hz → fD/Δf = 0.043 (minor ICI for 15 kHz SCS)

- At 28 GHz: fD = 9,072 Hz → fD/Δf = 0.605 (severe ICI for 15 kHz SCS)

- When fD/Δf > 0.1: SNR degradation of 3-5 dB

- 5G NR adaptive numerology: 15/30 kHz for <6 GHz, 120/240 kHz for mmWave

## 9.3 Comparison with Other Modulation Schemes

Comprehensive comparison highlights why OFDM dominates modern wireless despite challenges:

| Feature | OFDM | SC-FDMA | CDMA |
| --- | --- | --- | --- |
| PAPR | High (10-14 dB) | Low (5-6 dB) | Low (3-5 dB) |
| Spectral Efficiency | High (90-95%) | High (85-90%) | Medium (70-80%) |
| Equalization | Simple (1-tap) | Simple (1-tap) | Complex (RAKE) |
| Standards | LTE DL, 5G DL, Wi-Fi 6 | LTE UL, 5G UL | 3G (Legacy) |

Table 9.1: Comprehensive Comparison of Modern Wireless Modulation Schemes

Summary: OFDM dominates modern wireless (4G, 5G, Wi-Fi 6/7) due to superior multipath resistance, high spectral efficiency, and MIMO compatibility. However, high PAPR remains the primary challenge, addressed in uplink through SC-FDMA. Future 6G systems will likely evolve OFDM with PAPR reduction techniques while maintaining its core advantages.

10. MATLAB / C++ Implementation

This section presents the complete team-based C++ implementation of a 5G NR Physical Layer OFDM Modulation Simulator. The codebase is structured following 3GPP TS 38.211 specifications and divided across five developer modules, each responsible for a distinct subsystem. The combined simulator validates every stage of the OFDM transmission chain — from numerology configuration through cyclic-prefix assembly — and exposes an interactive console menu for per-module and full-chain testing.

| Category | Details |
| --- | --- |
| Language | C++17 |
| Standard Libraries | <iostream>, <vector>, <complex>, <cmath>, <random>, <algorithm>, <memory> |
| 3GPP Reference | TS 38.211 (5G NR Physical Channels and Modulation) |
| Team Size | 5 Developers (DEV 1 – DEV 5) |
| Compiler Tested | g++ 11+ / MSVC 2019+ |
| Build Command | g++ -std=c++17 -O2 -o ofdm_sim ofdm_simulator.cpp |

10.1 Complete OFDM Transmitter Code (C++)

The transmitter pipeline consists of five tightly coupled C++ classes, each authored by a dedicated developer. The pipeline follows the standard OFDM processing chain defined in 3GPP TS 38.211 Section 7:

| NumerologyManager (DEV 1) | → | ModulationEngine (DEV 2) | → | ResourceGrid Manager (DEV 3) | → | FFTProcessor (DEV 4) | → | CyclicPrefix Processor (DEV 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| DEV 1 | Numerology & Configuration Manager |
| --- | --- |

The NumerologyManager class is the system entry-point. It reads the 5G NR numerology index μ and derives all dependent parameters — subcarrier spacing, FFT size, CP lengths, and slot duration — exactly as specified in 3GPP TS 38.211 Table 4.2-1.

Class Responsibility Configure and expose the NRConfig structure containing all system-level numerology parameters used by every downstream module.

Key Design Decisions:

- enum class NumerologyIndex — type-safe μ values (0–4) prevent invalid numerology assignments.

- enum class CyclicPrefixType — distinguishes Normal CP (14 symbols/slot) from Extended CP (12 symbols/slot).

- calculateCPLengths() — computes per-symbol CP sample counts using the 9/128 ratio mandated by TS 38.211.

- map<int,int> mu_to_scs — lookup table mapping μ → SCS in kHz (15, 30, 60, 120, 240).

Class Declaration & NRConfig Structure:

class NumerologyManager { public: enum class NumerologyIndex { MU_0=0, MU_1=1, MU_2=2, MU_3=3, MU_4=4 }; enum class CyclicPrefixType { NORMAL, EXTENDED }; struct NRConfig { NumerologyIndex mu; int subcarrier_spacing_khz; // 15/30/60/120/240 kHz int fft_size; // 2048 (common for 20 MHz BW) int num_subcarriers; // 1200 subcarriers = 20 MHz int num_symbols_per_slot; // 14 (Normal CP) or 12 (Ext CP) int num_slots_per_subframe; // 2^mu double slot_duration_ms; // 1.0 / num_slots_per_subframe CyclicPrefixType cp_type; vector<int> cp_lengths; // CP length per symbol (samples) }; private: NRConfig config; map<int,int> mu_to_scs = {{0,15},{1,30},{2,60},{3,120},{4,240}}; void calculateCPLengths() { ... } // See CP section below public: NumerologyManager(NumerologyIndex mu = NumerologyIndex::MU_1, CyclicPrefixType cp = CyclicPrefixType::NORMAL); const NRConfig& getConfig() const; void displayConfiguration() const; };

CP Length Calculation Logic:

void calculateCPLengths() { // 3GPP TS 38.211 – Normal CP: first symbol gets extended CP if (config.cp_type == CyclicPrefixType::NORMAL) { config.cp_lengths.clear(); int first_cp = config.fft_size * 9 / 128; // Extended first symbol int normal_cp = config.fft_size * 9 / 128; // Subsequent symbols config.cp_lengths.push_back(first_cp); for (int i = 1; i < config.num_symbols_per_slot; i++) config.cp_lengths.push_back(normal_cp); } else { // Extended CP – single length for all symbols int ext_cp = config.fft_size / 4; for (int i = 0; i < config.num_symbols_per_slot; i++) config.cp_lengths.push_back(ext_cp); } }

5G NR Numerology Reference Table:

| μ | SCS (kHz) | Slot Duration (ms) | Slots/Subframe | FFT Size | Typical Use |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 | 1.0 | 1 | 2048 | LTE / FeMBB |
| 1 | 30 | 0.5 | 2 | 2048 | 5G Sub-6 GHz |
| 2 | 60 | 0.25 | 4 | 1024 | 5G mmWave |
| 3 | 120 | 0.125 | 8 | 512 | FR2 mmWave |
| 4 | 240 | 0.0625 | 16 | 256 | Reference Signals |

| DEV 2 | Digital Modulation Engine |
| --- | --- |

The ModulationEngine class generates normalized constellation mappings for QPSK, 16-QAM, 64-QAM, and 256-QAM. The modulate() method accepts a bit stream and returns a vector of complex IQ symbols ready for resource-grid mapping.

Design Principle All constellations are normalized to unit average power, ensuring that SNR comparisons across modulation orders are fair and consistent with 3GPP link-budget calculations.

Constellation Generation Methods:

class ModulationEngine { public: enum class ModulationType { QPSK, QAM16, QAM64, QAM256 }; private: ModulationType mod_type; int bits_per_symbol; vector<Complex> constellation; void generateQPSKConstellation() { double norm = 1.0 / sqrt(2.0); constellation = { Complex( norm, norm), // bit pattern 00 Complex( norm, -norm), // bit pattern 01 Complex(-norm, norm), // bit pattern 10 Complex(-norm, -norm) // bit pattern 11 }; } void generateQAM16Constellation() { double norm = 1.0 / sqrt(10.0); for (int i = 0; i < 16; i++) { int re_bits = (i >> 2) & 0x03; int im_bits = i & 0x03; double re = (2*(re_bits>>1)-1) * (2*(re_bits&1)+1); double im = (2*(im_bits>>1)-1) * (2*(im_bits&1)+1); constellation.push_back(Complex(re*norm, im*norm)); } } // generateQAM64Constellation() – norm = 1/sqrt(42), range ±1..±7 // generateQAM256Constellation() – norm = 1/sqrt(170), range ±1..±15 public: vector<Complex> modulate(const vector<int>& bits); void displayConstellation() const; int getBitsPerSymbol() const; };

Modulation Mapping Summary:

| Modulation | Bits/Symbol | Normalization Factor |
| --- | --- | --- |
| QPSK | 2 | 1 / √2 ≈ 0.7071 |
| 16-QAM | 4 | 1 / √10 ≈ 0.3162 |
| 64-QAM | 6 | 1 / √42 ≈ 0.1543 |
| 256-QAM | 8 | 1 / √170 ≈ 0.0767 |

| DEV 3 | Resource Grid & Mapping |
| --- | --- |

The ResourceGridManager allocates and populates the 2-D resource grid (OFDM symbols × subcarriers). The grid is initialized to zero-valued complex elements; mapSymbolsToGrid() fills cells sequentially from a provided symbol vector and records each allocation as a ResourceBlock.

class ResourceGridManager { private: int num_subcarriers; int num_ofdm_symbols; vector<vector<Complex>> resource_grid; // [symbol][subcarrier] struct ResourceBlock { int start_subcarrier, num_subcarriers; int start_symbol, num_symbols; }; vector<ResourceBlock> allocated_blocks; public: ResourceGridManager(int subcarriers, int symbols); // Map modulated symbols starting at (start_sc, start_sym) void mapSymbolsToGrid(const vector<Complex>& symbols, int start_sc = 0, int start_sym = 0); // Return one row of the grid (frequency-domain OFDM symbol) vector<Complex> getOFDMSymbol(int symbol_index) const; void displayResourceGrid() const; // Prints utilization + 8×8 preview };

Resource Grid Dimensions (20 MHz, μ=1):

| Parameter | Value |
| --- | --- |
| Number of Subcarriers | 1200 (100 Resource Blocks × 12 SC/RB) |
| OFDM Symbols per Slot | 14 (Normal CP) |
| Total Resource Elements | 16,800 per slot |
| Grid Data Type | vector<vector<complex<double>>> |
| Zero Padding | FFT size – active SCs = 848 guard subcarriers |

| DEV 4 | IFFT / FFT Processing & Signal Generation |
| --- | --- |

The FFTProcessor implements the Cooley-Tukey radix-2 DIT algorithm in C++ without external libraries, making the codebase fully self-contained. The constructor automatically rounds up any supplied size to the nearest power of two. The ifft() method performs the inverse transform and applies the 1/N normalization required to preserve signal energy.

class FFTProcessor { private: int fft_size; const double PI = 3.14159265358979323846; // Cooley-Tukey radix-2 DIT (in-place, recursive) void fft_recursive(vector<Complex>& x, bool inverse) { int N = x.size(); if (N <= 1) return; vector<Complex> even(N/2), odd(N/2); for (int i = 0; i < N/2; i++) { even[i] = x[2*i]; odd[i] = x[2*i + 1]; } fft_recursive(even, inverse); fft_recursive(odd, inverse); double angle = 2.0*PI/N * (inverse ? 1 : -1); Complex w(1), wn(cos(angle), sin(angle)); for (int k = 0; k < N/2; k++) { Complex t = w * odd[k]; x[k] = even[k] + t; x[k + N/2] = even[k] - t; w *= wn; } } public: FFTProcessor(int size); // rounds up to nearest power-of-2 vector<Complex> ifft(const vector<Complex>& freq_domain); // Pads to fft_size, runs inverse transform, normalises by 1/N vector<Complex> fft(const vector<Complex>& time_domain); // Pads to fft_size, runs forward transform (no normalisation) };

IFFT Normalisation After the inverse butterfly pass, every output sample is divided by fft_size. This ensures that the average power of the time-domain signal equals the average power of the frequency-domain input — a requirement for correct PAPR measurement.

FFT Size vs. Bandwidth (3GPP TS 38.211):

| Channel Bandwidth | FFT Size (μ=1, 30 kHz SCS) |
| --- | --- |
| 5 MHz | 512 |
| 10 MHz | 1024 |
| 20 MHz | 2048 ← simulator default |
| 50 MHz | 4096 |
| 100 MHz | 8192 |

| DEV 5 | Cyclic Prefix & OFDM Symbol Assembly |
| --- | --- |

The CyclicPrefixProcessor prepends a cyclic copy of each OFDM symbol's tail to its beginning, converting bare IFFT output into fully formed OFDM symbols. assembleSlot() concatenates all 14 CP-extended symbols into the final transmitted slot waveform.

class CyclicPrefixProcessor { private: vector<int> cp_lengths; // CP length per symbol (from NumerologyManager) public: CyclicPrefixProcessor(const vector<int>& cp_lens); // Prepend CP to a single OFDM symbol vector<Complex> addCyclicPrefix(const vector<Complex>& ofdm_symbol, int symbol_index) { int cp_len = cp_lengths[symbol_index]; vector<Complex> out; // 1. Copy last cp_len samples (tail) to front int start = ofdm_symbol.size() - cp_len; for (int i = start; i < ofdm_symbol.size(); i++) out.push_back(ofdm_symbol[i]); // 2. Append full symbol for (const auto& s : ofdm_symbol) out.push_back(s); return out; // size = fft_size + cp_len } // Concatenate all symbols into one slot-length waveform vector<Complex> assembleSlot( const vector<vector<Complex>>& ofdm_symbols); void displayCPProcessing(int num_symbols) const; };

CP Length Values for μ=1 (30 kHz SCS, FFT=2048):

| Symbol Index | CP Length (samples) |
| --- | --- |
| 0 (first symbol) | 144 (9/128 × 2048 = 144) |
| 1 – 13 | 144 (identical for normal CP) |
| Total CP / slot | 2016 samples |
| Useful data / slot | 28,672 samples (14 × 2048) |
| CP Overhead | ≈ 6.56 % |

10.2 Complete OFDM Receiver Code (C++)

The receiver reverses each transmitter step. In this simulator the receive path is embedded inside the OFDMSimulator orchestrator and uses FFTProcessor.fft() together with simple one-tap frequency-domain equalization. The following code excerpt illustrates the receive-chain logic executed after the channel model.

// ── RECEIVE CHAIN (excerpt from OFDMSimulator) ────────────────────────── // STEP R-1: Remove cyclic prefix vector<Complex> removeCyclicPrefix(const vector<Complex>& rx_symbol, int cp_len) { // Discard first cp_len samples return vector<Complex>(rx_symbol.begin() + cp_len, rx_symbol.end()); } // STEP R-2: FFT – convert time domain back to frequency domain vector<Complex> rx_freq = fft_proc->fft(rx_time_symbol); // STEP R-3: One-tap channel equalization // H[k] = channel frequency response at subcarrier k // Y[k] = received frequency-domain sample // X̂[k] = Y[k] / H[k] (ideal: no noise) for (int k = 0; k < num_subcarriers; k++) { rx_freq[k] /= channel_H[k]; // ZF equalizer } // STEP R-4: Extract active subcarriers from FFT output vector<Complex> equalized(rx_freq.begin(), rx_freq.begin() + config.num_subcarriers); // STEP R-5: Symbol de-mapping (nearest-neighbour decision) vector<int> decoded_bits = mod_engine->demodulate(equalized);

10.3 OFDMSimulator – System Orchestrator

OFDMSimulator owns all five module instances via std::unique_ptr and exposes runFullChainSimulation() which executes the complete transmit pipeline in the correct order, printing per-step diagnostics and final PAPR metrics.

class OFDMSimulator { private: NumerologyManager num_mgr; unique_ptr<ModulationEngine> mod_engine; unique_ptr<ResourceGridManager> grid_mgr; unique_ptr<FFTProcessor> fft_proc; unique_ptr<CyclicPrefixProcessor> cp_proc; vector<int> generateRandomBits(int num_bits); public: OFDMSimulator( NumerologyManager::NumerologyIndex mu = NumerologyManager::NumerologyIndex::MU_1, ModulationEngine::ModulationType mod = ModulationEngine::ModulationType::QPSK); void runFullChainSimulation() { // [STEP 1] Display numerology config (DEV 1) num_mgr.displayConfiguration(); // [STEP 2] Display constellation (DEV 2) mod_engine->displayConstellation(); // [STEP 3] Generate random bits & modulate auto bits = generateRandomBits(1000); auto symbols = mod_engine->modulate(bits); // [STEP 4] Map to resource grid (DEV 3) grid_mgr->mapSymbolsToGrid(symbols); // [STEP 5] Per-symbol IFFT (DEV 4) vector<vector<Complex>> td_symbols; for (int i = 0; i < config.num_symbols_per_slot; i++) td_symbols.push_back(fft_proc->ifft( grid_mgr->getOFDMSymbol(i))); // [STEP 6] Add cyclic prefix & assemble slot (DEV 5) auto slot = cp_proc->assembleSlot(td_symbols); // [STEP 7] Compute PAPR double avg_p=0, peak_p=0; for (auto& s: slot) { double p=norm(s); avg_p+=p; peak_p=max(peak_p,p); } avg_p /= slot.size(); cout << "PAPR: " << 10*log10(peak_p/avg_p) << " dB" << endl; } };

10.4 Interactive Console Menu (main.cpp)

The main() function provides a numbered console menu that exercises each developer module independently as well as full-chain simulations with different numerology/modulation combinations. This design allows step-by-step verification during integration testing.

// Main menu options DEVELOPER MODULE OPTIONS: 1. [DEV 1] Display Numerology Configuration (μ = 0,1,2,3,4) 2. [DEV 2] QPSK Modulation & Constellation 3. [DEV 2] 16-QAM Modulation & Constellation 4. [DEV 2] 64-QAM Modulation & Constellation 5. [DEV 2] 256-QAM Modulation & Constellation 6. [DEV 3] Resource Grid Mapping Demonstration 7. [DEV 4] IFFT Processing Demonstration 8. [DEV 5] Cyclic Prefix Addition Demonstration FULL SYSTEM SIMULATIONS: 9. Complete OFDM Chain (QPSK, μ=1, 30 kHz SCS) 10. Complete OFDM Chain (16-QAM,μ=1, 30 kHz SCS) 11. Complete OFDM Chain (64-QAM,μ=2, 60 kHz SCS) 12. Performance Comparison (All Modulation Schemes) 0. Exit

10.5 Performance Comparison – All Modulation Schemes

Menu option 12 calls runPerformanceComparison(), which instantiates all four modulation engines and tabulates key metrics. The table below mirrors the runtime console output.

| Modulation Scheme | Bits / Symbol | Constellation Size | Spectral Efficiency (b/s/Hz) |
| --- | --- | --- | --- |
| QPSK | 2 | 4 | 1.70 |
| 16-QAM | 4 | 16 | 3.40 |
| 64-QAM | 6 | 64 | 5.10 |
| 256-QAM | 8 | 256 | 6.80 |

Trade-off Analysis Higher-order modulations increase data rate but demand better SNR. QPSK is preferred at the cell edge (low SNR); 256-QAM is used in ideal near-base-station conditions. 5G NR performs Adaptive Modulation & Coding (AMC) to select the optimal scheme dynamically based on the reported CQI.

10.6 BER Performance Analysis

Bit Error Rate (BER) measures the fraction of incorrectly decoded bits over total transmitted bits. The theoretical BER for AWGN channels with OFDM modulation follows the closed-form expressions below. Extending the simulator to compute BER requires adding an AWGN channel model and a symbol-decision block in the receive path.

Theoretical BER Formulas (AWGN):

// BER theoretical formulas (AWGN channel): // // QPSK: BER = Q( sqrt(2 * Eb/N0) ) // // 16-QAM: BER ≈ (3/8) * erfc( sqrt(Eb/N0 * (4/5)) ) // // 64-QAM: BER ≈ (7/24) * erfc( sqrt(Eb/N0 * (2/7)) ) // // where: Eb/N0 = energy per bit / noise power spectral density // Q(x) = (1/2) * erfc(x / sqrt(2)) // erfc() = complementary error function // C++ snippet: compute Q-function using erfc #include <cmath> double Q_function(double x) { return 0.5 * erfc(x / sqrt(2.0)); } double BER_QPSK(double EbN0_linear) { return Q_function(sqrt(2.0 * EbN0_linear)); }

Required SNR for BER = 10⁻³:

| Modulation | Required Eb/N0 (dB) @ BER = 10⁻³ |
| --- | --- |
| QPSK | ≈ 6.8 dB |
| 16-QAM | ≈ 10.5 dB |
| 64-QAM | ≈ 15.0 dB |
| 256-QAM | ≈ 20.5 dB |

10.7 PAPR Analysis from Simulation Output

PAPR (Peak-to-Average Power Ratio) is a key OFDM drawback. The simulator computes PAPR as part of runFullChainSimulation() after the CP-assembly stage. The code below extracts the formula:

// PAPR computation in OFDMSimulator::runFullChainSimulation() double avg_power = 0.0, peak_power = 0.0; for (const auto& sample : final_signal) { double power = norm(sample); // ¦sample¦^2 avg_power += power; peak_power = max(peak_power, power); } avg_power /= final_signal.size(); double papr_dB = 10.0 * log10(peak_power / avg_power); cout << "PAPR: " << fixed << setprecision(2) << papr_dB << " dB" << endl;

Typical PAPR Values For QPSK OFDM with 1200 subcarriers the simulator typically reports PAPR in the range of 9 – 12 dB. Higher-order modulations and more active subcarriers generally increase PAPR. Techniques such as Clipping & Filtering, SLM, and Tone Reservation can reduce PAPR at the cost of complexity or slight BER degradation.

13. Compilation & Execution Guide

13.2 C++ Setup – Compiler & Libraries

The simulator is a single-file C++ program with zero external dependencies. All required functionality (complex arithmetic, random numbers, FFT) is implemented using the C++ Standard Library.

Supported Platforms:

- Linux / macOS — GCC 9+  or  Clang 10+

- Windows — MSVC 2019+ (via Developer Command Prompt) or MinGW-w64

Build Commands:

# ── Linux / macOS (GCC) ───────────────────────────────────────── g++ -std=c++17 -O2 -Wall -o ofdm_sim phy_ofdm_simulator.cpp # ── Linux / macOS (Clang) ─────────────────────────────────────── clang++ -std=c++17 -O2 -Wall -o ofdm_sim phy_ofdm_simulator.cpp # ── Windows (MSVC, Developer Command Prompt) ──────────────────── cl /std:c++17 /O2 /EHsc phy_ofdm_simulator.cpp /Fe:ofdm_sim.exe # ── Windows (MinGW-w64) ───────────────────────────────────────── g++ -std=c++17 -O2 -Wall -o ofdm_sim.exe phy_ofdm_simulator.cpp

13.3 Running the Simulation

# Run on Linux/macOS ./ofdm_sim # Run on Windows ofdm_sim.exe # ── Sample Session ─────────────────────────────────────────────── Enter your choice: 9 → Executes complete OFDM chain: QPSK + μ=1 + 30 kHz SCS → Prints: Numerology config, constellation, grid utilisation, IFFT samples, CP overhead, PAPR Enter your choice: 12 → Prints performance comparison table for all modulation orders

13.4 Troubleshooting

| Issue | Solution |
| --- | --- |
| 'complex' not found | Add #include <complex> and using Complex = complex<double>; |
| FFT size not power of 2 | FFTProcessor constructor auto-rounds up – no action needed |
| Garbage console output | Ensure terminal supports Unicode; on Windows use chcp 65001 |
| Segfault in getOFDMSymbol | symbol_index must be in [0, num_symbols_per_slot) |
| PAPR > 15 dB | Expected for large FFT – enable clipping before reporting |
| Compile error with /std:c++14 | Upgrade to /std:c++17 (make_unique requires C++14 minimum) |

11.1 Time Domain Waveforms

The time-domain OFDM signal represents the transmitted waveform after IFFT processing and cyclic prefix insertion. Analysis of time-domain waveforms reveals critical characteristics including:

Time Domain Signal Characteristics: • OFDM Symbol Duration: For μ=1 (30 kHz SCS), each OFDM symbol spans 35.68 μs • Cyclic Prefix Overhead: CP adds 9/128 ≈ 7.03% overhead per symbol • Signal Amplitude: Varies based on modulation scheme and number of active subcarriers • PAPR Observation: Peak-to-Average Power Ratio typically 8-12 dB for OFDM signals

Figure 11.1: Time-domain OFDM signal showing 3 symbols with cyclic prefix regions highlighted

Sample Output from C++ Implementation:

[OFDM TIME-DOMAIN SIGNAL GENERATION] Symbol 0: Duration = 2192 samples (2048 FFT + 144 CP) Real: [-0.342, 0.156, -0.089, 0.234, ...] Imag: [0.178, -0.267, 0.123, -0.045, ...] Peak Amplitude: 2.34, RMS Amplitude: 0.71 PAPR: 10.34 dB

Key Observations:

1. Cyclic Prefix Structure: The first 144 samples of each OFDM symbol are copies of the last 144 samples from the IFFT output, creating circular convolution properties essential for ISI mitigation.

2. Signal Continuity: The CP ensures phase continuity between consecutive OFDM symbols, preventing inter-symbol interference in multipath channels.

3. PAPR Challenge: High PAPR is inherent in OFDM due to coherent addition of multiple subcarriers. This requires linear power amplifiers with sufficient back-off to avoid signal clipping.

11.2 Frequency Domain Spectrum

The frequency-domain representation showcases OFDM's spectral efficiency through orthogonal subcarrier placement. Each subcarrier exhibits a sinc-shaped spectrum with nulls at adjacent subcarrier frequencies.

Spectral Analysis Parameters: • Subcarrier Spacing (Δf): 30 kHz for μ=1 numerology • Bandwidth (BW): 20 MHz (1200 subcarriers × 30 kHz) • Spectral Efficiency: Each subcarrier is orthogonal, minimizing ICI • Out-of-Band Emissions: Controlled through windowing and filtering

Figure 11.2: Frequency spectrum showing active subcarriers, guard bands, and DC null

FFT Spectrum Output:

[FREQUENCY DOMAIN SPECTRUM ANALYSIS] FFT Size: 2048 points Active Subcarriers: 1200 (600 on each side of DC) DC Subcarrier: Null (zero power) Guard Bands: 424 subcarriers on each side Subcarrier Power Distribution (sample): Subcarrier 100: Power = -3.2 dB, Phase = 45.3° Subcarrier 200: Power = -2.8 dB, Phase = 132.7° Subcarrier 300: Power = -3.5 dB, Phase = -78.4°

Figure 11.3: 5G NR resource grid structure showing data, pilot, and DMRS symbol allocation

11.3 BER vs SNR Curves

Bit Error Rate (BER) performance as a function of Signal-to-Noise Ratio (SNR) is the primary metric for evaluating digital communication systems. The simulation demonstrates theoretical and practical BER curves for various modulation schemes.

BER Performance Summary (at BER = 10⁻³): • QPSK: Required SNR ≈ 6.8 dB (2 bits/symbol, robust) • 16-QAM: Required SNR ≈ 10.5 dB (4 bits/symbol, moderate) • 64-QAM: Required SNR ≈ 16.4 dB (6 bits/symbol, high throughput) • 256-QAM: Required SNR ≈ 22.3 dB (8 bits/symbol, maximum throughput)

Figure 11.4: BER vs SNR performance curves for QPSK, 16-QAM, 64-QAM, and 256-QAM

Simulation Results Interpretation:

1. SNR Requirement Scaling: Each increase in modulation order (2× constellation points) requires approximately 4-6 dB additional SNR to maintain the same BER. This represents the fundamental trade-off between spectral efficiency and power efficiency.

2. Waterfall Effect: BER curves exhibit sharp transitions from high error rates to near-zero errors, demonstrating the threshold behavior characteristic of coherent detection systems.

3. Practical Implementation Gap: Real systems typically show 1-2 dB degradation from theoretical curves due to imperfect channel estimation, phase noise, and non-ideal components.

11.4 Effect of Cyclic Prefix Length

Cyclic prefix length is a critical design parameter that directly impacts OFDM system performance. The CP must be longer than the channel delay spread to completely eliminate ISI and ICI.

CP Length Analysis: • Standard CP (144 samples): Covers delay spreads up to 4.69 μs • Extended CP (512 samples): Covers delay spreads up to 16.67 μs • Overhead Impact: Normal CP = 7.03%, Extended CP = 25% • Use Case: Extended CP for high-mobility or large-cell scenarios

Figure 11.5: Cyclic prefix overhead and requirement analysis for different channel conditions

CP Length Comparison (Simulation):

Channel Delay Spread: 3.0 μs (Typical Urban) CP = 72 samples (2.34 μs): BER @ 15 dB SNR: 2.4 × 10⁻² [INSUFFICIENT CP] ISI/ICI Degradation: 3.5 dB CP = 144 samples (4.69 μs): BER @ 15 dB SNR: 3.2 × 10⁻⁴ [ADEQUATE CP] ISI/ICI Degradation: 0.2 dB CP = 512 samples (16.67 μs): BER @ 15 dB SNR: 2.8 × 10⁻⁴ [EXCESSIVE OVERHEAD] ISI/ICI Degradation: 0.0 dB (but 18% throughput loss)

11.5 Effect of FFT Size

FFT size determines subcarrier spacing and symbol duration, directly affecting frequency selectivity, Doppler tolerance, and computational complexity.

FFT Size Comparison (20 MHz Bandwidth, μ=1): • N = 512: Δf = 58.59 kHz, Ts = 17.07 μs (poor freq. selectivity) • N = 1024: Δf = 29.30 kHz, Ts = 34.13 μs (moderate) • N = 2048: Δf = 14.65 kHz, Ts = 68.27 μs (standard for 5G) • N = 4096: Δf = 7.32 kHz, Ts = 136.53 μs (high Doppler sensitivity)

Figure 11.6: Impact of FFT size on ICI degradation and computational complexity

Key Trade-offs:

1. Frequency Selectivity: Larger FFT (smaller Δf) provides better frequency resolution, enabling precise equalization in frequency-selective channels. However, this increases sensitivity to Doppler shift.

2. Doppler Tolerance: Smaller FFT (larger Δf) improves robustness to high-speed mobility but sacrifices frequency selectivity and increases CP overhead percentage.

3. Computational Complexity: FFT complexity scales as O(N log N). Doubling FFT size increases processing by approximately 2.1×, impacting power consumption and latency.

11.6 Constellation Diagrams at Different SNR

Constellation diagrams visualize the received signal quality by plotting I/Q samples. The scatter pattern reveals the combined effects of noise, interference, and channel impairments.

Figure 11.7: Constellation diagrams for QPSK, 16-QAM, and 64-QAM at various SNR levels with EVM measurements

EVM Requirements by Standard:

3GPP TS 38.104 - 5G NR Base Station Requirements: • QPSK: EVM ≤ 17.5% (-15.1 dB) • 16-QAM: EVM ≤ 12.5% (-18.1 dB) • 64-QAM: EVM ≤ 8.0% (-21.9 dB) • 256-QAM: EVM ≤ 3.5% (-29.1 dB)

Constellation Impairment Analysis:

• Phase Noise: Causes constellation rotation, increasing with integration time. More severe for higher-order QAM due to tighter decision boundaries.

• I/Q Imbalance: Manifests as amplitude and phase asymmetry between quadrature components. Typical values: 0.5 dB gain imbalance, 2-3° phase imbalance.

• Carrier Frequency Offset: Produces circular rotation of the entire constellation over time. Compensated via pilot-aided estimation in practical OFDM receivers.

11.7 PAPR (Peak-to-Average Power Ratio) Analysis

PAPR is one of the major challenges in OFDM systems. High PAPR occurs when multiple subcarriers add constructively, creating signal peaks that require linear amplification to avoid distortion.

Figure 11.8: PAPR distribution histogram and CCDF showing probability of exceeding thresholds

PAPR Impact on System Design:

• Power Amplifier Back-off: Typical OFDM PAPR of 10-12 dB requires 3-4 dB additional back-off to maintain linearity, reducing PA efficiency from ~40% to ~20%.

• ADC/DAC Resolution: High PAPR necessitates higher bit-depth converters (12-14 bits) to maintain SNR across the dynamic range without quantization noise.

• PAPR Reduction Techniques: Selective mapping (SLM), partial transmit sequences (PTS), and clipping with filtering can reduce PAPR by 2-4 dB at the cost of slight BER degradation (~0.5 dB).

11.8 Performance Summary and Conclusions

The comprehensive simulation results validate the theoretical OFDM performance characteristics and provide quantitative insights into system design trade-offs.

Key Performance Metrics Achieved: ✓ Spectral Efficiency: Up to 5.5 bps/Hz with 256-QAM ✓ ISI Mitigation: Complete elimination with proper CP design ✓ Frequency Selectivity: One-tap equalization in frequency domain ✓ Adaptive Modulation: BER-guided MCS selection (QPSK to 256-QAM) ✓ Implementation Verified: C++ code matches theoretical predictions

Design Recommendations:

1. Parameter Selection: For 5G NR FR1 (sub-6 GHz), use μ=1 (30 kHz SCS) with N=2048 FFT and normal CP (144 samples). This configuration balances frequency selectivity, Doppler tolerance, and overhead.

2. Link Adaptation: Implement CQI-based MCS selection with margins: QPSK for SNR < 10 dB, 16-QAM for 10-18 dB, 64-QAM for 18-28 dB, 256-QAM for SNR > 28 dB.

3. PAPR Management: Apply selective clipping (3-4 dB) combined with tone reservation for PAPR reduction, accepting 0.5 dB SNR degradation to improve power amplifier efficiency by 20-30%.

4. Channel Estimation: Use comb-type pilot patterns with spacing Δk = 6 subcarriers and Δℓ = 2 symbols. Wiener filtering in frequency domain provides optimal MSE with acceptable complexity.

Simulation Validation:

C++ Implementation Statistics: Total Lines of Code: 915 Modular Design: 5 independent developer modules Processing Speed: 14 OFDM symbols in 23.4 ms (real-time capable) Memory Footprint: 8.7 MB for full simulation Compliance: 3GPP TS 38.211 verified through test vectors

# 12.1 Important Formulas Summary

## Basic OFDM Parameters

| Formula | Description |
| --- | --- |
| Δf = 1/T | Subcarrier spacing (Δf) = 1 / Symbol duration (T) |
| B = N × Δf | Total bandwidth = Number of subcarriers × Subcarrier spacing |
| T = N/B | OFDM symbol duration without cyclic prefix |
| T_CP = T × (CP_Length/N) | Cyclic prefix duration |
| T_total = T + T_CP | Total OFDM symbol duration with cyclic prefix |
| R = (N × M) / T_total | Data rate (M = bits per modulation symbol) |
| η = R/B | Spectral efficiency (bits/s/Hz) |

## Orthogonality Condition

The orthogonality between two subcarriers i and j is maintained when:

∫[0 to T] e^(j2π(fᵢ - fⱼ)t) dt = 0   for i ≠ j

Where subcarrier spacing must satisfy: Δf = 1/T

## IFFT/FFT Implementation

Transmitter (IFFT):

x(n) = (1/N) Σ[k=0 to N-1] X(k) e^(j2πkn/N)

Receiver (FFT):

X(k) = Σ[n=0 to N-1] x(n) e^(-j2πkn/N)

## Channel Model and Equalization

Frequency Domain Channel Model:

Y(k) = H(k) · X(k) + W(k)

One-Tap Frequency Domain Equalization:

X̂(k) = Y(k) / H(k)

Where Y(k) = received signal, H(k) = channel frequency response, X(k) = transmitted signal, W(k) = noise

# 12.2 3GPP Technical Specifications

## 3GPP TS 38.211 - 5G NR Physical Channels and Modulation

Document Version: Release 17 (Latest stable version)

URL: https://www.3gpp.org/DynaReport/38211.htm

Key Sections:

| Section | Content |
| --- | --- |
| Section 4 | Frame structure (slots, symbols) |
| Section 5 | Physical channels (PDSCH, PUSCH) |
| Section 6 | Modulation and upconversion (CP-OFDM, DFT-s-OFDM) |
| Section 7 | Physical signals (reference signals, synchronization signals) |

### Important 5G NR OFDM Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Subcarrier Spacing (μ) | 15, 30, 60, 120, 240 kHz | Δf = 2^μ × 15 kHz |
| FFT Size | 128 to 4096 | Depends on bandwidth |
| CP Length | Normal: 144 Ts, Extended: N/A | 5G NR uses normal CP only |
| Symbols per Slot | 14 | Fixed for normal CP |

## 3GPP TS 36.211 - LTE Physical Channels and Modulation

Document Version: Release 16 (Latest LTE version)

URL: https://www.3gpp.org/DynaReport/36211.htm

Key Sections:

| Section | Content |
| --- | --- |
| Section 5 | Downlink physical channels (PDCCH, PDSCH) |
| Section 6 | Modulation and upconversion |
| Section 7 | Physical resource blocks and transmission bandwidth |

### LTE OFDM Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Subcarrier Spacing | 15 kHz | Fixed for all bandwidths |
| FFT Size | 128 to 2048 | Based on channel bandwidth |
| CP Type | Normal, Extended | Extended CP for high delay spread |
| Symbols per Slot | 7 (Normal), 6 (Extended) | Per 0.5 ms slot |

# 12.3 IEEE 802.11 Standards (Wi-Fi)

## IEEE 802.11-2020

Document: IEEE Standard for Information technology—Telecommunications and information exchange between systems - Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

URL: https://standards.ieee.org/ieee/802.11/7028/

### Wi-Fi Standards Evolution with OFDM

| Standard | Year | Max Data Rate | OFDM Features |
| --- | --- | --- | --- |
| 802.11a | 1999 | 54 Mbps | First Wi-Fi standard with OFDM, 5 GHz band |
| 802.11g | 2003 | 54 Mbps | OFDM on 2.4 GHz band |
| 802.11n | 2009 | 600 Mbps | MIMO-OFDM, 40 MHz channels |
| 802.11ac | 2014 | 6.9 Gbps | MU-MIMO, 160 MHz channels, 256-QAM |
| 802.11ax (Wi-Fi 6) | 2019 | 9.6 Gbps | OFDMA, 1024-QAM, BSS Coloring |
| 802.11be (Wi-Fi 7) | 2024 | 46 Gbps | 320 MHz channels, 4096-QAM, MLO |

### Wi-Fi OFDM Parameters (802.11a/g/n/ac)

| Parameter | Value |
| --- | --- |
| FFT Size | 64 |
| Number of Data Subcarriers | 48 |
| Number of Pilot Subcarriers | 4 |
| Subcarrier Spacing | 312.5 kHz |
| Symbol Duration | 3.2 μs |
| Guard Interval (GI) | 0.8 μs (Short GI: 0.4 μs in 802.11n) |
| Total Symbol Duration | 4.0 μs |
| Channel Bandwidth | 20 MHz (base), 40/80/160 MHz in newer standards |

# 12.4 Further Reading and Resources

## Textbooks and Academic References

1. "OFDM for Wireless Communications Systems" by Ramjee Prasad - Comprehensive coverage of OFDM theory and applications in wireless systems.

2. "Wireless Communications: Principles and Practice" by Theodore S. Rappaport - Fundamental wireless communication concepts including OFDM.

3. "Digital Communications" by John G. Proakis and Masoud Salehi - Mathematical foundations of digital communications and modulation.

4. "MIMO-OFDM Wireless Communications with MATLAB" by Yong Soo Cho et al. - Practical MATLAB implementations of OFDM systems.

5. "Fundamentals of Wireless Communication" by David Tse and Pramod Viswanath - Advanced wireless communication theory including OFDM and MIMO.

## Online Resources and Tutorials

1. 3GPP Portal: https://www.3gpp.org/ - Official 3GPP technical specifications for LTE and 5G NR

2. IEEE Standards Association: https://standards.ieee.org/ - IEEE 802.11 Wi-Fi standards and documentation

3. MATLAB Communications Toolbox: Documentation and examples for OFDM implementation

4. GNU Radio: https://www.gnuradio.org/ - Open-source software-defined radio toolkit with OFDM blocks

5. Wireless Pi: https://wirelesspi.com/ - Educational resources on wireless communication technologies

## Research Papers and Journals

Key Journals:

• IEEE Transactions on Communications

• IEEE Transactions on Wireless Communications

• IEEE Communications Magazine

• IEEE Wireless Communications Letters

• EURASIP Journal on Wireless Communications and Networking

## Tools and Simulation Platforms

| Tool | Language/Platform | Use Case |
| --- | --- | --- |
| MATLAB | MATLAB/Simulink | Rapid prototyping, simulation |
| GNU Radio | Python/C++ | SDR implementation, real-time |
| Python (NumPy/SciPy) | Python | Educational, algorithm development |
| C/C++ | C/C++ | Production, embedded systems |
| USRP (SDR Hardware) | Hardware | Over-the-air testing, research |

# 13. Compilation & Execution Guide

This section provides comprehensive instructions for setting up, compiling, and executing the OFDM simulation code. The guide covers multiple programming environments including MATLAB, Python, and C++.

# 13.1 MATLAB Setup and Execution

## System Requirements

| Component | Requirement |
| --- | --- |
| MATLAB Version | R2018b or later (recommended: R2023a or newer) |
| Required Toolboxes | Signal Processing Toolbox, Communications Toolbox |
| RAM | Minimum 4 GB (8 GB recommended) |
| Operating System | Windows 10/11, macOS 10.14+, Linux (Ubuntu 18.04+) |

## Installation Steps

Step 1: Install MATLAB

Download MATLAB from MathWorks website (https://www.mathworks.com/downloads/) and follow the installation wizard. Ensure you select the Signal Processing and Communications Toolboxes during installation.

Step 2: Verify Installation

Open MATLAB and run the following command in the Command Window:

ver

This will display your MATLAB version and installed toolboxes.

## Project File Structure

Organize your OFDM simulation files as follows:

OFDM_Project/ ├── main_ofdm_simulation.m ├── ofdm_transmitter.m ├── ofdm_receiver.m ├── channel_model.m ├── ber_analysis.m └── utils/ ├── plot_spectrum.m ├── plot_constellation.m └── calculate_papr.m

## Running the Simulation

Step 1: Set Working Directory

Navigate to the project folder in MATLAB:

cd '/path/to/OFDM_Project' addpath('utils');

Step 2: Execute Main Simulation

Run the main simulation file:

main_ofdm_simulation

Step 3: Configure Parameters

Modify simulation parameters at the top of main_ofdm_simulation.m:

%% OFDM Parameters N_fft = 64; % FFT size N_cp = 16; % Cyclic prefix length mod_order = 4; % QPSK (4-QAM) num_symbols = 1000; % Number of OFDM symbols SNR_dB = 0:2:30; % SNR range in dB

## Expected Output

Upon successful execution, MATLAB will generate:

• Time-domain OFDM signal plots

• Frequency-domain spectrum analysis

• Constellation diagrams at different SNR levels

• BER vs SNR performance curves

• PAPR statistics

# 13.2 Python Setup and Execution

## System Requirements

| Component | Requirement |
| --- | --- |
| Python Version | Python 3.8 or later (recommended: 3.10+) |
| Required Libraries | NumPy, SciPy, Matplotlib, CommPy |
| Package Manager | pip (included with Python) or conda |

## Installation Steps

Step 1: Install Python

Download Python from python.org or use Anaconda distribution for easier package management.

Step 2: Create Virtual Environment (Recommended)

# Create virtual environment python -m venv ofdm_env # Activate virtual environment # Windows: ofdm_env\Scripts\activate # Linux/macOS: source ofdm_env/bin/activate

Step 3: Install Required Libraries

pip install numpy scipy matplotlib scikit-commpy

Or install from requirements.txt:

pip install -r requirements.txt

## Project Structure

ofdm_python/ ├── main.py ├── ofdm_system.py ├── channel.py ├── utils.py ├── requirements.txt └── README.md

## Running the Simulation

Basic Execution:

python main.py

With Custom Parameters:

python main.py --fft_size 128 --cp_len 32 --mod qpsk --snr_max 30

# 13.3 C++ Implementation

## System Requirements

| Component | Requirement |
| --- | --- |
| Compiler | GCC 9.0+, Clang 10.0+, or MSVC 2019+ |
| C++ Standard | C++17 or later |
| Build System | CMake 3.15 or later |
| Libraries | FFTW3, Eigen3 (optional but recommended) |

## Installation Steps

Step 1: Install Compiler and CMake

For Windows:

Download and install Visual Studio 2019 or later with C++ development tools, or install MinGW-w64.

For Linux:

sudo apt-get update sudo apt-get install build-essential cmake

Step 2: Install FFTW Library

Ubuntu/Debian:

sudo apt-get install libfftw3-dev

macOS (using Homebrew):

brew install fftw

## Building the Project

Step 1: Navigate to Project Directory

cd /path/to/ofdm_cpp

Step 2: Create Build Directory

mkdir build cd build

Step 3: Configure with CMake

cmake ..

Step 4: Compile

make -j4

Step 5: Run the Executable

./ofdm_simulation

# 13.4 Troubleshooting Common Issues

## MATLAB Issues

| Problem | Cause | Solution |
| --- | --- | --- |
| Function not found error | Missing toolbox or incorrect path | Run 'ver' to check toolboxes; add path using 'addpath' |
| Out of memory error | Large FFT size or too many symbols | Reduce FFT size or process in batches; increase system RAM |
| Plots not displaying | Figure windows minimized | Add 'figure' command before each plot |
| Slow execution | Loops not vectorized | Use vectorized operations; enable parallel computing |

## Python Issues

| Problem | Cause | Solution |
| --- | --- | --- |
| ModuleNotFoundError | Library not installed | Install missing library: pip install <library> |
| Version conflicts | Incompatible library versions | Use virtual environment; specify versions in requirements.txt |
| Matplotlib backend error | No display available | Use 'Agg' backend: import matplotlib; matplotlib.use('Agg') |
| NumPy array dimension error | Incorrect array shape | Use reshape() or verify array dimensions with .shape |

## C++ Issues

| Problem | Cause | Solution |
| --- | --- | --- |
| CMake cannot find FFTW | Library not in default path | Set CMAKE_PREFIX_PATH or install FFTW |
| Linker errors | Missing library links | Add library to target_link_libraries in CMakeLists.txt |
| Segmentation fault | Memory access error | Use debugger (gdb); check array bounds and pointer validity |
| Compiler warnings | Implicit type conversions | Add explicit type casts; enable -Wall -Wextra flags |

## Performance Optimization Tips

For MATLAB:

• Pre-allocate arrays using zeros() or ones()

• Use vectorized operations instead of loops

• Enable parallel computing with parfor

• Use profiler to identify bottlenecks: profile on; profile viewer

For Python:

• Use NumPy's built-in functions (vectorized operations)

• Consider using Numba JIT compilation for critical loops

• Profile with cProfile: python -m cProfile main.py

• Use multiprocessing for parallel execution

For C++:

• Enable compiler optimizations: -O3 flag

• Use FFTW wisdom for faster FFT planning

• Leverage OpenMP for multi-threading

• Profile with gprof or Valgrind